Software Requirement Specification

MAL P18 DC-DC

| Version Status Date | 1.2 Draft 11th July 2024 |
| --- | --- |

Confidentiality Notice

Copyright (c) 2026 eInfochips. - All rights reserved

This document is authored by eInfochips and is eInfochips intellectual property, including the copyrights in all countries in the world. This document is provided under a license to use only with all other rights, including ownership rights, being retained by eInfochips. This file may not be distributed, copied, or reproduced in any manner, electronic or otherwise, without the express written consent of eInfochips.

Template Version 1.6

Contents

1	Document Details	5

1.1	Revision & Approval History	5

1.2	Definition, Acronyms and Abbreviations	6

1.3	References	6

2	introduction	7

2.1	Purpose of the Document	7

2.2	Intended Audience	7

3	software overview	8

3.1	System Overview	8

3.2	Scope	9

4	Functional Requirement	10

4.1	Communication	10

4.1.1	Requirements	10

4.1.2	Safety Requirements	80

4.1.3	Business Rules	80

4.1.4	Screens / Wireframes	80

4.2	State-Machine	80

4.2.1	Requirements	80

4.2.2	Safety Requirements	151

4.2.3	Business Rules	151

4.2.4	Screens / Wireframes	151

4.3	Diagnostics	151

4.3.1	Requirements	151

4.3.2	Safety Requirements	345

4.3.3	Business Rules	345

4.3.4	Screens / Wireframes	346

4.4	General Diagnostics	346

4.4.1	Requirements	346

4.4.2	Safety Requirements	357

4.4.3	Business Rules	357

4.4.4	Screens / Wireframes	357

4.5	Power Management Controller interface driver	358

4.5.1	Requirements	359

4.5.2	Safety Requirements	375

4.5.3	Business Rules	375

4.5.4	Screens / Wireframes	375

4.6	Specific OBD Requirements	375

4.6.1	Requirements	375

4.6.2	Safety Requirements	434

4.6.3	Business Rules	434

4.6.4	Screens / Wireframes	434

4.7	P18 OBD Required Documents	434

4.7.1	Requirements	434

4.7.2	Safety Requirements.	436

4.7.3	Business Rules.	436

4.8	Safety Requirements	436

4.8.1	Requirements	436

4.8.2	Safety Requirements	448

4.8.3	Business Rules	448

4.9	Non-Functional Requirements	448

4.9.1	Requirements	448

4.9.2  Safety Requirements	481

4.9.2	Business Rules	481

4.9.3	Screens / Wireframes	481

4.10	Operating environment	481

4.11	Configuration Parameters	482

5	External Interface	483

5.1	User Interface	483

5.2	Hardware Interface	483

5.3	Software Interface	483

# Document Details

## Revision & Approval History

| Version | Author |  | Reviewer |  | Approver |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name | Date (DD-MM-YYYY) | Name | Date (DD-MMM-YYYY) | Name | Date (DD-MM-YYYY) |
| Draft 0.1 | Manish Thakur Deep Darji Rahul Sonawane Praveen Mayappa | 4-Jan-2023 | Prashant Tajne / Laxmikant | 04-Jan-2023 |  |  |
| Draft 0.2 | Manish Thakur Deep Darji Rahul Sonawane Praveen Mayappa | 09-Feb-2023 | Prashant Tajne | 10-Feb-2023 |  |  |
| Draft 0.3 | Rahul Sonawane Deep Darji Yatish Thakre | 3-Jan-2024 | Mayur Gaikwad | 10-Jan-2024 |  |  |
| Draft 0.4 | Mayur Gaikwad | 12-Mar-2024 | Muhammad Fawzi, Simran Ruprah | 18-Mar-2024 |  |  |
| Draft 0.5 | Mayur Gaikwad | 23-Mar-2024 | Muhammad Fawzi, | 31-Mar-2024 |  |  |
| Draft 0.6 | Mayur Gaikwad | 02-Apr-2024 | Arturo | 17-April-2024 |  |  |
| Draft 0.7 | Mayur Gaikwad | 19-Apr-2024 | Muhammad Fawzi, | 22-Apr-2024 |  |  |
| Baseline 1.0 | Mayur Gaikwad | 2-May-2024 | Arturo | 7-May-2024 | Mohan Wankhede | 14-May-2024 |
| Draft 1.1 | Mayur Gaikwad | 14-May-2024 | Arturo | 14-May-2024 |  |  |
| Draft 1.2 | Shweta Sable | 11-July-2024 | Mayur Gaikwad | 12-May-2024 | Mohan Wankhede |  |

| Version | Description of Change |
| --- | --- |
| Draft 0.1 | First Draft for PF1.0 |
| Draft 0.2 | Second draft for PF1.0 |
| Draft 0.3 | -Reviewed document -Added PF2.0 Requirements details |
| Draft 0.4 | -Added PF2.1 & PF3.0 Requirements details -Added the proper comments in not applicable sections of each requirement (Derived Requirement, Access Restrictions, Mandatory Fields, Pre-Loaded Values, Default Values, Valid range of Values, Data Latency Period, Data Retention Period, Data Rate/ <Daily> Number of transactions, External Events, Temporal Events, Validation Rules/ Verification criteria etc) - Added all the diagnostics requirement |
| Draft 0.5 | -Modified the document after reviewer’s comments -Added NM software requirements -Added SW requirements for newly added requirements |
| Draft 0.6 | - Fixed review comments from Muhammad - Corrected Verification Criteria of mentioned requirements (SWRS_APP_STATE_04, SWRS_APP_STATE_05, SWRS_APP_STATE_06, SWRS_APP_STATE_07, SWRS_APP_STATE_08) - Added Abbreviations Added OBD requirements. Aspice specific comments fixed in the document. |
| Draft 0.7 | Fixed review comments from Arturo. - SWRS_Diag_01: In input, add that the fault should be generated in all states. (Applicable for all other requirements). - SWRS_Diag_07, SWRS_Diag_08, SWRS_Diag_09 => Added fault condition in input. - SWRS_Diag_115: Input to correct: The Conversion should be start and HVDCDC should be in HVON state. - SWRS_Diag_55: Add the exact state of HVIL in purpose. - SWRS_Diag_109: Add input as Sample Depth for each sensing should need diff calibratable. - NM requirements should be updated as per the HVDCDC state machine. - Corrected default timing for all the NM configurable parameters and change the same. |
| Draft 0.8 | Fixed review comments from Arturo on OBD requirements - SWRS_OBD_37: Do not consider DTC’s from DCDC ones, use DCU ZC1 - SWRS_OBD_03: Test Currently Failing: Bad calibration or wrong SW is not the correct cause. Issue at the application that will cause the fail of Diag monitor test. - Add requirement for SAE specific DTC's. Fixed comments from Quality team |
| Baseline 1.0 | Baseline the document after DM approval |
| Draft 1.1 | Fixed review comments from Arturo on OBD requirements - Section 4.7.1.1: The OBD DTC should not be of 3 byte. It should be 2 byte - Section 4.6.1.9: Flowchart need to be provided for all Diag Text Description. - Section 4.7.1.2: Summary table needs to be updated as per the example provided. |
| Draft 1.2 | Migrated to Latest QMS Template Version 1.6 Added SRS requirements for new SDS requirements from P18 HV DCDC System Design Specification_v2.5 Fixed the comments provided for below requirements - SDS-HVDCDC-304 - SDS-HVDCDC-435 - SDS-HVDCDC-542 to SDS-HVDCDC-551 |

## Definition, Acronyms and Abbreviations

| Definition/Acronym/Abbreviation | Description |
| --- | --- |
| RTM | Requirement Traceability Matrix |
| QTS | Query Tracking Sheet |
| FMEA | Failure Mode & Effect Analysis |
| SAFREQ | Safety Requirement |
| ASIL | Automotive Safety Integrity Level |
| FTTI | Fault Handling Time Interval |
| HV | High Voltage |
| MAL | Maclaren Automotive Ltd |
| SNA | Signal Not Available |
| PM Bus | Power Management Bus |
| PMIC | Power Management Integrated Circuit |
| CAN | Controller Area Network |
| DCM | DC-DC Converter Module |
| I2C | Inter-Integrated Circuit |
| SPI | Serial peripheral interface |
| SSS | Solid State Switch |
| PEC | Packet Error Checking |
| EDSU | External ECU |
| VIN | Vehicle CAN Input messages |
| SCR | Secondary Core |
| DSO | Digital Storage Oscilloscope |
| DMM | Digital Multimeter |

Table 1: Definitions, Acronyms and Abbreviations

## References

| No. | Document | Version | Remarks |
| --- | --- | --- | --- |
| 1 | P18 HV DCDC System Design Specification.xlsx | 2.5 |  |

Table 2: References

# introduction

## Purpose of the Document

The purpose of the document is to specify software requirements for the P18 HV DCDC Project.

## Intended Audience

The intended audience of this document is software developers and integration, system testers.

# software overview

## System Overview

The DCDC converter is responsible for converting the energy between 800V HV Bus of the ESS and 400V HV Bus of the IPU.

- 400V Power Conversion (Buck) – The HV DCDC shall convert electrical power from the 800V HV bus to the 400V HV bus. The HV DCDC shall supply power to the 400V bus in form of electrical current and maintain the voltage at the 400V bus to sustain the loads on 12V HVDCDC, PTC and eAC.

- 800V Power Conversion (Boost) – The DCDC shall convert electrical power from the 400V HV bus to the 800V ESS HV bus during charging. The DCDC shall control voltage and current on the 800V bus during charging.

- Control Topology – The HV DCDC shall respond to commands from the Electric Drive Supervisory Unit for all supported functions.

- Cooling – The HV DCDC shall have one inlet/outlet cooling interfaces to cool the internal components.

Figure 1: System Input/Output Diagram

Figure 2: System Architecture Diagram

## Scope

The software block diagram is a logical view of the software being specified with a set of logical blocks and each block has a set of requirements.

- Application Business Logic – This module contains the handling of overall application state machine, start-up and runtime routines and diagnostic, communication glue logics.

- Middle Ware Communication – This module contains the CAN Communication logic for vehicle communication.

- Middle Ware Diagnostics – This module contains the Diagnostics logic for failures monitoring.

- Middle Ware Sensors – This module contains the Sensor interfacing logic for Current, Voltage and temperature sensors.

- Middle Ware PMIC IF – This module contains the Diagnostics logic for failures monitoring.

- Middle Ware Scheduler – This module contains the task scheduler.

- HAL – This module contains hardware abstraction layer for MCU peripherals.

- MCU– Infineon SAK-TC397XA-256F300W is selected as choice of controller used in this project.

Figure 2: Logical View

# Functional Requirement

## Communication

### Requirements

#### Vehicle & Debug CAN communication on HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_01 |
| Customer Req ID | SDS-HVDCDC-84 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVHVDCDC shall integrate the low CAN drivers provided by a third party. The HVHVDCDC shall support below two CAN interfaces: 1. MCAN interface with LV connector (Vehicle communication) 2. DCAN interface with LV connector (Debug communication) |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | For MCAN Interface : CAN stack/ARXML provided by a third party. For DCAN Interface : Document having information of DCAN messages |
| Output(s) | HVDCDC device has communication handler which handles the system status transmission over CAN. - MCAN interfaces for communication with vehicle EDSU. The HVHVDCDC broadcast the operating modes, fault levels, HVIL status, DC link currents and voltages on both the 800V side and 400V side, and temperatures etc. over MCAN. - DCAN interfaces for communication with vehicle diagnostic port. The HVHVDCDC broadcast the information reported by the power module, the internal state of the state machine transition, 400V side solid state switch status, precharge contactor status, 400V side active discharge switch status, internal fault ID, and Calibration parameters etc. over DCAN. |
| Process | 1. Set 800V side (high side) voltage of the power module greater than 480V. 2. After voltage stabilize at 480V, wait for a duration of 200ms 3. Send NHVDCDCCommand as 0x01 (Enable) using MCAN interface |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No Tx and Rx frame should be there |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN networks |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Integration of DBC or ARXML on HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_02 |
| Customer Req ID | SDS-HVDCDC-84 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVHVDCDC shall integrate communication extract (DBC or ARXML file) provided by customer. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | For MCAN Interface: CAN stack/ARXML provided by a third party. For DCAN Interface: Document having information of DCAN messages |
| Output(s) | Below CAN messages are observed on the MCAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x1 ("Normal State") Below CAN messages are observed on the DCAN interface NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0x6 ("Normal state") |
| Process | 1. Set 800V side (high side) voltage of the power module greater than 480V. 2. After voltage stabilize at 480V, wait for a duration of 200ms 3. Send NHVDCDCCommand as 0x01 (Enable) using MCAN interface |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC= Normal, HVDCDC_Status1=Normal, HVDCDC_Debug=Normal, NHVDCDCFault= No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool for developing and debugging high-speed CAN networks |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Invalid CAN signal attributes or SNA requirements.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_03 |
| Customer Req ID | SDS-HVDCDC-85 |
| Milestone | PF2.0 |
| Purpose1 | Each CAN Signal received or transmitted by the HVDCDC shall have a definition of "Signal not available", which indicates that the signal content is invalid. For physical quantities, "SNA" shall be defined as the highest positive number. For state signals, "SNA" shall be defined as a dedicated state, with the name "Unknown" or "SNA" or "Invalid". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | CAN signal parameter without “SNA” validation |
| Output(s) | Status of signal for which value is set to its maximum value is set to SNA(Invalid_SingalName_NW). Here, SignalName is the respective signal name from PDU structure |
| Process | Software shall send the received fault/status information to MCAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. As this requirement is to verify SNA value. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### CAN signal level upper limit requirements on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_04 |
| Customer Req ID | SDS-HVDCDC-265 |
| Milestone | PF2.0 |
| Purpose1 | All transmit CAN signals shall be capped to a maximum value of "SNA" -1. This is so that "SNA" can be differentiated from signal railed at upper limit. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | CAN signal parameter without “SNA” validation. The Maximum value should be "SNA" -1 |
| Output(s) | Status of signal for which value is set to its maximum value is set to SNA-1. Here, SignalName is the respective signal name from PDU structure |
| Process | Observe that the Software shall send the maximum value of SNA-1 over the MCAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. As this requirement is to verify SNA value. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. Verify that the maximum value of all the transmit CAN signals shall be SNA-1 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### ‘Operation’ commands receive feature on HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_05 |
| Customer Req ID | SDS-HVDCDC-91 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC shall must be able to receive operation commands from the vehicle's CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Vehicle CAN transmit 0x0 "Standby” operation command to the HVDCDC Vehicle CAN transmit 0x1 "Enable" operation command to the HVDCDC Vehicle CAN transmit 0x2 "Safe State" operation command to the HVDCDC |
| Output(s) | HVHVDCDC should execute these commands as per the command function. |
| Process | - Vehicle CAN broadcast the 0x0 "Standby” operation command to the HVHVDCDC go into the Standby mode. - Vehicle CAN broadcast the 0x1 "Enable" operation command to the HVHVDCDC go into the Enable mode. - Vehicle CAN broadcast the 0x2 "Safe State" operation command to the HVHVDCDC go into the Safe State mode. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC= Normal, HVDCDC_Status1=Normal, HVDCDC_Debug=Normal, NHVDCDCFault= No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCStatus signal valid value range is = 0 - 7 |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN networks. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Fault unlatching request over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_06 |
| Customer Req ID | SDS-HVDCDC-92 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC must be able to receive a fault unlatching request from the vehicle's CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Vehicle CAN transmit 0x1 "unlatch" request to the HVDCDC for fault unlatching. |
| Output(s) | In HVHVDCDC a fault is "latched", the act of "unlatching" removes the system ability to forcefully continue to broadcast the active fault state. |
| Process | - Vehicle CAN broadcast the 0x1 "unlatch" request to HVDCDC for the clear fault in Fault unlatching state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus=Faulty(6), NHVDCDCFault= Faulty. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NFaultUnlatchRequestHVDCDC signal valid value range is = 0 - 3. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN networks. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### VIN related messages over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_07 |
| Customer Req ID | SDS-HVDCDC-338 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive VIN related messages. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Information/messages related to VIN shall be received over CAN |
| Output(s) | 1. The HVDCDC shall handle VIN related messages. 2. The HVDCDC shall Store VIN related messages into memory. |
| Process | 1. Verify that the VIN related messages shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle VIN related messages. 2. The HVDCDC shall Store VIN related messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Total distance information feature over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_08 |
| Customer Req ID | SDS-HVDCDC-339 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the total distance in km IPART1Totalizer from the ZC1. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Total distance in km IPART1Totalizer from the ZC1 shall be received over CAN |
| Output(s) | 1. The HVDCDC shall handle the total distance in km IPART1Totalizer received from the ZC1. 2. The HVDCDC shall Store the total distance in km IPART1Totalizer received from the ZC1 into memory. |
| Process | Verify that the total distance in km IPART1Totalizer from the ZC1 shall receive, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the total distance in km IPART1Totalizer received from the ZC1. 2. The HVDCDC shall Store the total distance in km IPART1Totalizer received from the ZC1 into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### CentralTimePDU related messages over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_09 |
| Customer Req ID | SDS-HVDCDC-340 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the CentralTimePDU messages from the ZC1. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Information/messages related to CentralTimePDU shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle CentralTimePDU messages from the ZC1. 2. The HVDCDC shall Store CentralTimePDU messages from the ZC1. |
| Process | 1. Verify that the CentralTimePDU messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the CentralTimePDU messages. 2. The HVDCDC shall Store the CentralTimePDU messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Receive the Outside Temperature (TAmbient) in Hybrid_Therm_Sts_PDU from the ZC1.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_36 |
| Customer Req ID | SDS-HVDCDC-429 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the Outside Temperature (TAmbient) in Hybrid_Therm_Sts_PDU from the ZC1. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. The Signal “TAmbientRaw” of “HVAC_Status_PDU_Hybrid_CAN” shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle HVAC_Status_PDU_Hybrid_CAN messages from the ZC1. 2. The HVDCDC shall Store information from HVAC_Status_PDU_Hybrid_CAN messages from the ZC1. |
| Process | 1. Verify that the HVAC_Status_PDU_Hybrid_CAN messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the HVAC_Status_PDU_Hybrid_CAN messages. 2. The HVDCDC shall Store the information from HVAC_Status_PDU_Hybrid_CAN messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Receive the NIgnitionStatus in VehState_PDU from ZC1.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_37 |
| Customer Req ID | SDS-HVDCDC-430 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the NIgnitionStatus in VehState_PDU from ZC1. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Information/messages related to NIgnitionStatus in VehState_PDU shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle VehState_PDU messages from the ZC1. 2. The HVDCDC shall Store VehState_PDU messages from the ZC1. |
| Process | 1. Verify that the VehState_PDU messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the VehState_PDU messages. 2. The HVDCDC shall Store the VehState_PDU messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Receive the nEngine in ECM_4_PDU from ZC1.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_38 |
| Customer Req ID | SDS-HVDCDC-431 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the nEngine in ECM_4_PDU from ZC1. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Information/messages related to ECM_4_PDU shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle ECM_4_PDU messages from the ZC1. 2. The HVDCDC shall Store ECM_4_PDU messages from the ZC1. |
| Process | 1. Verify that the ECM_4_PDU messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the ECM_4_PDU messages. 2. The HVDCDC shall Store the ECM_4_PDU messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### HVDCDC shall receive the Battery Voltage.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_39 |
| Customer Req ID | SDS-HVDCDC-432 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the Battery Voltage. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. The Battery voltage signal vLVBattery@LV_Batt_HybridCAN_PDU shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle Battery voltage vLVBattery@LV_Batt_HybridCAN_PDU messages from the ZC1. 2. The HVDCDC shall Store Battery voltage vLVBattery@LV_Batt_HybridCAN_PDU messages from the ZC1. |
| Process | 1. Verify that the Battery voltage vLVBattery@LV_Batt_HybridCAN_PDU messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the Battery voltage vLVBattery@LV_Batt_HybridCAN_PDU messages. 2. The HVDCDC shall Store the Battery voltage vLVBattery@LV_Batt_HybridCAN_PDU messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Receive the ZC1_ObdMode4_Request PDU for OBD diagnostics..

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_40 |
| Customer Req ID | SDS-HVDCDC-433 |
| Milestone | PreC-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall receive the ZC1_ObdMode4_Request PDU for OBD diagnostics. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The CAN communication is established 2. Information/messages related to ZC1_ObdMode4_Request PDU shall be received from ZC1 over CAN |
| Output(s) | 1. The HVDCDC shall handle ZC1_ObdMode4_Request PDU messages from the ZC1. 2. The HVDCDC shall Store ZC1_ObdMode4_Request PDU messages from the ZC1. |
| Process | 1. Verify that the ZC1_ObdMode4_Request PDU messages from the ZC1shall be received, handled and store by HVDCDC convertor |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State 2. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the below functionality perform by HVDCDC 1. The HVDCDC shall handle the ZC1_ObdMode4_Request PDU messages. 2. The HVDCDC shall Store the ZC1_ObdMode4_Request PDU messages into memory. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Operating mode status over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_10 |
| Customer Req ID | SDS-HVDCDC-94 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC shall broadcast the operating mode status to the vehicle's CAN network as per specified in SharePoint PDU cycle time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | HVHVDCDC shall broadcast below operating mode status to the vehicle's CAN network 0x0 "Standby" 0x1 "Normal" 0x2 "Enable power module" 0x3 "Precharge 400V" 0x4 "Disable_power_module" 0x5 "Discharge 400V" 0x6 "Faulty" |
| Output(s) | In HVDCDC Software the all-state will reflect the operating mode status when they are working. |
| Process | - The HVDCDC Software working at Standby Phase the status will reflect 0x0 "Standby" mode. - The HVDCDC Software working at HV_ON Enabling Phase the status will reflect 0x2 ”Enable Power Module” mode. - The HVDCDC Software working at HV_ON Precharge Phase the status will reflect 0x3 “ Precharge 400v” mode. - The HVDCDC Software working at HV_ON Normal Phase the status will reflect 0x1 ”Normal” mode. - The HVDCDC Software working at Discharge Disabling the converter power module Phase the status will reflect 0x4 "Disable_power_module" mode. - The HVDCDC Software working at Discharge Phase (Open the solid state switch on the 400V side, Actively discharge 400V side DC Link, Disable active discharge on 400V side DC Link) the status will reflect 0x5 ” Discharge 400V” mode. - HVDCDC Application at Fault_Unlatching and Faulty phase the status will reflects 0x6 “Faulty” mode. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 both are off |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. UART to check operating mode status. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Fault levels over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_11 |
| Customer Req ID | SDS-HVDCDC-95 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC shall broadcast the fault levels to the vehicle's CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC shall broadcast the fault levels arises in state machine are given below: NHVDCDCFault = 0X0 "No Fault" 0X1 "Level 0" 0X2 "Level 1" 0X3 "Level 2" |
| Output(s) | The HVHVDCDC shall detect the fault and the fault levels to the vehicle's CAN network. |
| Process | - The HVHVDCDC shall detect the Level 2 fault in the state machine at following condition : HVDCDC needs to be disabled to protect itself / The 400v DC Link needs to be discharged to protect the 400v HV components / The vehicle needs to open HV battery contactor to isolate the failure effects from the rest of HV components. The HVHVDCDC send NHVDCDCFault = 0X3 "Level 2" message over vehicle's CAN. - The HVHVDCDC shall detect the Level 1 fault in the state machine at following condition : HVDCDC or the vehicle needs to derates its performance to mitigate the failure effects. The HVHVDCDC send NHVDCDCFault = 0X2 "Level 1" message over vehicle's CAN. - The HVHVDCDC shall detect the Level 0 fault in the state machine at following condition : The failure effects are invisible to rest of HV system / driver and HVDCDC or vehicle do not have to alternate its performance to mitigate the failure. The HVHVDCDC send NHVDCDCFault = 0X1 "Level 0" message over vehicle's CAN. - The HVHVDCDC shall detect the No Fault in the state machine. The HVHVDCDC send NHVDCDCFault = 0X1 "No Fault" message over vehicle's CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 both are off |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCFault signal valid value range is 0x0 - 0x3. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### HVIL status over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_12 |
| Customer Req ID | SDS-HVDCDC-96 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVHVDCDC shall broadcast the HVIL status to the vehicle's CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC HVIL status from HVIL Monitoring circuit. |
| Output(s) | NHVDCDCHVIL and Fault levels |
| Process | The HVHVDCDC shall detect the status of the HVIL Monitoring circuit in the system. The HVHVDCDC sends an NHVDCDCHVIL message over the vehicle's CAN. And also triggers the fault level when fault detected at HVIL monitoring circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 are off. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCHVIL signal valid value range is 0 – 3. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Measured DC link currents and voltages on 800V side over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_13 |
| Customer Req ID | SDS-HVDCDC-97 (Modified SDS) |
| Milestone | C-Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVHVDCDC shall broadcast the measured DC link currents and voltages on 800V side to the vehicle's CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Provide the volatge at 800V side (High Side) |
| Output(s) | Measured DC link currents and voltages on both the 800 V side and the 400 V side give to the vehicle's CAN network |
| Process | - Measured current and voltage on 800V_emi side of the VICOR module and transmitted it on vehicle's CAN network. - Measured current and voltage on 400V_emi side of the VICOR module and transmitted it on vehicle's CAN network. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No fault). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC800V_emi signal valid value range is 375V - 700V. VHVDCDC400V_emi signal valid value range is 180V - 350V. IHVDCDC800V signal valid value range is -30A - 30A. IHVDCDC400V signal valid value range is -60A - 60A. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signals. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Measured DC link currents and voltages on 400V side over CAN on HVDCDC.

| Term |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REQ ID |  | SWRS_COMM_35 |  |  |  |
| Customer Req ID |  | SDS-HVDCDC-97 |  |  |  |
| Milestone |  | C-Sample/ PF3.0 Phase 2 |  |  |  |
| Purpose1 |  | The HVHVDCDC shall broadcast the measured DC link currents and voltages on 400V side to the vehicle's CAN network. |  |  |  |
| Derived Requirement |  | No Such derived requirement is applicable. |  |  |  |
| Access Restrictions |  | No Specific access restrictions is applicable. |  |  |  |
| Input(s) |  | 1. Provide the voltage at 400V side (high Side) |  |  |  |
| Output(s) |  | Measured DC link currents and voltages on the 400 V side give to the vehicle's CAN network |  |  |  |
| Process |  | Measured current and voltage on 400V_emi side of the VICOR module and transmitted it on vehicle's CAN network. |  |  |  |
| Mandatory Fields |  | No such mandatory fields is applicable. |  |  |  |
| Pre-Loaded Values |  | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No fault). |  |  |  |
| Default Values |  | Default values shall be applicable with respect to Pre-Loaded Values |  |  |  |
| Valid range of Values |  | VHVDCDC800V_emi signal valid value range is 375V - 700V. VHVDCDC400V_emi signal valid value range is 180V - 350V. IHVDCDC800V signal valid value range is -30A - 30A. IHVDCDC400V signal valid value range is -60A - 60A. |  |  |  |
| Data Latency Period |  | Data latency period is not applicable |  |  |  |
| Data Retention Period |  | Data Retention period is not applicable. |  |  |  |
| Data Rate/ <Daily> Number of transaction |  | Data rate is not applicable. |  |  |  |
| External Events |  | External event is not required. |  |  |  |
| Temporal Events |  | Temporal event is not applicable. |  |  |  |
| Validation Rules/ Verification criteria2 |  | CANape or CAN BUS Analyzer is a software tool Validating CAN Signals. |  |  |  |
| Constraints |  | Constraints is not applicable. |  |  |  |
| Effects on other systems/sub system |  | No effects on other system |  |  |  |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  | YES |  |  |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  | YES |  |  |  |
| Critical (Yes/No) |  | YES |  |  |  |
| Acceptance Criteria |  | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |  |  |  |
| #### Measured DC link currents and voltages on 400V side over CAN on HVDCDC. |  |  |  |
| Term |  |  | Description |  |  |
| REQ ID |  |  | SWRS_COMM_14 |  |  |
| Customer Req ID |  |  | SDS-HVDCDC-276 |  |  |
| Milestone |  |  | B-Sample/PF2.0 |  |  |
| Purpose1 |  |  | The HVDCDC shall broadcast the measured 400V side power module voltage measurement using the following signals onto the vehicle CAN network. VHVDCDCPwrModule400V |  |  |
| Derived Requirement |  |  | No Such derived requirement is applicable. |  |  |
| Access Restrictions |  |  | No Specific access restrictions is applicable. |  |  |
| Input(s) |  |  | 1. Set 800V side (high side) voltage of the power module greater than 480V 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface |  |  |
| Output(s) |  |  | vHVDCDC400V @ HVDCDC_CurrentVoltage1 is equal to VICOR output voltage (400V side (low side) voltage of the power module) VHVDCDCPwrModule400V @ HVDCDC_CurrentVoltage1 is equal to VICOR output voltage (400V side (low side) voltage of the power module) |  |  |
| Process |  |  | The HVDCDC broadcasts the measured the 400V side power module voltage measurement using the signal "VHVDCDCPwrModule400V" onto the vehicle CAN network. |  |  |
| Mandatory Fields |  |  | No such mandatory fields is applicable. |  |  |
| Pre-Loaded Values |  |  | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0 (No Faults) |  |  |
| Default Values |  |  | Default values shall be applicable with respect to Pre-Loaded Values |  |  |
| Valid range of Values |  |  | VHVDCDC400V signal valid value range is 180V - 350V. |  |  |
| Data Latency Period |  |  | Data latency period is not applicable |  |  |
| Data Retention Period |  |  | Data Retention period is not applicable. |  |  |
| Data Rate/ <Daily> Number of transaction |  |  | Data rate is not applicable. |  |  |
| External Events |  |  | External event is not required. |  |  |
| Temporal Events |  |  | Temporal event is not applicable. |  |  |
| Validation Rules/ Verification criteria2 |  |  | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |  |  |
| Constraints |  |  | Constraints is not applicable. |  |  |
| Effects on other systems/sub system |  |  | No effects on other system |  |  |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |  | YES |  |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |  | YES |  |  |
| Critical (Yes/No) |  |  | YES |  |  |
| Acceptance Criteria |  |  | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |  |  |
| #### Sign rules for DC link current measurements on both the 800V and 400V sides. |  |  |  |  |  |  |  |
| Term | Description |  |  |  |  |  |
| REQ ID | SWRS_COMM_15 |  |  |  |  |  |
| Customer Req ID | SDS-HVDCDC-98 |  |  |  |  |  |
| Milestone | A-Sample/ PF1.0 |  |  |  |  |  |
| Purpose1 | The HVHVDCDC applies the below sign rules for DC link current measurements on both the 800V and 400V sides. - A positive value indicates buck operation (HV battery discharge). - A negative value indicates boost mode (HV battery charging). |  |  |  |  |  |
| Derived Requirement | SWRS_COMM_08 |  |  |  |  |  |
| Access Restrictions | No Specific access restrictions is applicable. |  |  |  |  |  |
| Input(s) | IHVDCDC800V IHVDCDC400V |  |  |  |  |  |
| Output(s) | The HVHVDCDC measured DC link current at HV battery discharging: +(IHVDCDC800V) +(IHVDCDC400V) The HVHVDCDC measured DC link current at HV battery charging: -(IHVDCDC800V) -(IHVDCDC400V) |  |  |  |  |  |
| Process | - The HVHVDCDC measured DC link current , When HV battery discharging & starts current flowing in ‘+’ direction towards VICOR module is +(IHVDCDC800V) and away from VICOR module is +(IHVDCDC400V) - The HVHVDCDC measured DC link current , When HV battery charging & starts current flowing in ‘-’ direction towards VICOR module is -(IHVDCDC400V) and away from VICOR module is -(IHVDCDC800V) |  |  |  |  |  |
| Mandatory Fields | No such mandatory fields is applicable. |  |  |  |  |  |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No fault). |  |  |  |  |  |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |  |  |  |  |  |
| Valid range of Values | IHVDCDC800V signal valid value range is -30A - 30A. IHVDCDC400V signal valid value range is -60A - 60A. |  |  |  |  |  |
| Data Latency Period | Data latency period is not applicable |  |  |  |  |  |
| Data Retention Period | Data Retention period is not applicable. |  |  |  |  |  |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |  |  |  |  |  |
| External Events | External event is not required. |  |  |  |  |  |
| Temporal Events | Temporal event is not applicable. |  |  |  |  |  |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signals. |  |  |  |  |  |
| Constraints | Constraints is not applicable. |  |  |  |  |  |
| Effects on other systems/sub system | No effects on other system |  |  |  |  |  |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |  |  |  |  |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |  |  |  |  |  |
| Critical (Yes/No) | YES |  |  |  |  |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |  |  |  |  |  |

#### Broadcast the measured critical component temperatures over CAN on HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_16 |
| Customer Req ID | SDS-HVDCDC-99 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVHVDCDC shall broadcast the measured critical component temperatures to the vehicle's CAN network using the below signals for the EDSU to provide derating protection. THVDCDC1 THVDCDC2 THVDCDC3 THVDCDC4 THVDCDC5 |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Temperatures of the critical components: 1) Temperature of the 800V EMI filter. (Temp_EMI_800V) 2) Temperature of the VICOR Module. (Temp_Vicor_A) 3) Temperature of the VICOR Module. (Temp_Vicor_B) 4) Temperature of the Solid-State Switch. (Temp_SSS) 5) Temperature of the 400V EMI filter. (Temp_EMI_400V) |
| Output(s) | Measured Temperatures of the critical components sent to the vehicle's CAN network. |
| Process | - The temperature of the VICOR Module (Temp_Vicor_B) me) is measured directly by the ADC_SPI to the controller. - The temperature of the 800V EMI filter (Temp_EMI_800V) and the Temperature of the 400V EMI filter (Temp_EMI_400V) measured SPI-ADC to the controller. - Temperature of the Solid-State Switch. (Temp_SSS) measured SPI-ADC to the controller. - All measured temperatures are sent to the vehicle's CAN network. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No Faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | 400v over temperature value in degC 90°C 400v under temperature value in degC -35°C 800v over temperature value in degC 90°C 800v under temperature value in degC -35°C Vicor over temperature value in degC 90°C Vicor under temperature value in degC -35°C |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Vehicle CAN average cycle time of all PDUs over CAN

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_17 |
| Customer Req ID | SDS-HVDCDC-101 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC shall have vehicle CAN average cycle time of all PDUs in the network must be within ±2% of the specified PDU cycle time with a maximum of ±10 ms. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Required CAN cycle time for each PDU given by the customer |
| Output(s) | The average PDU cycle time for HVDCDC_Debug_PDU PDU is within 10ms(±2%). The average PDU cycle time for HVDCDC_NBM PDU is within 20ms(±2%). The average PDU cycle time for HVDCDC_Status1 PDU is within 10ms(±2%). The average PDU cycle time for HVDCDC_Status2 PDU is within 20ms(±2%). The average PDU cycle time for HVDCDC_Status3 PDU is within 1000ms.(±2%). The average PDU cycle time for HVDCDC_Status4 PDU is within 20ms.(±2%). The average PDU cycle time for HVDCDC_Status5 PDU is within 20ms.(±2%) The average PDU cycle time for HVDCDCPin_Sts PDU is within 20ms.(±2%) |
| Process | Observe the delta t time within consecutive frames for all CAN signals using the CANalyzer tool by monitoring the minimum and maximum time for the below-mentioned PDUs, HVDCDC_Debug_PDU HVDCDC_Status1 HVDCDC_Status2 HVDCDC_Status3 HVDCDC_Status4 HVDCDC_Status5 HVDCDCPin_Sts |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC= Normal, HVDCDC_Status1=Normal, HVDCDC_Debug=Normal, NHVDCDCFault= No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast all the information reported by the power module on to the debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_18 |
| Customer Req ID | SDS-HVDCDC-103 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVHVDCDC shall broadcast all the information reported by the power module on to the debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Power module information like internal fault of power module and status received over i2c |
| Output(s) | Debug CAN message related to power module. |
| Process | HVHVDCDC shall send the received fault/status information to Debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No Faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is as per data base file(.dbc) for all signal. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Debug CAN message shall be verified on Canalyzer on Debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### CAN message for internal state of the state machine transitions.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_19 |
| Customer Req ID | SDS-HVDCDC-104 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVDCDC Software should use NHVDCDCStatusDebug on debug CAN to send internal state of state machine transitions. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC shall broadcast the NHVDCDCStatusDebug (for debugging purpose of state transition) to the vehicle's CAN network using the below signals: 0x0 "Standby" 0x2 "Enable power module" 0x3 "PrechargeLS_ClosePrechargeCtt" 0x4 "PrechargeLS_CloseSolidStateSw" 0x5 "PrechargeLS_OpenPrechargeCtt" 0x6 "Normal" 0x7 "Shutdown_DisablePM" 0x8 "Shutdown_EnableAD400V" 0x9 "Shutdown_DisableAD400V" 0xA "Shutdown_OpenSolidStateSw" 0xB "Faulty_OpenSolidStateSw" 0xC "Faulty_Enable400VAD" 0xD "Faulty_Disable400VAD" 0xE "Faulty_DisablePM" 0X0F "FaultyUnlatch_ClearPM" 0x10 "FaultyUnlatch_ClearSys" |
| Output(s) | All state transitions debug status is sent to the DebugCAN. |
| Process | - 0x0 "Standby" Debug Message used for informing the status of system in StandBy Mode - 0x2 "Enable power module" Debug Message used for informing the status of the system working on HV and enables the converter power module - 0x3 "PrechargeLS_ClosePrechargeCtt" Debug Message used for informing the status of Close the precharge contactors - 0x4 "PrechargeLS_CloseSolidStateSw" Debug Message used for informing the status of Close the solid state switch on 400V side DC Link; - 0x5 "PrechargeLS_OpenPrechargeCtt" Debug Message used for informing the status of Open the precharge contactors; - 0x6 "Normal" Debug Message used for informing the status of system working at normal condition. - 0x7 "Shutdown_DisablePM" Debug Message used for informing the status of Disable the converter power module; - 0x8 "Shutdown_EnableAD400V" Debug Message used for informing the status of Actively discharge 400V side DC Link; - 0x9 "Shutdown_DisableAD400V" Debug Message used for informing the status of Disable active discharge on 400V side DC Link; - 0xA "Shutdown_OpenSolidStateSw" Debug Message used for informing the status of Open the solid state switch on the 400V side; - 0xB "Faulty_OpenSolidStateSw" Debug Message used for informing the status of Open the solid state switch on the 400V side; - 0xC "Faulty_Enable400VAD" Debug Message used for informing the status of Actively discharge 400V side DC Link; - 0xD "Faulty_Disable400VAD" Debug Message used for informing the status of Disable active discharge on 400V side DC Link; - 0xE "Faulty_DisablePM" Debug Message used for informing the status of Disable the converter power module; - 0X0F "FaultyUnlatch_ClearPM" Debug Message used for informing the status of Command the converter power module to clear fault. - 0x10 "FaultyUnlatch_ClearSys" Debug Message used for informing the status of Command to unlatch the main controller faults. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No Faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCStatusDebug signal valid value is 0 - 19 and 31. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Debug CAN message NHVDCDCStatusDebug shall be verified on Canalyzer. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast the status of 400V side solid state switch on debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_20 |
| Customer Req ID | SDS-HVDCDC-105 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVHVDCDC uses NHVDCDC400VMainSwitch to broadcast the status of 400V side solid state switch on debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC shall broadcast the NHVDCDC400VMainSwitch to the vehicle's CAN network for the knowing status of the solid-state switch |
| Output(s) | The status of the solid-state switch is ON or OFF. |
| Process | To measure the status of the Solid-state switch via ADC to the SPI Bus. Then this measured value gives to debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC400V signal valid value range is 180V - 350V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast the status of 400V side pre-charge contactor on debug CAN

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_21 |
| Customer Req ID | SDS-HVDCDC-106 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVHVDCDC uses NHVDCDC400VPrechargeSwitch to broadcast the status of 400V side pre-charge contactor on debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC shall broadcast the NHVDCDC400VPrechargeSwitch to the vehicle's CAN network for the knowing status of of 400V side pre-charge contactor. |
| Output(s) | the status of 400V side pre-charge contactor is ON Or OFF. |
| Process | To measure the status of 400V side pre-charge contactor via ADC to the SPI Bus. Then this measured value gives to debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC400V signal valid value range is 180V - 350V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast the status of 400V side active discharge switch on debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_22 |
| Customer Req ID | SDS-HVDCDC-107 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | HVDCDC uses NHVDCDC400VDischargeStatus to send the status of 400V side active discharge switch on debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVHVDCDC shall broadcast the NHVDCDC400VDischargeStatus to the vehicle's CAN network for the knowing status of 400V side active discharge switch. |
| Output(s) | The status of 400V side active discharge switch is ON Or OFF. |
| Process | To measured Status of 400V side active discharge switch via ADC to the SPI Bus. Then this measured value shall be transmitted on debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0(No faults). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC400V signal valid value range is 180V - 350V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal on Debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_23 |
| Customer Req ID | SDS-HVDCDC-108 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | HVDCDC uses NHVDCDCFaultID to send the status of internal fault ID on debug CAN. |
| Derived Requirement | SWRS_COMM_19 |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The fault generated at internally in state machine. |
| Output(s) | The status of the fault generated at internally in state machine will broad cast to the debug CAN. |
| Process | To measured Status of internal fault ID via ADC to the SPI Bus. Then this measured value gives to debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 6, NHVDCDCFault = 1 (Faulty). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCFaultID signal valid value range is 0 - 255. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Fault levels to the vehicle CAN network for defining the faults and its reaction on CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_24 |
| Customer Req ID | SDS-HVDCDC-174 |
| Milestone | A-Sample/ PF1.0 |
| Purpose1 | The HVHVDCDC shall have fault level categorization, diagnostic debounces, fault reactions on HVDCDC system and the vehicle, and the CAN interface for HVDCDC to report the fault level to the vehicle CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The fault levels to the vehicle CAN network for defining the faults and its reaction on HVDCDC system Or Vehicle. |
| Output(s) | The HVHVDCDC CAN Output NHVDCDCFault |
| Process | - At No Fault: - Fault Severity: NA - Fault categorization guide: NA - HVDCDC CAN Output: NHVDCDCFault = 0X0 "No Fault" - Debounce strategy: NA - HVDCDC Fault Reaction: Normal Operation - HVDCDC Fault Recovery Mechanism: NA - Set DTC in HVDCDC: NA - Vehicle reaction: Normal Operation - At Level 0: - Fault Severity: LOW - Fault categorization guide: 1) The failure effects are invisible to rest of HV system / driver; and 2) HVDCDC or vehicle do not have to alternate its performance to mitigate the failure. - HVDCDC CAN Output: NHVDCDCFault = 0X1 " Level 0" - Debounce strategy: HVDCDC shall apply appropriate debounce strategy (e.g. counter-based or timer-based) to confirm the fault. This is required to prevent fault toggling or serve system rection on false diagnostic, for robustness. - HVDCDC Fault Reaction: Normal Operation - HVDCDC Fault Recovery Mechanism: NA - Set DTC in HVDCDC: Yes - Vehicle reaction: Normal Operation - At Level 1: - Fault Severity: MEDIUM - Fault categorization guide: HVDCDC or the vehicle needs to derates its performance to mitigate the failure effects. - HVDCDC CAN Output: NHVDCDCFault = 0X2 " Level 1" - Debounce strategy: HVDCDC shall apply appropriate debounce strategy (e.g. counter-based or timer-based) to confirm the fault. This is required to prevent fault toggling or serve system rection on false diagnostic, for robustness. - HVDCDC Fault Reaction: Derated Operation (if applicable). - HVDCDC Fault Recovery Mechanism: Self-heatable. The HVDCDC shall unlatch the Level 1 fault rection as soon as the failure is not present( diagnostic tests pass, debounce may apply) - Set DTC in HVDCDC: Yes - Vehicle reaction: Derated operation. Display warning to driver. - At Level 2: - Fault Severity: HIGH - Fault categorization guide: 1) HVDCDC needs to be disabled to protect itself; and / or 2) The 400v DC Link needs to be discharged to protect the 400v HV components; and/ or 3) The vehicle needs to open HV battery contactor to isolate the failure effects from the rest of HV components. - HVDCDC CAN Output: NHVDCDCFault = 0X3 " Level 2" - Debounce strategy: HVDCDC shall apply appropriate debounce strategy (e.g. counter-based or timer-based) to confirm the fault. This is required to prevent fault toggling or serve system rection on false diagnostic, for robustness. - HVDCDC Fault Reaction: Depending on failure modes the HVDCDC power shall be disabled or expect the EDSU to command it to be disabled safety. The HVDCDC shall actively discharge the 400V DC link when possible. Keep CAN communication. - HVDCDC Fault Recovery Mechanism: Non Self-heatable. The HVDCDC shall latch the Level 2 fault rection until the unit sleeps or the unit receives the unlatch from the vehicle control. - Set DTC in HVDCDC: Yes - Vehicle reaction: Depending on the failure modes, the vehicle will command the HV systems to be disabled immediately or when its safe to do so. Vehicle limhome when possible. Display warning to driver. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15(IGN) is Off |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NHVDCDCFaultID signal valid value range is 0 - 255. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Fault levels to the vehicle CAN network for defining the faults and its reaction on CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_25 |
| Customer Req ID | SDS-HVDCDC-276 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall broadcast the measured 400V side power module voltage measurement using the following signals onto the vehicle CAN network. VHVDCDCPwrModule400V |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set 800V side (high side) voltage of the power module greater than 480V 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface |
| Output(s) | vHVDCDC400V @ HVDCDC_CurrentVoltage1 is equal to VICOR output voltage (400V side (low side) voltage of the power module) VHVDCDCPwrModule400V @ HVDCDC_CurrentVoltage1 is equal to VICOR output voltage (400V side (low side) voltage of the power module) |
| Process | The HVDCDC broadcasts the measured the 400V side power module voltage measurement using the signal "VHVDCDCPwrModule400V" onto the vehicle CAN network. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC400V signal valid value range is 180V - 350V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### 800V side power module voltage measurement information on CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_26 |
| Customer Req ID | SDS-HVDCDC-330 |
| Milestone | Pre-C Sample/PF3.0 Phase - 1 |
| Purpose1 | The measured 800V side power module voltage measurement shall be broadcast on vehicle CAN network using following Signal. VHVDCDCPwrModule800V |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set 800V side (high side) voltage of the power module at any voltage level |
| Output(s) | VHVDCDCPwrModule800V @ HVDCDC_CurrentVoltage2 is equal to VICOR input voltage (800V side (high side) voltage of the power module) |
| Process | The HVDCDC broadcasts the measured the 800V side power module voltage measurement using the signal "VHVDCDCPwrModule800V" onto the vehicle CAN network. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC800V signal valid value range is greater than 480V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool VHVDCDCPwrModule800V @ HVDCDC_Status2 is equal to VICOR input voltage (800V side (high side) voltage of the power module |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast critical power module PMBus output information on CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_27 |
| Customer Req ID | SDS-HVDCDC-283 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall broadcast the critical power module PMBus output onto the PDU: HVDCDC_NBM and HVDCDC_NBM2. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | 1) Received Values for signals iHSHVDCDC @ HVDCDC_NBM_PDU, vHSHVDCDC @ HVDCDC_NBM_PDU, iLSHVDCDC @ HVDCDC_NBM_PDU, vLSHVDCDC @HVDCDC_NBM_PDU and THVDCDCNBM @ HVDCDC_NBM_PDU on MCAN interface. 2. Received Values for signals NHVDCDCNBMStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMFLT @ HVDCDC_NBM2_PDU, NHVDCDCNBMVLOStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMILOStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMVHIStatus @HVDCDC_NBM2_PDU, NHVDCDCNBMTempStatus @ HVDCDC_NBM2_PDU NHVDCDCNBMCMLStatus @ HVDCDC_NBM2_PDU and NHVDCDCNBMMFRSpecStatus @ HVDCDC_NBM2_PDU on MCAN interface. |
| Process | 1. Check for CAN signals iHSHVDCDC @ HVDCDC_NBM_PDU, vHSHVDCDC @ HVDCDC_NBM_PDU, iLSHVDCDC @ HVDCDC_NBM_PDU, vLSHVDCDC @HVDCDC_NBM_PDU, THVDCDCNBM @ HVDCDC_NBM_PDU and NHVDCDCNBMSts @ HVDCDC_NBM_PDU on MCAN interface. 2. Check for CAN signals NHVDCDCNBMStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMFLT @ HVDCDC_NBM2_PDU, NHVDCDCNBMVLOStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMILOStatus @ HVDCDC_NBM2_PDU, NHVDCDCNBMVHIStatus @HVDCDC_NBM2_PDU, NHVDCDCNBMTempStatus @ HVDCDC_NBM2_PDU NHVDCDCNBMCMLStatus @ HVDCDC_NBM2_PDU and NHVDCDCNBMMFRSpecStatus @ HVDCDC_NBM2_PDU on MCAN interface. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | HVDCDC_NBM PDU’s signal valid value’s is as per (.dbc) file. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN Signal as mentioned in the input and output. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast critical power module PMBus output information on CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_28 |
| Customer Req ID | SDS-HVDCDC-267 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall broadcast the critical debug CAN information of SDS Requirements (SDS-HVDCDC-104, 105, 106,107,108) onto the vehicle CAN network for development purpose. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set KL30 and KL15(IGN) to On 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 4. Once the HVDCDC application is reached in "Normal" mode send NHVDCDCCommand as to 0x00 ("Standby") using the CAN interface 5. Once the HVDCDC application is reached in "Standby" mode send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 6. Once the HVDCDC application reached into "Normal" mode send NHVDCDCCommand as 0x02 ("Safe State") using the CAN interface |
| Output(s) | Application shall provide CAN messages status as below, 1) NHVDCDCStatus @ HVDCDC_Status1 is set to 0x0 (Standby) 2)CAN messages are observed sequentially on the MCAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x2 ("Enable power module") NHVDCDCStatus @ HVDCDC_Status1 is set to 0x3 ("Precharge 400V") NHVDCDCStatus @ HVDCDC_Status1 is set to 0x1 ("Normal") NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x4("Disable_power_module") NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x5 (“Discharge 400V”) NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x6 (“Faulty") |
| Process | The HVHVDCDC broadcasts the operating mode status to the vehicle's CAN network as per specified in SharePoint PDU cycle time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast all the information reported by the power module onto the debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_29 |
| Customer Req ID | SDS-HVDCDC-103 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall broadcast all the information reported by the power module onto the debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Check for signals HVDCDC_VED_pin_sts @ HVDCDCPin_sts and HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU on DCAN interface. 2. Check for CAN signals iHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, vHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, iLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, vLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, THVDCDCNBM @ HVDCDC_PM_I2C_Info_PDU and NHVDCDCNBMSts @ HVDCDC_PM_I2C_Info_PDU on DCAN interface. 3. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 4. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | 1) Received value for signals HVDCDC_VED_pin_sts @ HVDCDCPin_sts is equal to 0 on Vehicle CAN and HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU is equal to 0 on DCAN. 2) Received value for signals iHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, vHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, iLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, vLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, THVDCDCNBM @ HVDCDC_PM_I2C_Info_PDU and NHVDCDCNBMSts @ HVDCDC_PM_I2C_Info_PDU on DCAN interface all equal to 0. 3) Received high side current value on signal iHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, received high side voltage on signal vHSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, received low side current on signal iLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, received low side voltage on signal vLSHVDCDC @ HVDCDC_PM_I2C_Info_PDU, received VICOR temperature value on signal THVDCDCNBM @ HVDCDC_PM_I2C_Info_PDU and receive value equal to 0 on signal NHVDCDCNBMSts @ HVDCDC_PM_I2C_Info_PDU on DCAN interface. |
| Process | The HVDCDC broadcasts the critical power module PMBus output onto the PDU "HVDCDC_NBM". |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_30 |
| Customer Req ID | SDS-HVDCDC-298 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall broadcast the internal temperature of the microcontroller using T HVDCDCuC onto the debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Internal temperature of the microcontroller using HVDCDCuC on Debug CAN. |
| Output(s) | 1) Below CAN messages are observed on the DCAN interface THVDCDCuC @ HVDCDC_MISC_Info_PDU shows the microcontroller's internal temperature. 2) Below CAN messages are observed on the DCAN interface THVDCDCuC @ HVDCDC_MISC_Info_PDU shows the micro-controller internal temperature. |
| Process | The HVDCDC broadcasts the internal temperature of the microcontroller using THVDCDCuC onto the debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Broadcast all the ADC channels readings onto the debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_31 |
| Customer Req ID | SDS-HVDCDC-324 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall broadcast all the ADC channels readings onto the debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 3. Check ADC_CH0 to ADC_CH4 signals from HVDCDC_ADC_Info_PDU0 PDU, ADC_CH5 to ADC_CH9 signals from HVDCDC_ADC_Info_PDU1 PDU, ADC_CH10 to ADC_CH14 signals from HVDCDC_ADC_Info_PDU2 PDU and ADC_CH15 signal from HVDCDC_ADC_Info_PDU3 PDU on DCAN interface. |
| Output(s) | The HVDCDC shall broadcast of all ADC raw data over DCAN as below, Received values for signals 400V Bus @ EMI, 400V Bus @ VICOR, +Amps @ 400V Bus, -Amps @ 400V Bus, and 800V Bus @ EMI at ADC_CH0 to ADC_CH4 signals from HVDCDC_ADC_Info_PDU0 PDU respectively on DCAN interface. Received values for signals 800V Bus @ VICOR, +Amps @ 800V Bus, -Amps @ 800V Bus, 400V Bus EMI Filter Temperature, 800V Bus EMI Filter Temperature at ADC_CH5 to ADC_CH9 signals from HVDCDC_ADC_Info_PDU1 PDU respectively on DCAN interface. Received values for signals VICOR Temperature, SSS Temperature, Precharge / Discharge Status, eFuse Status, Local 5VS rail at ADC_CH10 to ADC_CH14 signals from HVDCDC_ADC_Info_PDU2 PDU respectively on DCAN interface. Received values for signal Local 5VV rail at ADC_CH15 signal from HVDCDC_ADC_Info_PDU3 PDU on DCAN interface. |
| Process | The HVDCDC broadcasts all the ADC channel readings onto the debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Controller output PM I2C interface.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_32 |
| Customer Req ID | SDS-HVDCDC-325 |
| Milestone | C-Sample / PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report the controller output PM I2C and Enables signals: Vicor_SDAc Vicor_SL3ENc Vicor_FLT Vicor_SCLc |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 30V |
| Output(s) | 1. Signal HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU data on DCAN. 2. Signal HVDCDC_VICOR_FLT @ HVDCDC_NBM_PDU data on DCAN. 2. Signal HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU data on DCAN. 2. Signal HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU data on DCAN. |
| Process | The HVDCDC reports the controller output PM I2C and Enables the below-mentioned signals, HVDCDC_VED_pin_sts VICOR_FLT |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool HVDCDC_VED_pin_sts @ HVDCDCPin_sts is equal to 0 on Vehicle CAN Signal HVDCDC_VICOR_FLT @ HVDCDC_NBM2_PDU data on DCAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_33 |
| Customer Req ID | SDS-HVDCDC-327 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall report the HVIL_good_uC |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Check the signal NHVDCDCHVIL @ HVDCDC_Status1 on the CAN interface. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. |
| Process | The HVDCDC reports the HVIL status as 2 when the HVDCDC apllication is powered On. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | HVDCDCHVIL signal valid value range is 0 - 3 |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Reporting status of control IO’s.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_COMM_34 |
| Customer Req ID | SDS-HVDCDC-326 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall report the status of control io’s belows signals: Discharge_ENc Precharge_Enc SSS_Enc |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 200ms right after the 800V voltage stabilise at 480V |
| Output(s) | Application shall provide CAN messages status as below, 1) Below CAN messages are observed on the CAN interface when the application is in the "PrechargeLS_ClosePrechargeCtt" substate of the "precharge 400V" state: Over the DCAN interface, NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x3 ("PrechargeLS_ClosePrechargeCtt") HVDCDC_SSS_ENc @ HVDCDCPin_sts is set to 0 ("OFF") Below CAN messages are observed on the CAN interface when the application is in the "PrechargeLS_CloseSolidStateSw" substate of the "precharge 400V" state: Over the DCAN interface, NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x4 ("PrechargeLS_CloseSolidStateSw") HVDCDC_SSS_ENc @ HVDCDCPin_sts is set to 1 ("ON") 3) Below CAN messages are observed on the CAN interface when the application is in the "Shutdown_OpenEFuse" substate of the "Discharge 400V" state: Over the DCAN interface, NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0xA ("Shutdown_OpenEFuse") HVDCDC_SSS_ENc @ HVDCDCPin_sts is set to 0 ("OFF") |
| Process | The HVDCDC broadcasts the precharge contactor status using HVDCDC_Precharge_ENc onto the debug CAN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: Sleep Requirement

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_01 |
| Customer Req ID | SDS-HVDCDC-117 |
| Milestone | A-Sample/PF1.0 (These requirements are only useful for EOL testing) |
| Purpose1 | The function Sleep/Wake shall put the HVDCDC into a sleep state, where it saves energy. It shall also provide a mechanism for waking the HVDCDC back up. Moreover, CAN network management is part of this function. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - KL15 line shall be removed/OFF. |
| Output(s) | 1. HVDCDC shall go into sleep state. 2. All the peripherals shall achieve the state which is configured for sleep mode |
| Process | - Detect KL15 line signal interrupt. - Verify the signal whether it is sleep signal. - Run the shutdown sequence. - Put PMIC IC into the standby mode. - The TriCore in shutdown state and only SCR core will be active. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure an HVDCDC to detect the KL15 OFF state. 2. Configure an interrupt that can wakeup HVDCDC from Sleep mode. 3. Configure/DE initialize the peripherals before going into sleep mode. 4. The HVDCDC should be in Standby/Run state |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | Need to connect KL15 signal. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the current consumption is reduced and lowest rate. Verify that system shall entered sleep state. Verify that all the peripherals shall be reached to the configured state/disable state |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: Wakeup Requirement

| Term | Description |
| --- | --- |
| REQ ID | SWRS APP_STATE_02 |
| Customer Req ID | SDS-HVDCDC-116, SDS-HVDCDC-123, SDS-HVDCDC-124 These requirements are only useful for EOL testing) |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | Application shall be able to monitor the KL15 line to wake up the system |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | KL15 line must be up/ON |
| Output(s) | 1. HVDCDC should go to stand-by state after wakes up. 2. All the required peripherals shall be initialized after wakeup state is detected |
| Process | - Detect KL15 line signal interrupt. - Verify the signal whether it is wake up signal. - SCR core shall wake up the TriCore. - TriCore shall be powered on and shall move to standby state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. No battery is connected to KL30(+12 V). 2. Configure an HVDCDC to detect the KL15 ON state. 3. Configure an interrupt that can wakeup HVDCDC from Sleep mode. 4. The HVDCDC should be in Sleep state |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | Need to connect KL15 signal. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVDCDC Software shall set CAN Signal NHVDCDCStatus = 0x0 Standby in "Standby" state. The HVDCDC Software shall set CAN Signal NHVDCDCStatusDebug = 0x0 Standby. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: Power module and HVDCDC Communication

| Term | Description |
| --- | --- |
| REQ ID | SWRS APP_STATE_03 |
| Customer Req ID | SDS-HVDCDC-335 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Communication between the power module and HVDCDC via I2C shall be establish when the 800V DC link voltage is above a certain voltage level (HVDCDC_CAL) |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 800V_Vicor (high side) voltage of the power module shall be greater than 30V which can also be a calibratable value. 2. 800V_emi voltage of the power module shall be greater than 30V which can also be a calibratable value. 3. 800V_IND voltage of the power module shall be greater than 30V which can also be a calibratable value. |
| Output(s) | 1. Enabling HVDCDC power module. NHVDCDCStatus = 0x2 Enable power module. 2. The power module shall establish communication with HVDCDC in all state except power down safe State. 3. Communication should establish with power module |
| Process | Below are the conditions establish communication between power module and HVDCDC 1. 800V side DC link voltage > 30V (HVDCDC_CAL) and VHDCDC800V_Vicor > 30V (HVDCDC_CAL) and VHDCDC800V_ind > 30V (HVDCDC_CAL) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC should be in IGN ON State |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | Need to connect KL15 signal. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVDCDC shall set CAN Signal as NHVDCDCStatus = 0x2 Enable power module. The power module shall establish communication with HVDCDC in all state except power down self state. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: Monitor the status of VICOR

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_04 |
| Customer Req ID | SDS-HVDCDC-126 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | Application shall send query to VICOR module with defined interval using PMBus specification and monitor the status of VICOR |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall send the i2c query message to VICOR module to check the communication is established |
| Output(s) | VICOR module shall provide the response once optimum voltage level condition meets(800v) HV battery contactors are closed. |
| Process | Below are the conditions for Enable state to get trigger, 1. 800V side DC link voltage > 480V (HVDCDC_CAL) 2. NHVDCDCCommand = 0x01 ("Enable") Received 3. Power Module Initialized and Communication established |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Application shall be able to read response of VICOR module over I2C bus. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### HV_Initializtion state transition requirement.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_05 |
| Customer Req ID | SDS-HVDCDC-332 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to the "HV_Initializtion state" |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set 800V side (high side) voltage of the power module greater than 480V 2. Send NHVDCDCCommand as 0x01 ("Enable") from EDSU to HVDCDC using the CAN interface |
| Output(s) | Application should be able to transition to the "HV_Initializtion state" The HVDCDC shall set NHVDCDCStatus = 0x8 "HV_Initializtion" state (On Vehicle CAN). The HVDCDC shall set NHVDCDCStatusDebug = 0xF "HV_Initializtion" state (On Debug and Vehicle CAN). |
| Process | Below are the conditions for Enable state to get trigger? 1. 800V side DC link voltage > 480V (HVDCDC_CAL) 2. NHVDCDCCommand = 0x01 ("Enable") Received 3. Power Module Initialized and Communication established 4. NHVDCDCFault != 0x3 ("Level 2" fault) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, 2. The converter power module is initialized and the communication is established |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No any valid range is required |
| Data Latency Period | No data latency period is applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. The DCDC Power Module shall be enabled Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 2. The HVDCDC shall set CAN Signal as NHVDCDCStatus = 0x8 "HV_Initializtion" state. 3. The HVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xF "HV_Initializtion" state. |
| Constraints | No constraints is applicable |
| Effects on other systems/sub system | No any effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Transition requirement for substate “Enable” of state HV_ON.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_06 |
| Customer Req ID | SDS-HVDCDC-129 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Enable” of state HV_ON |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Enable” of HV_ON once meets the conditions mentioned in “Process” Section |
| Output(s) | Application should be able to transition to “Enable” substate. |
| Process | Below are the conditions for Enable state to get trigger? 1. 800V side DC link voltage > 480V (HVDCDC_CAL) 2. NHVDCDCCommand = 0x01 ("Enable") Received 3. Power Module Initialized and Communication established 4. NHVDCDCFault != 0x3 ("Level 2" fault) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, HVDCDCStatusDebug = 0, NHVDCDCFault = 0, |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No any valid range is required |
| Data Latency Period | No data latency period is applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. The DCDC Power Module shall be enabled Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 2. Verify NHVDCDCStatus = 0x2 “Enable power module” on Vehicle CAN. 3. Verify the NHVDCDCStatusDebug = 0x2 “Enable power module” on debug CAN. |
| Constraints | No constraints is applicable |
| Effects on other systems/sub system | No any effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Transition requirement for substate “Precharge” & and entering to “Close_precharge_Ctt” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_07 |
| Customer Req ID | SDS-HVDCDC-130 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Precharge” and enters to “Close_precharge_Ctt” state |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) |
| Output(s) | Application state transitioned to “Close_precharge_Ctt” substate after 500msec |
| Process | Below are the conditions for Precharge state to get trigger: - The power module status confirms to be enabled. - The power module reports the 400V side output greater than a calibration threshold. - The EDSU command NHVDCDCCommand == 0x1 "Enable" remains. - - No Level 2 fault is present. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 2, HVDCDCStatusDebug = 2. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall transit to "Precharge" state and set CAN Signal as NHVDCDCStatus = 0x3 Precharge 400V. - The HVHVDCDC shall close the 400V side precharge contactor. - The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0x3 PrechargeLS_ClosePrechargeCtt. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Transition requirement from substate “Close_precharge_Ctt” to “Close_400V_SolidStateSw” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_08 |
| Customer Req ID | SDS-HVDCDC-131 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit from substate “Close_precharge_Ctt” to “Close_400V_SolidStateSw” state |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Close_400V_SolidStateSw” of Precharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application state transitioned to “Close_400V_SolidStateSw” substate. |
| Process | Below are the conditions for “Close_400V_SolidStateSw” substate to get trigger: - The 400V side DC link is sufficiently pre-charged. - The EDSU command NHVDCDCCommand == 0x1 "Enable" remains. - No Level 2 fault is present. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 3, HVDCDCStatusDebug = 3. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall continue to report NHVDCDCStatus = 0x3 Precharge 400V over CAN. - The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0x4 PrechargeLS_CloseEFuse. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Transition requirement from substate “Close_400V_SolidStateSw” to “Open_precharge_Ctt” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_09 |
| Customer Req ID | SDS-HVDCDC-132 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit from substate “Close_400V_SolidStateSw” to “Open_precharge_Ctt” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Open_precharge_Ctt” of Precharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application state transitioned to “Open_precharge_Ctt” substate. |
| Process | Below are the conditions for “Open_precharge_Ctt” substate to get trigger: - The status of the 400V side solid state switch confirms as closed. - The EDSU command NHVDCDCCommand == 0x1 "Enable" remains. - No Level 2 fault is present. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 3, HVDCDCStatusDebug = 4. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall continue to report NHVDCDCStatus = 0x3 Precharge 400V on Vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x5 PrechargeLS_OpenPrechargeCtt on Debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Transition requirement from substate “Open_precharge_Ctt” substate to “Normal” substate.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_10 |
| Customer Req ID | SDS-HVDCDC-133 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit from substate “Open_precharge_Ctt” substate to “Normal” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Normal” of Precharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application state transitioned to “Normal” substate. |
| Process | Condition for “Normal” substate to get trigger is on completion of opening of precharge contractors. . |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 3, HVDCDCStatusDebug = 5. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall set NHVDCDCStatus = 0x2 Normal on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x6 Normal on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Timeout in transition between Enable and Precharge results in Faulty state (Level 2 fault) of HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_33 |
| Customer Req ID | SDS-HVDCDC-401 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when timeout is detected while transition between Enable and Precharge. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) - Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | - Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Process | 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. Ensure that NHVDCDCFault is equal to 0x0 (no fault) on the CAN trace window. 4. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Enable and Precharge. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 1. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. 3. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Timeout in transition between Close_precharge_Ctt and Close_400V_SolidStateSw results in Faulty state (Level 2 fault) of HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_36 |
| Customer Req ID | SDS-HVDCDC-401 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when timeout is detected while transition between Close_precharge_Ctt and Close_400V_SolidStateSw. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) - Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | Application state transitioned to “Normal” substate. |
| Process | Condition for “Normal” substate to get trigger is on completion of opening of precharge contractors. . |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Enable and Precharge. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall set NHVDCDCStatus = 0x2 Normal on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x6 Normal on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Timeout in transition between Close_400V_SolidStateSw and Open_precharge_ctt results in Faulty state (Level 2 fault) of HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_37 |
| Customer Req ID | SDS-HVDCDC-401 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when timeout is detected while transition between Close_400V_SolidStateSw and Open_precharge_ctt. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) - Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | Application state transitioned to “Normal” substate. |
| Process | Condition for “Normal” substate to get trigger is on completion of opening of precharge contractors. . |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Enable and Precharge. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall set NHVDCDCStatus = 0x2 Normal on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x6 Normal on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Timeout in transition between Open_precharge_ctt (Precharge) and Normal results in Faulty_PowerDn_SafeState state of HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_38 |
| Customer Req ID | SDS-HVDCDC-402 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to Faulty_PowerDn_SafeState state when timeout is detected while transition between Open_precharge_ctt (Precharge) and Normal. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) - Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x15(21) ("DIAGNOSTIC_SM_HVON_TIMEOUT_06_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Process | 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. Ensure that NHVDCDCFault is equal to 0x0 (no fault) on the CAN trace window. 4. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Open_precharge_ctt (Precharge) and Normal. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x15(21) ("DIAGNOSTIC_SM_HVON_TIMEOUT_06_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## State-Machine

### Requirements

#### Application State: Discharge

##### Transition requirements to “Disable” substate of Discharge state from “Normal” substate of HV_ON state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_11 |
| Customer Req ID | SDS-HVDCDC-141 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Disable” substate of Discharge state from “Normal” substate of HV_ON state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Disable” of Discharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application should be able to transition to “Disable” substate. |
| Process | Below are the conditions for Disable state to get trigger: 1. NHVDCDCCommand = 0x00 ("Standby") Received 2. NHVDCDCFault != 0x3 ("Level 2" fault) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 1, HVDCDCStatusDebug = 6. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| zzData Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall set NHVDCDCStatus = 0x4 Disable_power_module on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x7 Shutdown_DisablePM on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_12 |
| Customer Req ID | SDS-HVDCDC-144 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Open_400V_SolidStateSw” substate of Discharge state from “Disable” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Open_400V_SolidStateSw” of Discharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the substate to “Open_400V_SolidStateSw”. |
| Process | The conditions for Open_400V_SolidStateSw substate to get trigger is discharge switch status confirms to be open. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 1, HVDCDCSTatusDebug = 6, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall continue to report NHVDCDCStatus = 0x5 Discharge 400V on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0xA Shutdown_OpenEFuse on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirements to “Active_Discharge_400V” substate of Discharge state from “Open_400V_SolidStateSw” substate

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_13 |
| Customer Req ID | SDS-HVDCDC-142 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Active_Discharge_400V” substate of Discharge state from “Open_400V_SolidStateSw” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Active_Discharge_400V” of Discharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the substate to “Active_Discharge_400V”. |
| Process | The conditions for Active_Discharge_400V substate to get trigger is once solid state switch status confirms to be open. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 5, HVDCDCStatusDebug = A. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall transit to "Active Discharge 400V" state. The HVDCDC shall set NHVDCDCStatus = 0x5 Discharge 400V on vehicle CAN - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x8 Shutdown_EnableAD400V on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirements to “Disable_400V_AD” substate of Discharge state from “Active_Discharge_400V” substate

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_14 |
| Customer Req ID | SDS-HVDCDC-143 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Disable_400V_AD” substate of Discharge state from “Active_Discharge_400V” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Disable_400V_AD” of Discharge once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the substate to “Disable_400V_AD”. |
| Process | The conditions for “Disable_400V_AD” substate to get trigger is below: [VHVDCDC400V < 60V(HVDCDC_CAL) or Fails to meet the timing requirement. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 5, HVDCDCStatusDebug = 8. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall continue to report NHVDCDCStatus = 0x5 Discharge 400V on vehicle CAN. - The HVHVDCDC shall set NHVDCDCStatusDebug = 0x9 Shutdown_DisableAD400V on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirements to “Standby” state from “Discharge” state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_15 |
| Customer Req ID | SDS-HVDCDC-145 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Standby” state from “Discharge” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to state “Standby” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Standby”. |
| Process | The conditions for “Standby” state to get trigger is when the active discharge switch status confirms to be open. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 5, NHVDCDCStatusDebug = 9. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVHVDCDC shall set NHVDCDCStatus = 0x0 "Standby" on vehicle CAN. The HVDCDC shall set NHVDCDCStatusDebug = 0x0 "Standby" on debug CAN. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: HVDCDC Faulty

##### Transition requirement to “Faulty” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_16 |
| Customer Req ID | SDS-HVDCDC-149 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Faulty” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to state “Faulty” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Faulty”. |
| Process | The conditions for “Faulty” state to get trigger is when HVDCDC Level 2 fault is present in the system or safe state command received from EDSU. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 2, NHVDCDCStatusDebug = 2. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVHVDCDC shall transit to the “Faulty” state (safe state) The HVHVDCDC shall set CAN Signal as NHVDCDCStatus = 0x6 “Faulty”. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_17 |
| Customer Req ID | SDS-HVDCDC-150 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Open_400V_ctt” of “Faulty” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Open_400V_ctt” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Open_400V_ctt”. |
| Process | The conditions for “Open_400V_ctt” to get trigger is entry into the “Faulty” state |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 1, NHVDCDCStatusDebug = 6. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall Open the solid state switch on the 400V side. - The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xB "Faulty_OpenEFuse". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Active_Discharge_400V” from substate “Open_400V_ctt” within “Faulty” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_18 |
| Customer Req ID | SDS-HVDCDC-151 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Active_Discharge_400V” from substate “Open_400V_ctt” within “Faulty” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Active_Discharge_400V” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Active_Discharge_400V”. |
| Process | The conditions for “Active_Discharge_400V” to get trigger : The status of the 400V solid state switch status confirms to be open OR Fails to meet the timing requirement. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatusDebug = 0x12, NHVDCDCSTatusDebug = 0xE. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xC "Faulty_EnableAD400V". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Disable_400V_AD” from substate “Active_Discharge_400V” within “Faulty” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_19 |
| Customer Req ID | SDS-HVDCDC-152 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Disable_400V_AD” from substate “Active_Discharge_400V” within “Faulty” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Disable_400V_AD” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Disable_400V_AD”. |
| Process | The conditions for “Disable_400V_AD” to get trigger: The 400V DC link is sufficiently discharged. (VHVDCDC400V < 20V (HVDCDC_CAL)) OR Fails to meet the timing requirement. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStatusDebug = 0xC. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xD "Faulty_Disable400VAD". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Disable_400V_AD” from substate “Active_Discharge_400V” within “Faulty” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_20 |
| Customer Req ID | SDS-HVDCDC-153 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Disable” from substate “Disable_400V_AD” within “Faulty” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Disable” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Disable”. |
| Process | The conditions for “Disable” to get trigger: The status of the 400V active discharge switch confirms to be open. OR Fails to meet the timing requirement. (HVDCDC_CAL sec has expired) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStatusDebug = 0xD. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xE "Faulty_DisablePM". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Open_PreCharge_Ctt and Select_Faulty_Shutdown_Sequence.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_43 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Open_PreCharge_Ctt and Select_Faulty_Shutdown_Sequence transition. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) - Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. - The power module status shall be Enabled. - Precharge contractor confirmed to be open |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Open_PreCharge_Ctt” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Precharge contractor confirmed to be open 5. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 4. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Open_PreCharge_Ctt and Select_Faulty_Shutdown_Sequence. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Open_400V_SolidStateSw_2 and Disable_PM_and_Active_Discharge_400V_2.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_44 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Open_400V_SolidStateSw_2 and Disable_PM_and_Active_Discharge_400V_2. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. Open the solid state switch on the 400V side |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Open_400V_SolidStateSw_2” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Open the solid state switch on the 400V side 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Open_400V_SolidStateSw_2 and Disable_PM_and_Active_Discharge_400V_2. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable_PM_and_Active_Discharge_400V_2 and Disable_400V_AD_2.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_45 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable_PM_and_Active_Discharge_400V_2 and Disable_400V_AD_2. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be disabled. 4. 400V side DC link shall actively discharge |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_PM_and_Active_Discharge_400V_2” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be disabled. 3. 400V side DC link shall actively discharge 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Disable_PM_and_Active_Discharge_400V_2 and Disable_400V_AD_2. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable_400V_AD_2 and Shutdown_Sequence_Completed.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_46 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable_400V_AD_2 and Shutdown_Sequence_Completed. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be disabled. 4. Disable active discharge on 400V side DC link |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT3_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_400V_AD_2” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be disabled. 3. Disable active discharge on 400V side DC link 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT3_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Disable_400V_AD_2 and Shutdown_Sequence_Completed. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_EFS_TIMEOUT3_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable_3 and Open_400V_SolidStateSw_3.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_47 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable_3 and Open_400V_SolidStateSw_3. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. Open the solid state switch on the 400V side |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_3” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Open the solid state switch on the 400V side 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Disable_3 and Open_400V_SolidStateSw_3. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT1_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Open_400V_SolidStateSw_3 and Active_Discharge_400V_3.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_48 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Open_400V_SolidStateSw_3 and Active_Discharge_400V_3. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. Open the solid state switch on the 400V side |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Open_400V_SolidStateSw_3” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Open the solid state switch on the 400V side 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Open_400V_SolidStateSw_3 and Active_Discharge_400V_3. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT2_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable_400V_AD_3 and Shutdown_Sequence_Completed.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_49 |
| Customer Req ID | SDS-HVDCDC-405 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable_400V_AD_3 and Shutdown_Sequence_Completed. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. Open the solid state switch on the 400V side |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT4_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_400V_AD_3” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Open the solid state switch on the 400V side 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 5. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT4_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Disable_400V_AD_3 and Shutdown_Sequence_Completed. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("DIAGNOSTIC_SM_FAULTY_NFS_TIMEOUT4_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Raise Level 0 fault when Timeout detects while transition between Active_Discharge_400V_3 and Disable_400V_AD_3.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_50 |
| Customer Req ID | SDS-HVDCDC-406 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to raise level 0 fault upon Timeout detects while transition between Active_Discharge_400V_3 and Disable_400V_AD_3. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be disabled. 3. Actively discharge on 400V side DC Link. 4. The Timeout parameter shall be calibratable. 5. Diagnostics shall be done in every 10msec. |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 (“level 0") NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall raise level 0 fault after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be disabled. 3. Actively discharge on 400V side DC Link. 4. Configure the timeout parameter 5. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 (“level 0") 6. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") 7. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC shall set calibratable timeout time as HVDCDC_CAL for transition between Active_Discharge_400V_3 and Disable_400V_AD_3. 2. NHVDCDCStatus @ HVDCDC_Status1 shall set to Normal state. 3. The power module status shall be Enabled. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 (“level 0") NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to “Shutdown_Sequence_Completed” when Active Discharge switch status is confirmed as open.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_51 |
| Customer Req ID | SDS-HVDCDC-407 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to “Shutdown_Sequence_Completed” when Active Discharge switch status is confirmed as open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be disabled. 4. Disable active discharge on 400V side DC Link 5. Active Discharge switch status shall be confirmed as open |
| Output(s) | Below CAN messages are observed 1. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x15 ("Faulty_Shutdown_Completed ") 2. The HVDCDC shall go into Sleep State 3. All the peripherals shall achieve the state which is configured for sleep mode |
| Process | The HVDCDC application does transits to the "Shutdown_Sequence_Completed" state from the “Disable_400V_AD” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. The power module status shall be disabled. 2. Disable active discharge on 400V side DC Link 3. Active Discharge switch status shall be confirmed as open 4. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x15 ("Faulty_Shutdown_Completed ") 5. The HVDCDC shall go into Sleep State |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x1, NHVDCDCStatus = 0x6, NHVDCDCFault = 0x0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No such Valid range of Values are applicable. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 1. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x15 ("Faulty_Shutdown_Completed ") 2. The HVDCDC shall go into Sleep State |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to “Sleep State” when delay (100msec) elapsed and NM messaged is stopped receiving.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_52 |
| Customer Req ID | SDS-HVDCDC-408 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to “Sleep State” when delay (100msec) elapsed and NM messaged is stopped receiving. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall stop receiving CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Output(s) | 1. HVDCDC shall go into sleep state after specified delay timeout. 2. Within the specified delay, all the peripherals shall achieve the state configured for sleep mode. i.e shutdown sequence shall be reported. 3. The HVDCDC shall enable the CAN module for reception of only NM messages in Sleep mode. Application Message Transmission and Reception shall be stop on CAN bus. |
| Process | 1. HVDCDC shall stop receiving CAN NM message 2. Within the specified delay, the shutdown sequence shall be reported. 3. HVDCDC should transition from any state (except Sleep state) to "Sleep" state of Application state machine. 4. Disable the peripherals before going into sleep mode. 5. The TriCore shall be in shutdown state and only SCR core will be active. 6. SCR core shall put the CAN transceiver into the standby mode to save more power 7. SCR core shall set CAN transceiver configuration to read only mode |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle received CAN NM message. 2. Configure a GPIO pin “STB” as output pin. 3. The HVDCDC should be in any state except Sleep Mode of NM state machine |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall enter into sleep state which can be monitored on serial console. Verify that the current consumption is reduced on DSO or DMM. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to “check_precharge_ctt_open” upon entering the "Faulty" state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_21 |
| Customer Req ID | SDS-HVDCDC-317 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall transit to “check_precharge_ctt_open” upon entering the "Faulty" state |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall start transitioning to state “Faulty” once meets the conditions mentioned in “Process” Section |
| Output(s) | The HVDCDC transitioned the state to “Faulty”. The HVDCDC shall set NHVDCDCStatusDebug = 0x12 "Faulty_CheckPreChargeCtt". |
| Process | 1. On the power board connect connect TP411 to GND 2. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x02 ("Safe State") via CAN interface to HVDCDC application |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x1, NHVDCDCStatus = 0x6, NHVDCDCFault = 0x0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVHVDCDC shall set NHVDCDCStatusDebug = 0x12. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Pre-charge contactor control requirement.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_22 |
| Customer Req ID | SDS-HVDCDC-318 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall command to open the pre-charge contactor if the check in reports The pre-charge contactor is closed. The HVDCDC shall set NHVDCDCStatusDebug = 0x13 "Faulty_OpenPreChargeCtt". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. On the power board connect Pull-Up from TP411 to 5V 2. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x02 ("Safe State") via CAN interface to HVDCDC application. 3. Check fault response time from the time when the system transits to "Open_PreCharge_Ctt" to the time when the level 2 fault is raised. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x13 ("Faulty_OpenPreChargeCt") HVDCDC_PED_pin_sts @ HVDCDCPin_sts is set to 1 ("Enable"). NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x1A(26) after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall transits to substate "Faulty_PowerDn_SafeState" from substate “Open_PreCharge_Ctt” of the “Faulty” state when the precharge contractor is confirmed to be closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 6, NHVDCDCStatusDebug = 13. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVDCDC shall set NHVDCDCStatusDebug = 0x13 "Faulty_OpenPreChargeCtt". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### In Select_Faulty_Shutdown_Sequence state once precharge contractor got open.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_34 |
| Customer Req ID | SDS-HVDCDC-319 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall be in Select_Faulty_Shutdown_Sequence state once precharge contractor got open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x02 ("Safe State") via CAN interface to HVDCDC application. 2. Check fault response time from the time when the system transits to "Open_PreCharge_Ctt" to the time when the level 2 fault is raised. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x13 ("Faulty_OpenPreChargeCt") HVDCDC_PED_pin_sts @ HVDCDCPin_sts is set to 1 ("Enable"). NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x1A(26) after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall transits to substate “Faulty_Shutdown_Sequence" from substate “Open_PreCharge_Ctt” of the “Faulty” state when the precharge contractor is confirmed to be closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 6, NHVDCDCStatusDebug = 13. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVDCDC shall set NHVDCDCStatusDebug = 0x13 "Faulty_OpenPreChargeCtt". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### HVDCDC shall follow one of the Shutdown Sequences.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_35 |
| Customer Req ID | SDS-HVDCDC-320 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall follow one of the Shutdown Sequences as per defined in the StateFlow diagram [113] and selected in [319]. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x02 ("Safe State") via CAN interface to HVDCDC application. 2. Check fault response time from the time when the system transits to "Open_PreCharge_Ctt" to the time when the level 2 fault is raised. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x13 ("Faulty_OpenPreChargeCt") HVDCDC_PED_pin_sts @ HVDCDCPin_sts is set to 1 ("Enable"). NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x1A(26) after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall transits to substate “Faulty_Shutdown_Sequence" from substate “Open_PreCharge_Ctt” of the “Faulty” state when the precharge contractor is confirmed to be closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 6, NHVDCDCStatusDebug = 13. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |

##### Transition requirement to "Faulty_PowerDN_SafeState" state upon a HVDCDC level 2 fault and Faulty state not able to shutdown the system.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_23 |
| Customer Req ID | SDS-HVDCDC-286 |
| Milestone | B-Sample /PF1.0 |
| Purpose1 | The HVDCDC shall transit to "Faulty_PowerDN_SafeState" state upon a HVDCDC level 2 fault and Faulty state not able to shutdown the system. The HVDCDC shall set NHVDCDCStatus = 0x7 "PowerDown_Safe_State_NHVDCDCStatus_NW". The HVDCDC shall set NHVDCDCStatusDebug = 0x11 "PowerDown_Safe_State". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall transit to "Faulty_PowerDN_SafeState" state |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x6 (“Faulty State”) NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0xE ("Faulty_DisablePM") NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x21(33) after the fault response time of 10000ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_3" substate of the "Faulty" state after the defined timeout of 2ms when the below conditions are met, 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status is Enabled. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStatusDebug = 0xD. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. The HVDCDC shall set NHVDCDCStatus = 0x7. The HVDCDC shall set NHVDCDCStatusDebug = 0x11 "PowerDown_Safe_State". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Exit requirement from "Faulty_PowerDN_SafeState" state upon a sleep request.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_24 |
| Customer Req ID | SDS-HVDCDC-288 |
| Milestone | B-Sample /PF2.0 |
| Purpose1 | The HVDCDC shall exit the "Faulty_PowerDN_SafeState" state upon a sleep request. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1) Keep the power module Enabled by shorting J101 pin 5 to high on the power board. 2) provide Ignition off by making KL15 off |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall not transit to sleep state from the "Faulty_PowerDn_SafeState" state when KL15 is turned off and power board turned on. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x7, NHVDCDCStatusDebug = 0x11. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for level 0 fault response.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_25 |
| Customer Req ID | SDS-HVDCDC-176 |
| Milestone | B-Sample/ PF2.0 |
| Purpose1 | Once a level 0 fault is triggered, the HVDCDC shall latch NHVDCDCFault = 0x1 "Level 0" for 5 seconds (HVDCDC_CAL) or until the fault trigger is no longer present, whichever is longer. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Send the invalid "VehDynState_PDU" signal for less than 5 seconds and again start sending the valid "VehDynState_PDU" signal. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x4B(75) Below CAN messages are observed after 5 seconds, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Process | Send the invalid "VehDynState_PDU" signal for less than 5 seconds and again start sending the valid "VehDynState_PDU" signal. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") Below CAN messages are observed after 5 seconds, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable and Open_400V_SolidStateSw.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_39 |
| Customer Req ID | SDS-HVDCDC-403 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable and Open_400V_SolidStateSw. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x16(22) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_01_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_3" substate of the "Faulty" state after the defined timeout of 2ms when the below conditions are met, 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x16(22) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_01_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStatusDebug = 0xE. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x16(22) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_01_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Open_400V_SolidStateSw and Active_Discharge_400V.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_40 |
| Customer Req ID | SDS-HVDCDC-403 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Open_400V_SolidStateSw and Active_Discharge_400V. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. Open the solid state switch on 400V side |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x17(23) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_02_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_3" substate of the "Faulty" state after the defined timeout of 2ms when the below conditions are met, 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. Open the solid state switch on 400V side 5. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 6. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x17(23) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_02_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x17(23) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_02_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to Faulty_PowerDn-SafeState state when timeout between Disable_400V_AD and Standby.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_41 |
| Customer Req ID | SDS-HVDCDC-403 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Faulty_PowerDn-SafeState” upon Timeout between Disable_400V_AD and Standby. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. Open the solid state switch on 400V side 5. Disable active discharge on 400V side DC link |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x18(24) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_03_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable_3" substate of the "Faulty" state after the defined timeout of 2ms when the below conditions are met, 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. Open the solid state switch on 400V side 4. Disable active discharge on 400V side DC link 5. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 6. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x18(24) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_03_FAULT_ID") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x18(24) ("DIAGNOSTIC_SM_FAULTY_DISCHARGE_TIMEOUT_03_FAULT_ID") and is raised after the fault response calibratable time. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Raise Level 0 fault when Timeout between Active_Discharge_400V and Disable_400V_AD.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_42 |
| Customer Req ID | SDS-HVDCDC-404 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall raise Level 0 fault when Timeout between Active_Discharge_400V and Disable_400V_AD. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with signal value as 0x02 ("Safe State") via Canalyzer to HVDCDC application. 2. Once the HV DCDC application shall in the "Disable_3" substate of the "Faulty" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V and give more than 60V to 400V bus(400V EMI) 3. Check the fault response time by checking the HVDCDC applications transition time from "Active_Discharge_400V" to "Disable_400V_AD" substate of the "Discharge" state. |
| Output(s) | Below CAN messages are observed sequentially, NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 70 and is raised after the fault response calibratable time. |
| Process | 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status shall be Disabled. 4. Open the solid state switch on 400V side 5. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with signal value as 0x02 ("Safe State") via Canalyzer to HVDCDC application. 6. Once the HV DCDC application shall in the "Disable_3" substate of the "Faulty" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V and give more than 60V to 400V bus(400V EMI) 7. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 (“level 0") 8. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 70 and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 70 and is raised after the fault response calibratable time. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Open_400V_SolidStateSw_ 2 state when Emergency_FaultyShutdown is provided.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_53 |
| Customer Req ID | SDS-HVDCDC-410 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to substate “Open_400V_SolidStateSw_ 2” state When Emergency_FaultyShutdown is provided. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. The precharge contactor is confirmed to be open |
| Output(s) | Below CAN messages are observed NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x12 ("Faulty_Enable400VAD_NHVDCDCStatusDebug_NW ") NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_FaultyShutdown") |
| Process | The HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Open_PreCharge_Ctt” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. Open the solid state switch on the 400V side 4. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6("Faulty_NHVDCDCStatus_NW ") 5. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x12 ("Faulty_Enable400VAD_NHVDCDCStatusDebug_NW ") 6. NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_FaultyShutdown") |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6("Faulty_NHVDCDCStatus_NW ") |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x12 ("Faulty_Enable400VAD_NHVDCDCStatusDebug_NW ") NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_FaultyShutdown") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Disable_PM_and_Active_Discharge_400V_2 state when the status of the 400V Solid State Switch is open.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_54 |
| Customer Req ID | SDS-HVDCDC-411 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application shall be able to transit to Disable_PM_and_Active_Discharge_400V_2 state when the 400V Solid State Switch is open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. The status of the 400V solid state switch is open |
| Output(s) | Below CAN messages are observed NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty_NHVDCDCStatus_NW") NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_NHVDCDCFaultyShutdown_NW”) NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x14, ("Faulty_DisablePM_Enable400VAD") |
| Process | The HVDCDC application does transits to the "Disable_PM_and_Active_Discharge_400V_2" state from the “Open_400V_SolidStateSw_2” substate of the "Faulty" state after the defined the calibratable timeout when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. The power module status shall be Enabled. 4. The status of the 400V solid state switch is open 5. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty_NHVDCDCStatus_NW") 6. NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_NHVDCDCFaultyShutdown_NW”) 7. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x14, ("Faulty_DisablePM_Enable400VAD") |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall be in Open_400V_SolidStateSw_2. NHVDCDCStatusDebug @ HVDCDC_Debug_PDU shall be 0xB ("Faulty_OpenSolidStateSw") |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool.NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty_NHVDCDCStatus_NW") NHVDCDCFaultyShutdown @ HVDCDC_Status1 is set to 0x2 ("Emergency_NHVDCDCFaultyShutdown_NW”) NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x14, ("Faulty_DisablePM_Enable400VAD") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Disable_400V_AD_2 when Power Module confirmed as disabled and the Active Discharge has discharged the 400V DC Link.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_55 |
| Customer Req ID | SDS-HVDCDC-412 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to the Disable_400V_AD_2 when Power Module is disabled, and the Active Discharge has discharged the 400V DC Link. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Disable_3 state upon the Normal_FaultyShutdown is provided.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_56 |
| Customer Req ID | SDS-HVDCDC-414 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to Disable_3 state upon the Normal_FaultyShutdown is provided |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Open_400V_SolidStateSw_ 3 state When the Power Module confirms to be disabled.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_57 |
| Customer Req ID | SDS-HVDCDC-415 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to Open_400V_SolidStateSw_ 3 state When the Power Module confirms to be disabled |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the Active_Discharge_400V_3 state when the status of the 400V Solid State Switch is open..

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_58 |
| Customer Req ID | SDS-HVDCDC-416 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to Active_Discharge_400V_3 state when the status of the 400V Solid State Switch is open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to the transit to Disable_400V_AD_3 when the Active Discharge has discharged the 400V DC Link or a defined timeout has expired.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_59 |
| Customer Req ID | SDS-HVDCDC-417 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to Disable_400V_AD_3 when the Active Discharge has discharged the 400V DC Link or a defined timeout has expired. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to “Faulty” state upon the EDSU command

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_60 |
| Customer Req ID | SDS-HVDCDC-557 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall be able to transit to Faulty state upon the EDSU command |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Provide the signal EDSU NHVDCDCCommand with value 0x2 (Safe State) |
| Output(s) | 1.The signal NHVDCDCStatus shall be 0x6 (Faulty state) 2. The signal NHVDCDCFaultyShutdown shall be 0x2 (Emergency_FaultyShutdown) |
| Process | 1.The NM should be up and running 2.The application should be in normal state 3. Provide the signal EDSU NHVDCDCCommand with value 0x2 (Safe State) 4.Observe that the signal NHVDCDCStatus set with value 0x6 (Faulty state) 5.Observe that signal NHVDCDCFaultyShutdown is set with value 0x2 (Emergency_FaultyShutdown) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool. 1.The NHVDCDCStatus = 0x6 (Faulty state) 2. The NHVDCDCCommand == 0x2 (Safe State) 3.To verify that the application follows an “Emergency faulty shut down” sequence |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State: HVDCDC Fault unlatching

##### Requirement to latch the level 2 fault reactions until the unit sleeps or until the healing is allowed by the EDSU.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_60 |
| Customer Req ID | SDS-HVDCDC-158 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall latch the level 2 fault reactions until the unit sleeps or until the healing is allowed by the EDSU. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to discard the signal NFaultUnlatchRequestHVDCDC when the HVDCDC in normal operation.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_61 |
| Customer Req ID | SDS-HVDCDC-159 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall discard the signal NFaultUnlatchRequestHVDCDC when the HVDCDC in normal operation, or in the case of failure while the unit is not brought to or latched in a non-hazardous status. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to transit into "Fault Unlatching" state if all the conditions are met.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_62 |
| Customer Req ID | SDS-HVDCDC-160 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | When the value of the signal NFaultUnlatchRequestHVDCDC transition to "Clear Fault Requested (0x1)" when the HVDCDC is brought to or latched in a non-hazardous status due to a level 2 fault, the HVDCDC shall evaluate if the fault unlatching is permitted by evaluating if the following conditions are met: - 400V DC link voltage < HVDCDC_CAL V - the power module is disabled - the precharge contactor status is open - the 400V solid state switch status is open - the 400V active discharge switch status is open |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Confirm_fault_source” upon entering to the “Fault_Unlatching” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_26 |
| Customer Req ID | SDS-HVDCDC-161 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Confirm_fault_source” upon entering to the “Fault_Unlatching” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Confirm_fault_source” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Confirm_fault_source” |
| Process | The conditions for “Confirm_fault_source” to get trigger is to entry into the “Fault_Unlatching” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Observed on CANalyzer tool that the Application transitioned the state to “Confirm_fault_source” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Unlatch_power_module” from “Confirm_fault_source” substate.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_27 |
| Customer Req ID | SDS-HVDCDC-162 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Unlatch_power_module” from “Confirm_fault_source” substate. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Unlatch_power_module” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Unlatch_power_module” |
| Process | The conditions for “Unlatch_power_module” to get trigger is to confirms the status of the power module is faulty. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x06, NHVDCDCStatusDebug = 0x0D. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | CANape or CAN BUS Analyzer is a software tool Validating CAN networks - The HVHVDCDC shall command the power module to clear its internal fault. - The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0xF "FaultyUnlatch_ClearPM". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Transition requirement to substate “Unlatch_main_control”.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_28 |
| Customer Req ID | SDS-HVDCDC-163 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to substate “Unlatch_main_control”. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Unlatch_main_control” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Unlatch_main_control” |
| Process | The conditions for “Unlatch_main_control” to get trigger as below: The status of Power module is not faulty. OR The power module fault clearing is successful within a calibratable period time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | If the power module is not in faulty state NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall command to unlatch the main controller faults (HVDCDC level 2 faults). - The HVHVDCDC shall set CAN Signal as NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for response to fault unlatching pre-condition(s).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_29 |
| Customer Req ID | SDS-HVDCDC-266 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall command to unlatch the HVDCDC level 2 faults if fault unlatching pre-conditions are met. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to substate “Unlatch_main_control” once meets the conditions mentioned in “Process” Section |
| Output(s) | 1. Application transitioned the state to “Unlatch_main_control” 2. HVDCDC shall set NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys |
| Process | The conditions for “Unlatch_main_control” to get trigger as below: The status of Power module is not faulty. AND NFaultUnlatchRequestHVDCDC == 0x1 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | If the power module is not in faulty state NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 1. The HVHVDCDC shall command to unlatch the main controller faults (HVDCDC level 2 faults). 2. The HVHVDCDC shall set NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition back to “Faulty” state from “Fault_Unlatching” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_30 |
| Customer Req ID | SDS-HVDCDC-164 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit back to “Faulty” state from “Fault_Unlatching” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning back to state “Faulty” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Faulty” and transits through internal substates of Faulty states as described in the section 4.2.5 |
| Process | The conditions for “Faulty” to get trigger from “Fault_Unlatching” state as below: - The power module fault clearing is not successful within a calibratable period time. - The power module reports critical faults (“Level 2”) |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStatusDebug = 0x10, NHVDCDCFault = 0x03. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall Transit to "Faulty" state - The HVHVDCDC shall set CAN Signal as NHVDCDCStatus = 0x6 "Faulty". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition to “Standby” state from “Fault_Unlatching” state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_31 |
| Customer Req ID | SDS-HVDCDC-166 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to transit to “Standby” state from “Fault_Unlatching” state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start transitioning to state “Standby” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned the state to “Standby”. |
| Process | The conditions for “Standby” to get trigger from “Fault_Unlatching” state as below: - The power module fault clearing is not successful within a calibratable period time. - The critical faults not present in the system. (“Level 2”). |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 1, NHVDCDCStatusDebug = 6, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. - The HVHVDCDC shall Transit to "Standby" state - The HVHVDCDC shall set CAN Signal as NHVDCDCStatus = 0x0 "Standby". |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition from “Faulty” state upon sleep request.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_APP_STATE_32 |
| Customer Req ID | SDS-HVDCDC-169 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The application shall be able to exit from “Faulty” state upon sleep request. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Application shall start exiting from state “Faulty” once meets the conditions mentioned in “Process” Section |
| Output(s) | Application transitioned to “Sleep” state. |
| Process | The HVHVDCDC shall exit from the “Faulty” state once received the Sleep request from the Vehicle. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x6, NHVDCDCStausDebug = 0xD. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVHVDCDC shall Transit to "Sleep” state Verify that the current consumption is reduced and lowest rate. Verify that all the peripherals shall be reached to the configured state/disable state |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Application State Network Management Requirements

##### Requirement for transition to sleep

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_01 |
| Customer Req ID | SDS-HVDCDC-115 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transition to sleep, if the HVDCDC stops receiving the NM messages. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall stop receiving CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Output(s) | 1. HVDCDC shall go into sleep state after CAN NM message timeout. 2. All the peripherals shall achieve the state which is configured for sleep mode 3. The HVDCDC shall enable the CAN module for reception of only NM messages in Sleep mode. Application Message Transmission and Reception shall be stop on CAN bus |
| Process | 1. HVDCDC shall stop receiving CAN NM message 2. CAN NM message timeout shall get overflow 3. HVDCDC should transition from "Repeat Message" state to the "Ready Sleep State" of Network Management. 4. HVDCDC should transition from any state (except Sleep state) to "Sleep" state of Application state machine. 5. Disable the peripherals before going into sleep mode. 6. The TriCore shall be in shutdown state and only SCR core will be active. 7. SCR core shall put the CAN transceiver into the standby mode to save more power 8. SCR core shall set CAN transceiver configuration to read only mode |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle received CAN NM message. 2. Configure a GPIO pin “STB” as output pin. 3. The HVDCDC should be in any state except Sleep Mode of NM state machine |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall enter into sleep state which can be monitor on serial console. Verify that the current consumption is reduced on DSO or DMM. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Response to NM messages for transition to standby

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_02 |
| Customer Req ID | SDS-HVDCDC-115 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall wakeup and transition to standby state if the HVDCDC starts receiving the NM messages |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall receive CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Output(s) | 1. HVDCDC should go to stand-by state on wakes up. 2. All the required peripherals shall be reinitialized after wakeup state |
| Process | 1. HVDCDC shall receive CAN NM message 2. Start the timeout for NM_MSG_RECEIVE_TIME. 3. HVDCDC should transition from "Bus Sleep” Mode to the "Repeat Message" state of Network Management. 4. HVDCDC should transition from "Sleep" state to the "Standby" state of Application state machine. 5. The TriCore in standby state and SCR core shall deactivate. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle CAN message received over vehicle CAN/main CAN. 2. The HVDCDC should be in Sleep state 3. The HVDCDC shall not receive CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from "Bus Sleep” Mode to the "Repeat Message" state 2. HVDCDC should transition from "Sleep" state to the "Standby" state of Application state machine by verifying signal NHVDCDCStatus over CANalyzer tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Response requirements to NM messages request via vehicle CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_03 |
| Customer Req ID | SDS-HVDCDC-118 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall wake up on reception of NM request via vehicle CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall receive CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Output(s) | 1. HVDCDC should go to NM_REPEATE_MSG_STATE of NM state on wakes up. 2. All the required peripherals shall be reinitialized after wakeup state 3. The HVDCDC shall be in standby state of application state machine. 4. The HVDCDC shall send signal NHVDCDCStatus as 0 (Standby_NHVDCDCStatus) over vehicle CAN |
| Process | 1. HVDCDC shall receive CAN NM message 2. Start the timeout for NM_MSG_RECEIVE_TIME. 3. HVDCDC should transition from "Bus Sleep” Mode to the "Repeat Message" state of Network Management. 4. HVDCDC should transition from "Sleep" state to the "Standby" state of Application state machine. 5. The TriCore shall be in standby state and SCR core shall be in deactivate state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle CAN message received over vehicle CAN/main CAN. 2. The HVDCDC should be in Sleep state 3. The CAN Transceiver should be in standby mode (Read only mode) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall transition from "Bus Sleep” Mode to the "Repeat Message" state. Verify that the HVDCDC should set signal NHVDCDCStatus as 0 (Standby_NHVDCDCStatus) over vehicle CAN |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Response requirements to no NM messages on vehicle CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_04 |
| Customer Req ID | SDS-HVDCDC-118 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transition to sleep state when no NM messages on the vehicle CAN network. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall stops receiving CAN NM message “NMZC1Message_Hybrid_ASR_PDU” 2. The timer NM_MSG_RECEIVE shall be elapsed |
| Output(s) | 1. HVDCDC shall go into sleep state after NM_MSG_RECEIVE timeout. 2. All the peripherals shall achieve the state which is configured for sleep mode 3. The HVDCDC shall enable the CAN module for reception of only NM messages in Sleep mode. Application Message Transmission and Reception shall be stop on CAN bus |
| Process | 1. HVDCDC shall stop receiving CAN NM message 2. The timer NM_MSG_RECEIVE shall get overflow 3. HVDCDC should transition from "Repeat Message" state to the "Ready Sleep State" of Network Management. 4. HVDCDC should transition from "Ready Sleep" state to the "Prepare Bus Sleep Mode" of Network Management upon CANNM_TIMEOUT_TIME overflow. 5. HVDCDC should transition from "Prepare Bus Sleep” Mode to the "Bus Sleep Mode" of Network Management upon “CANNM_WAIT_BUS_SLEEP_TIME” overflow. (Need to get details for PDU for NM States from Sanket) 6. Disable the peripherals before going into sleep mode. 7. HVDCDC should transition from any state (except Sleep state) to "Sleep" state of Application state machine. 8. The TriCore shall be in shutdown state and only SCR core will be active. 9. SCR core shall put the CAN transceiver into the standby mode to save more power 10. SCR core shall set CAN transceiver configuration to read only mode |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle received CAN NM message. 2. Configure a GPIO pin “STB” as output pin. 3. The HVDCDC should be in any state except Sleep state 4. Set the configurable timer parameters (CANNM_TIMEOUT_TIME, CANNM_WAIT_BUS_SLEEP_TIME, NM_MSG_RECEIVE). |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall transition to the "Bus Sleep” Mode. (Check with Sanket if any PDU/Signal available for this) Verify that the current consumption is reduced on DSO or DMM. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements to save all the relevant data into NvM.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_05 (Need to get relevant data to be store from MAL) |
| Customer Req ID | SDS-HVDCDC-120 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall save all the relevant data into NvM before entering into sleep mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall stops receiving CAN NM message “NMZC1Message_Hybrid_ASR_PDU”. 2. All the below relevant data (Add relevant data) shall be store in Non-Volatile memory before entering in to Sleep mode (Check with Sanket on which section of NVM used for storing) |
| Output(s) | 1. HVDCDC should save all the required data into NvM 2, HVDCDC shall return status as NVM_WRITE_SUCCESSFUL. 3. HVDCDC should transition to Sleep state. |
| Process | 1. HVDCDC shall stop receiving CAN NM message 2. The timer NM_MSG_RECEIVE shall get overflow 3. HVDCDC should transition from "Repeat Message" state to the "Ready Sleep State" of Network Management upon NM_REPEAT_MESSAGE_TIME overflow. 4. HVDCDC should transition from "Ready Sleep" state to the "Prepare Bus Sleep Mode" of Network Management upon CANNM_TIMEOUT_TIME overflow. 5. The HVDCDC shall write all the required data one by one into NVM 6. HVDCDC shall return status as NVM_WRITE_SUCCESSFUL for each write operation 7. Disable the peripherals before going into sleep mode. 8. HVDCDC should transition from any state (except Sleep state) to "Sleep" state of Application state machine. 9. The TriCore shall be in shutdown state and only SCR core will be active. 10. SCR core shall put the CAN transceiver into the standby mode to save more power 11. SCR core shall set CAN transceiver configuration to read only mode |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure NvM drivers and NvM Application for Write operation. 2. Configure a CAN drivers and CAN routine to handle received CAN NM message. 3. Configure a GPIO pin “STB” as output pin. 4. The HVDCDC should be in any state except Sleep state 5. Set the configurable timer parameters (CANNM_TIMEOUT_TIME, CANNM_WAIT_BUS_SLEEP_TIME, NM_MSG_RECEIVE). 6. Identify and prepare list of all the relevant data to be store in memory |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the HVDCDC shall successfully complete NvM write operation for all the relevant data. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for CANNM_WAIT_BUS_SLEEP_TIME configurable timing parameter while transition between Prepare bus sleep Mode to Bus sleep Mode.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_06 |
| Customer Req ID | SDS-HVDCDC-121 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use configurable timing parameter like CANNM_WAIT_BUS_SLEEP_TIME which is used for the transition between Prepare bus sleep Mode to Bus sleep Mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The parameter CANNM_WAIT_BUS_SLEEP_TIME shall be used for the transition between Prepare bus sleep state to Bus sleep state with configurable value HVDCDC_CAL. |
| Output(s) | 1. The parameter CANNM_WAIT_BUS_SLEEP_TIME shall be configurable with value HVDCDC_CAL. |
| Process | 1. Configure the parameter CANNM_WAIT_BUS_SLEEP_TIME. 2. HVDCDC should transition from "Prepare Bus Sleep” Mode to the "Bus Sleep State" of Network Management upon “CANNM_WAIT_BUS_SLEEP_TIME” overflow. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter CANNM_WAIT_BUS_SLEEP_TIME with default value. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for transition from Prepare bus sleep mode to Bus sleep state shall be greater than equal to CANNM_WAIT_BUS_SLEEP_TIME. Verify the transition time by providing different for CANNM_WAIT_BUS_SLEEP_TIME |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for like CANNM_Repeat_Message_Time configurable timing parameter while transition from “Repeat Message” State to “Ready sleep” State.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_07 |
| Customer Req ID | SDS-HVDCDC-121 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use configurable timing parameter like CANNM_Repeat_Message_Time which is used for the transition between Repeat Message state to Ready sleep state |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The parameter CANNM_Repeat_Message_Time shall be used for the transition between Repeat Message state to Ready sleep state with configurable value HVDCDC_CAL. |
| Output(s) | 1. The parameter CANNM_Repeat_Message_Time shall be configurable with value HVDCDC_CAL. |
| Process | 1. Configure the parameter CANNM_Repeat_Message_Time. 2. HVDCDC should transition from "Repeat Message state” to the "Ready Sleep State" of Network Management upon CANNM_Repeat_Message_Time overflow. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter CANNM_Repeat_Message_Time with default value. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.Verify that the time required for transition from "Repeat Message state” to the "Ready Sleep State" shall be equal to CANNM_Repeat_Message_Time. 2. Verify the transition time by providing different for CANNM_Repeat_Message_Time |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for CANNM_MESSAGE_RECEIVE_TIME configurable timing parameter while transition between “Bus sleep Mode” to “Repeat Message State”.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_08 |
| Customer Req ID | SDS-HVDCDC-121 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use configurable timing parameter like CANNM_MESSAGE_RECEIVE_TIME which is used for the transition between “Bus sleep Mode” to “Repeat Message State” |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The parameter CANNM_MESSAGE_RECEIVE_TIME shall be used for the transition between Bus sleep state to Repeat Message State with configurable value HVDCDC_CAL. |
| Output(s) | 1. The parameter CANNM_MESSAGE_RECEIVE_TIME shall be configurable with value HVDCDC_CAL. |
| Process | 1. Configure the parameter CANNM_MESSAGE_RECEIVE_TIME. 2. HVDCDC should transition from “Bus Sleep State” to the "Repeat Message" state" of Network Management upon “NMZC1Message_Hybrid_ASR_PDU” message receive. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter CANNM_MESSAGE_RECEIVE_TIME with default value. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that the time required for transition from Prepare bus sleep state to Bus sleep state shall be equal to CANNM_MESSAGE_RECEIVE_TIME. 2. Verify the transition time by providing different for CANNM_MESSAGE_RECEIVE_Time |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for CANNM_MESSAGE_TIME configurable timing parameter while transition between Ready sleep state to Prepare Bus Sleep Mode

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_09 |
| Customer Req ID | SDS-HVDCDC-121 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use configurable timing parameter like CANNM_MESSAGE_TIME which is used for the transition between Ready sleep state to Prepare Bus Sleep Mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The parameter CANNM_MESSAGE_TIME shall be used for the transition between Ready sleep state to Prepare Bus Sleep Mode configurable with value HVDCDC_CAL. |
| Output(s) | 1. The parameter CANNM_MESSAGE_TIME shall be configurable with value HVDCDC_CAL. |
| Process | 1. Configure the parameter CANNM_MESSAGE_TIME. 2. HVDCDC should transition from “Ready sleep state” to the " Prepare Bus Sleep Mode" of Network Management upon CANNM_MESSAGE_TIME overflow. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter CANNM_MESSAGE_TIME. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for transition from Prepare bus sleep mode to Bus sleep mode shall be equal to CANNM_MESSAGE_TIME. Verify the transition time by providing different for CANNM_MESSAGE_TIME. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for CANNM_PR_MESSAGE_ RECEIVE_TIME configurable timing parameter while transition between “Prepare Bus Sleep” Mode to “Repeat Message” State

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_10 |
| Customer Req ID | SDS-HVDCDC-121 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use configurable timing parameter like CANNM_PR_MESSAGE_ RECEIVE_TIME which is used for the transition between “Prepare Bus Sleep” Mode to “Repeat Message” State |
| Derived Requirement | No Such derived requirement is applicable |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.The parameter CANNM_PR_MESSAGE_ RECEIVE_TIME shall be used for the transition between “Prepare Bus sleep” Sate to “Repeat Message” State configurable with value HVDCDC_CAL. |
| Output(s) | 1.The parameter CANNM_PR_MESSAGE_ RECEIVE_TIME shall be configurable with value HVDCDC_CAL. |
| Process | 1. Configure the parameter CANNM_PR_MESSAGE_ RECEIVE_TIME. 2. HVDCDC should transition from “Prepare Bus Sleep” Mode to “Repeat Message” State of Network Management upon “NMZC1Message_Hybrid_ASR_PDU” message receive. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter CANNM_PR_MESSAGE_ RECEIVE_TIME with default value. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for transition from “Prepare Bus Sleep” Mode to “Repeat Message” State shall be equal to CANNM_PR_MESSAGE_ RECEIVE_TIME. Verify the transition time by providing different for CANNM_PR_MESSAGE_RECEIVE_TIME. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for HVDCDC NM state machine.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_11 |
| Customer Req ID | SDS-HVDCDC-122 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall follow below mention NM state machine. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall support below NM State transition: 1. The HVDCDC shall transition from Bus Sleep Mode to Repeat Message state 2. The HVDCDC shall transition from Repeat Message state to Ready Sleep State 3. The HVDCDC shall transition from Ready Sleep State to Prepare Bus Sleep Mode 4. The HVDCDC shall transition from Prepare Bus Sleep Mode to Bus Sleep Mode 5. The HVDCDC shall transition from Prepare Bus Sleep Mode to Repeat Message state The HVDCDC shall set below configuration parameters: 1. NM_MSG_RECEIVE_TIME shall be configurable with value HVDCDC_CAL. 2. NM_REPEAT_MESSAGE_TIME shall be configurable with value HVDCDC_CAL. 3. CANNM_TIMEOUT_TIME shall be configurable with value HVDCDC_CAL. 4. CANNM_WAIT_BUS_SLEEP_TIME shall be configurable with value HVDCDC_CAL. 5. NM_MSG_PR_RECEIVE_TIME shall be configurable with value HVDCDC_CAL. |
| Output(s) | 1. The HVDCDC shall transition from Bus Sleep Mode to Repeat Message state after Wake-up message reception 2. The HVDCDC shall transition from Repeat Message state to Ready Sleep State after CANNM_REPEAT_MESSAGE_TIME elapsed 3. The HVDCDC shall transition from Ready Sleep State to Prepare Bus Sleep Mode after CANNM_TIMEOUT_TIME elapsed 4. The HVDCDC shall transition from Prepare Bus Sleep Mode to Bus Sleep Mode after CANNM_WAIT_BUS_SLEEP_TIME elapsed 5. The HVDCDC shall transition from Prepare Bus Sleep Mode to Repeat Message state after Wake-up message reception |
| Process | 1. The HVDCDC shall transition from Bus Sleep Mode to Repeat Message state when NM message received with in NM_MSG_RECEIVE_TIME 2. The HVDCDC shall transition from Prepare Bus Sleep Mode to Repeat Message state when NM message received with in NM_MSG_RECEIVE_TIME 3. The HVDCDC shall transition from Repeat Message state to Ready Sleep State upon NM_REPEAT_MESSAGE_TIME overflow 4. The HVDCDC shall transition from Ready Sleep State to Prepare Bus Sleep Mode upon CANNM_TIMEOUT_TIME overflow 5. The HVDCDC shall transition from Prepare Bus Sleep Mode to Bus Sleep Mode upon CANNM_WAIT_BUS_SLEEP_TIME overflow |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The HVDCDC shall configured as passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that the HVDCDC shall transition from Bus Sleep Mode to Repeat Message state when NM message received with in NM_MSG_RECEIVE_TIME 2. Verify that the HVDCDC shall transition from Prepare Bus Sleep Mode to Repeat Message state when NM message received with in NM_MSG_RECEIVE_TIME 3. Verify that the HVDCDC shall transition from Repeat Message state to Ready Sleep State upon NM_REPEAT_MESSAGE_TIME overflow 4. Verify that the HVDCDC shall transition from Ready Sleep State to Prepare Bus Sleep Mode upon CANNM_TIMEOUT_TIME overflow 5. Verify that the HVDCDC shall transition from Prepare Bus Sleep Mode to Bus Sleep Mode upon CANNM_WAIT_BUS_SLEEP_TIME overflow |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements to configure HVDCDC as NM passive mode.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_12 |
| Customer Req ID | SDS18-VST01-02-3444 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure as NM passive mode. |
| Derived Requirement | The HVDCDC shall be subscriber of NMZC1Message_Hybrid_ASR_PDU |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.The HVDCDC shall only receive “NMZC1Message_Hybrid_ASR_PDU” message. |
| Output(s) | 1.The HVDCDC shall only receive “NMZC1Message_Hybrid_ASR_PDU” message in low power mode. 2.The HVDCDC shall not transmit any CAN NM and Application message 3.The HVDCDC shall transit from Bus Sleep Mode to Repeat Message state |
| Process | 1. The HVDCDC shall be in SCR core on Power on reset 2. The HVDCDC shall be configured as CAN receiver Mode. 3. The HVDCDC shall be continuously monitor the NM wakeup signal “NMZC1Message_Hybrid_ASR_PDU” |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such preloaded value required |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.Verify that HVDCDC should be receive only NM CAN messages “NMZC1Message_Hybrid_ASR_PDU”. 2.Verify that HVDCDC should not transmit any NM CAN and Application messages. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition from the "Bus Sleep" Mode to the "Repeat Message" state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_13 |
| Customer Req ID | SDS18-VST01-02-3445 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transition from the "Bus Sleep" Sate to the "Repeat Message" state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall receive CAN NM message. “NMZC1Message_Hybrid_ASR_PDU” with wakeup value (TBD). |
| Output(s) | 1. HVDCDC should transition from "Bus Sleep” Mode to the "Repeat Message" state of Network Management. |
| Process | 1. Start the NM_MSG_RECEIVE_TIME. 2. HVDCDC should update the state NM State = Repeat Message state of Network Management. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode 2. The HVDCDC shall be in "Bus Sleep” Mode 3. Application CAN Tx/Rx messages shall be Disabled. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from "Bus Sleep” Mode to the "Repeat Message" state 2. Verify that HVDCDC shall Transmit and receive the CAN messages. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements to start the transmit and receive CAN messages after the Bus communication.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_14 |
| Customer Req ID | SDS18-VST01-02-3445 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall start the transmit and receive CAN messages after the Bus communication” NMZC1Message_Hybrid_ASR_PDU” Received |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall receive CAN NM message (Bus Communication = Received) “NMZC1Message_Hybrid_ASR_PDU” with wakeup value(TBD). |
| Output(s) | 1. HVDCDC should transition from "Bus Sleep” Mode to the "Repeat Message" state of Network Management. 2. HVDCDC should transition from "Sleep" state to the "Standby" state of Application. 3.HVDCDC shall receive Nm CAN message 4. Application shall start transmitting and receive application CAN messages |
| Process | 1. Start the NM_MSG_RECEIVE_TIME. 2. HVDCDC should update the state NM State = Repeat Message state of Network Management. 3. HVDCDC should update the state App State = Standby state of Application State Machine. 4.Application Should start the CAN Tx/Rx Messages. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode 2. The HVDCDC shall be in "Bus Sleep” Mode 3. Application CAN Tx/Rx messages shall be Disabled. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from "Bus Sleep” Mode to the "Repeat Message" state 2. Verify that HVDCDC shall Transmit and receive the CAN messages. 3. Verify that HVDCDC shall transition from "Sleep" state to the "Standby" state of Application. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition from the "Repeat Message" state to the "Ready Sleep" State after the "RepeatMessageTime" State after "RepeatMessageTime" has elapsed.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_15 |
| Customer Req ID | SDS-HVDCDC-121, SDS18-VST01-02-3446 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The DCDC Converter shall transition from the "Repeat Message" state to the "Ready Sleep" State after the "RepeatMessageTime" has elapsed. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The parameter NM_REPEAT_MESSAGE_TIME shall be configurable with value HVDCDC_CAL. |
| Output(s) | 1. HVDCDC shall be transit from “Repeat Message” State to “Ready sleep” State after the NM_REPEAT_MESSAGE_TIME time overflow |
| Process | 1. Start the parameter NM_REPEAT_MESSAGE_TIME. 2. Increment the NM_REPEAT_MESSAGE_TIME counter 3. Verify that NM_REPEAT_MESSAGE_TIME is overflow 4. Set the NM state = Ready Sleep State |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. HVDCDC shall receive NM message continuously. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that NM State = Ready Sleep State 2. Verify that the time required for transition from “Repeat Message” State to “Ready sleep” State shall be greater than equal to NM_REPEAT_MESSAGE_TIME. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for transition from the "Ready Sleep" state to the "Prepare Bus Sleep” Mode when "CANNM_Timeout_Time" has elapsed.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_16 |
| Customer Req ID | SDS-HVDCDC-121, SDS18-VST01-02-3447 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The DCDC Converter shall transition from the "Ready Sleep" state to the "Prepare Bus Sleep” Mode after the "CANNM_Timeout_Time" has elapsed. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall use configurable parameter CANNM_TIMEOUT_TIME for the state transition between the "Ready Sleep" state to the "Prepare Bus Sleep” Mode |
| Output(s) | 1. HVDCDC shall be transit from Ready sleep state to Prepare Bus Sleep Mode after CANNM_TIMEOUT_TIME overflows |
| Process | 1. Start the parameter CANNM_TIMEOUT_TIME. 2. Increment the CANNM_TIMEOUT_TIME counter 3. Verify that CANNM_TIMEOUT_TIME is overflow 4. Set the NM state = Prepare Bus Sleep Mode |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. NM state = Ready Sleep State |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for transition from Ready sleep state to Prepare Bus Sleep Mode shall be greater than equal to CANNM_TIMEOUT_TIME. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements to suppress all application messages.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_17 |
| Customer Req ID | SDS-HVDCDC-121, SDS18-VST01-02-3447 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall suppress all application messages after the state transition between “Ready Sleep" state to the "Prepare Bus Sleep” Mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall transit from "Ready Sleep" state to the "Prepare Bus Sleep” Mode |
| Output(s) | 1. HVDCDC shall be stop the application CAN Tx/Rx messages after the transit from Ready sleep state to Prepare Bus Sleep Mode |
| Process | 1. Start the parameter CANNM_TIMEOUT_TIME. 2. Increment the CANNM_TIMEOUT_TIME counter 3. Verify that CANNM_TIMEOUT_TIME is overflow 4. Set the NM state = Prepare Bus Sleep Mode 5. Disable the application CAN Tx/Rx messages 6. HVDCDC shall only receive the CAN NM “NMZC1Message_Hybrid_ASR_PDU” PDU |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. Configure the parameter Prepare Bus Sleep Mode. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from " Ready sleep state " to the " Prepare Bus Sleep” Mode 2. Verify that HVDCDC shall not Transmit and Receive the application CAN messages . |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements to implement SDS18-VST01-02_DCDC document for HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_25 |
| Customer Req ID | SDS-HVDCDC-336, SDS-HVDCDC-337 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall implement the requirements and design constraints stated in SDS18-VST01-02_DCDC document for HVDCDC Network Management. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | All the requirements from SDS18-VST01-02_DCDC document |
| Output(s) | Implementation of all the requirements identified from SDS18-VST01-02_DCDC document |
| Process | 1. Identify requirements and design constraint. 2. Implementation of Identified requirements and design constraint. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per the Identified requirements and design constraint. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the Identified requirements and design constraint. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

##### Requirements for " CANNM_WAIT_BUS_SLEEP_TIME" elapsed.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_18 |
| Customer Req ID | SDS-HVDCDC-121, SDS18-VST01-02-3452 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | No Such derived requirement is applicable. |
| Derived Requirement | The DCDC Converter shall transition from the " Prepare bus sleep " state to the " Bus sleep " state after the " CANNM_WAIT_BUS_SLEEP_TIME " has elapsed |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall use configurable parameter CANNM_WAIT_BUS_SLEEP_TIME for the state transition between the " Prepare bus sleep " state to the " Bus sleep " state |
| Output(s) | 1.HVDCDC shall be transit from Prepare bus sleep to Bus Sleep Mode after CANNM_WAIT_BUS_SLEEP_TIME overflows |
| Process | 1. Start the parameter CANNM_WAIT_BUS_SLEEP_TIME. 2. Increment the CANNM_WAIT_BUS_SLEEP_TIME counter 3. Verify that CANNM_WAIT_BUS_SLEEP_TIME is overflow 4. Set the NM state = Bus Sleep Mode |
| Mandatory Fields | No such Mandatory fields are applicable |
| Pre-Loaded Values | 1. Configure the parameter NM_MSG_RECEIVE. 2. Configure the parameter CANNM_WAIT_BUS_SLEEP_TIME. 3. NM state = Prepare bus sleep |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for transition from Prepare Bus Sleep Mode to Bus Sleep Mode shall be greater than equal to CANNM_WAIT_BUS_SLEEP_TIME. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for release the CAN bus in the "Bus Sleep” Mode.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_19 |
| Customer Req ID | SDS-HVDCDC-121, SDS18-VST01-02-3452 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | No Such derived requirement is applicable. |
| Derived Requirement | The DCDC Converter shall release the CAN Bus after transition from the " Prepare bus sleep " state to the " Bus sleep " state after the " CANNM_WAIT_BUS_SLEEP_TIME " has elapsed . |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall transit from " Prepare bus sleep” to the " Bus sleep " state |
| Output(s) | 1. HVDCDC shall be stop the application CAN Tx/Rx messages and release the bus after the transit from Prepare bus sleep to Prepare Bus Sleep Mode after CANNM_WAIT_BUS_SLEEP_TIME overflows |
| Process | 1. Start the parameter CANNM_WAIT_BUS_SLEEP_TIME. 2. Increment the CANNM_WAIT_BUS_SLEEP_TIME counter 3. Verify that CANNM_WAIT_BUS_SLEEP_TIME is overflow 4. Set the NM state = Bus Sleep Mode 5. Disable the application CAN Tx/Rx messages 6. HVDCDC shall only receive the CAN NM “NMZC1Message_Hybrid_ASR_PDU” PDU 7. Set the App state = Sleep State |
| Mandatory Fields | No such Mandatory fields are applicable |
| Pre-Loaded Values | 1. Configure the parameter NM_MSG_RECEIVE. 2. Configure the parameter CANNM_WAIT_BUS_SLEEP_TIME. 3. NM state = Prepare bus sleep |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that the time required for transition from Prepare Bus Sleep Mode to Bus sleep mode shall be greater than equal to CANNM_WAIT_BUS_SLEEP_TIME. 2. Verify that HVDCDC shall not Transmit and Receive the application CAN messages. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for receive NM message within NM_MSG_RECEIVE_TIME of 500msec

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_20 |
| Customer Req ID | SDS18-VST01-02-3449 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure parameter “NM_REPEAT_MESSAGE_TIME" with a value of 500ms which is used to transition from “Repeat Message” State to “Ready sleep” State |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The default value of the NM_REPEAT_MESSAGE_TIME = 500 msec 2. The parameter NM_REPEAT_MESSAGE_TIME is in milliseconds 3. The NM_REPEAT_MESSAGE_TIME parameter data type shall be unsigned type 4. The parameter NM_REPEAT_MESSAGE_TIME is calibratable |
| Output(s) | 1. HVDCDC should transition from the "Repeat Message" state to the "Ready Sleep" State after the 500msec time of " NM_REPEAT_MESSAGE_TIME " has elapsed. |
| Process | 1. Configure the parameter NM_REPEAT_MESSAGE_TIME = 500msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from "Repeat Message" state to the "Ready Sleep" State after the 500msec time of "NM_REPEAT_MESSAGE_TIME " has elapsed |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for configuring the parameter “CANNM_Timeout_Time" with a value of 5000ms

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_21 |
| Customer Req ID | SDS18-VST01-02-3455 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure parameter “CANNM_Timeout_Time" with a value of 5000ms which is used to transition from “Ready sleep” State to “Prepare Bus Sleep” Mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The default value of the CANNM_Timeout_Time= 5000 msec 2. The parameter CANNM_Timeout_Time is in milliseconds 3. The CANNM_Timeout_Timeparameter data type shall be unsigned type 4. The parameter CANNM_Timeout_Time is calibratable |
| Output(s) | 1. HVDCDC should transition from the " Ready sleep " state to the " Prepare Bus Sleep " State after the 5000msec time of " CANNM_Timeout_Time" has elapsed. |
| Process | 1. Configure the parameter CANNM_Timeout_Time = 500msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from " Ready sleep " state to the " Prepare Bus Sleep " State after the 5000msec time of "CANNM_Timeout_Time" has elapsed |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for configuring the parameter “CANNM_Wait_Bus_Sleep_Time" with a value of 5000ms.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_22 |
| Customer Req ID | SDS18-VST01-02-3456 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure parameter “CANNM_Wait_Bus_Sleep_Time" with a value of 5000ms which is used to transition from “Prepare Bus Sleep” Mode to “Bus Sleep” Mode |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The default value of the CANNM_Wait_Bus_Sleep_Time 5000 msec 2. The parameter CANNM_Wait_Bus_Sleep_Timeis in milliseconds 3. The CANNM_Wait_Bus_Sleep_Time parameter data type shall be unsigned type 4. The parameter CANNM_Wait_Bus_Sleep_Time is calibratable |
| Output(s) | 1. HVDCDC should transition from the " Prepare Bus Sleep” " state to the "Bus Sleep " State after the 5000msec time of " CANNM_Wait_Bus_Sleep_Time" has elapsed. |
| Process | 1. Configure the parameter CANNM_Wait_Bus_Sleep_Time= 5000msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from " Prepare Bus Sleep” " state to the " Bus Sleep " State after the 5000msec time of "CANNM_Wait_Bus_Sleep_Time" has elapsed |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirements for configuring the parameter “CANNM_MSG_RECEIVE_Time" with a value of 2000ms.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_23 |
| Customer Req ID | SDS18-VST01-02-3457 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure parameter “CANNM_MSG_RECEIVE_Time" with a value of 2000ms which is used to transition from “Bus Sleep” Mode to “Repeat Message” State |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The default value of the CANNM_MSG_RECEIVE_Time 2000msec 2. The parameter CANNM_MSG_RECEIVE_Time in milliseconds 3. The CANNM_MSG_RECEIVE_Time parameter data type shall be unsigned type 4. The parameter CANNM_MSG_RECEIVE_Time is calibratable |
| Output(s) | 1. HVDCDC should transition from the "Bus Sleep " state to the " Repeat Message " State if NM message received with in 2000msec time of "CANNM_MSG_RECEIVE_Time ". |
| Process | 1. Configure the parameter CANNM_MSG_RECEIVE_Time = 2000msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from " Bus Sleep” " state to the "Repeat Message" State if NM message received with in 2000msec time of " CANNM_MSG_RECEIVE_Time" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to configure parameter “CANNM_PR_MESSAGE_ RECEIVE_TIME " with a value of 2000ms

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_24 |
| Customer Req ID | SDS18-VST01-02-3457 |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall configure parameter “CANNM_PR_MESSAGE_ RECEIVE_TIME " with a value of 2000ms which is used to transition from “Prepare Bus Sleep” Mode to “Repeat Message” State |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The default value of the CANNM_PR_MESSAGE_ RECEIVE_TIME 2000msec 2. The parameter CANNM_PR_MESSAGE_ RECEIVE_TIME in milliseconds 3. The CANNM_PR_MESSAGE_ RECEIVE_TIME parameter data type shall be unsigned type 4. The parameter CANNM_PR_MESSAGE_ RECEIVE_TIME is calibratable |
| Output(s) | 1. HVDCDC should transition from the " Prepare Bus Sleep " state to the " Repeat Message " State if NM message received with in 2000msec time of "CANNM_PR_MESSAGE_ RECEIVE_TIME". |
| Process | 1. Configure the parameter CANNM_PR_MESSAGE_ RECEIVE_TIME = 2000msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC configured for passive mode |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that HVDCDC shall transition from " Prepare Bus Sleep” " state to the "Repeat Message" State if NM message received with in 2000msec time of "CANNM_PR_MESSAGE_ RECEIVE_TIME" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for AUTOSAR NM on Hybrid CAN

| Term | Description |
| --- | --- |
| REQ ID | SWRS_NM_APP_26 |
| Customer Req ID | SDS18-VST01-02-3443 (No SWRS req required for this) |
| Milestone | C-Sample/PF3.0 Phase-2 |
| Purpose1 | HVDCDC shall implement AUTOSAR NM on Hybrid CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values |  |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## Diagnostics

### Requirements

#### HVDCDC Application UDS-Diagnostics

##### Requirement against Level 2 fault if the vehicle CAN message EDSU_HVDCDC_Cmd has timed out for a calibratable time.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_01 |
| Customer Req ID | SDS-HVDCDC-183 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the vehicle CAN message EDSU_HVDCDC_Cmd has timed out for a calibratable time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms 2. Diagnostics shall be done in all state except Sleep state and Power down safe state. 3. System will set the level 2 fault for the CAN communication timeout 4. The Timeout parameter shall be calibratable. 5. Diagnostics shall be done in every 10msec. |
| Output(s) | Application shall provide CAN messages status as below, A. Initial status when no fault present - NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") - NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") B. Status when fault detected - NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") - NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 36(0x24) ("Time out NHVDCDCCommand") and is raised after the fault response time of 500ms. C. The level 2 fault should be raised in All States except Sleep state and Power down Safe state. D. DTC for this fault shall be provided as per DTC table and store it in NVM before going into sleep mode. |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if command in not received 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 36(0x24). 5. Set the DTC for this fault as per provided DTC table. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 36(0x24) DTC for this fault shall be provided as per DTC table |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against raise a Level 2 fault if the value received on the signal NHVDCDCCommand is invalid for a calibratable time.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_02 |
| Customer Req ID | SDS-HVDCDC-183 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the value received on the signal NHVDCDCCommand is invalid for a calibratable time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as "Invalid" using the CAN interface 2. Diagnostics shall be done in all state except Sleep state and Power down safe state. 3. System will set the level 2 fault for the Invalid value of NHVDCDCCommand 4. The Timeout parameter shall be calibratable. 5. Diagnostics shall be done in every 10msec. |
| Output(s) | Application shall provide CAN messages status as below, 1) NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") 2) NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 36(0x24) ("Time out NHVDCDCCommand") and is raised after the fault response time of 500ms. 3) The level 2 fault should be raised in All States |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if NHVDCDCCommand in received as INVALID 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 36(0x24). |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 36(0x24) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against E2E check failure on the vehicle CAN message EDSU_HVDCDC_Cmd

| Term | Description |
| --- | --- |
| REQ ID | SWRS_ Diag_03 |
| Customer Req ID | SDS-HVDCDC-270 |
| Milestone | Pre C-Sample/PF3.5 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the E2E check on the vehicle CAN message EDSU_HVDCDC_Cmd has failed for a calibratable time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("enable") using the CAN interface cyclically every 10ms 2. Provide invalid checksum or CRC from the received message ”EDSU_HVDCDC_Cmd” 3. E2E Check shall be done with in a calibratable time 4. Diagnostics shall be done in all state except Sleep state and Power down safe state. |
| Output(s) | Application shall provide CAN messages status as below, 1) NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") 2) NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID 36(0x24) for E2E check failed within calibratable time. 3) The level 2 fault should be raised in All States except Sleep state and Power down Safe state |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if E2E check has failed 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 36(0x24). |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 36(0x24) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against vehicle CAN in Bus Off State

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_04 |
| Customer Req ID | SDS-HVDCDC-271 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the vehicle CAN is in Bus Off State. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Remove Vehicle CAN connection on the CAN BUS. 2. Diagnostics shall be done in all state except Sleep state and Power down safe state. 3. Diagnostics shall be done within the HVDCDC_CAL time. |
| Output(s) | A. Initial status when no fault present - NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") - NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") B. Status when fault detected NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 81 and this fault shall raise after 50ms. C. The level 2 fault should be raised in All States except Sleep state and Power down Safe state |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if CAN communication does not present on vehicle CAN 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 81. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCCommand as 0x01 ("Enable"), HVDCDC= Normal, HVDCDC_Status1=Normal, HVDCDC_Debug=Normal, NHVDCDCFault= No Fault, Ensure CANalyzer Shall be connected. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Data Latency Period | Valid range value is not required. |
| Data Retention Period | Data latency period is not applicable |
| Data Rate/ <Daily> Number of transaction | Data Retention period is not applicable. |
| External Events | Data rate is not applicable. |
| Temporal Events | External event is not required. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 81 and this fault shall raise after 50ms. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against CAN message VehDynState time-out for a calibratable time

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_05 |
| Customer Req ID | SDS-HVDCDC-186 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 0 fault if the vehicle CAN messages VehDynState has timed out for a calibratable time. The HVDCDC shall latch the last valid values received on these messages/signals until the communication resumes or a power cycle. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Stop sending CAN Command VehDynState via CAN interface to HVDCDC applicationVehDynState. 2. Diagnostics shall be done in all state except Sleep state and Power down safe state. 3. System will set the level 2 fault for the CAN communication timeout 4. The Timeout parameter shall be calibratable. 5. Diagnostics shall be done every 10msec. |
| Output(s) | Application shall provide CAN messages status as below, A. Initial status when no fault present - NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") - NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") B. Status when fault detected - NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") - NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 74 after fault response time of 1000ms. C. The level 2 fault should be raised in All States except Sleep state and Power down Safe state |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if VehDynState command in not received 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 74. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCFault @ HVDCDC_Status1 = 0 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU = 74 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against Level 0 fault if the vehicle CAN messages VehDynState values received are invalid for a calibratable time.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_06 |
| Customer Req ID | SDS-HVDCDC-186 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 0 fault if the vehicle CAN messages VehDynState values received are invalid for a calibratable time. The HVDCDC shall latch the last valid values received on these messages/signals until the communication resumes or a power cycle. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send Invalid CAN Command VehDynState via CAN interface to HVDCDC applicationVehDynState. 2. Diagnostics shall be done in all state except Sleep state and Power down safe state. 3. System will set the level 2 fault for the CAN communication timeout 4. The Timeout parameter shall be calibratable. 5. Diagnostics shall be done every 10msec. |
| Output(s) | Application shall provide CAN messages status as below, A. Initial status when no fault present - NHVDCDCFault @ HVDCDC_Status1 is set to 0x0 ("No Fault") - NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x0 ("No fault") B. Status when fault detected. - NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") - NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 75 after fault response time of 50ms. |
| Process | 1. Run the Diagnostics for every 10msec. 2. Increment the CAN timeout counter if VehDynState received Invalid. 3. Check for the CAN timeout counter overflow condition is match. 4. If the condition is match raise the level 2 fault with Fault ID 75. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 = 0 NHVDCDCFaultID @ HVDCDC_Debug_PDU = 75 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 400V DC link current circuit failure & response

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_07 |
| Customer Req ID | SDS-HVDCDC-255 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link circuit current short to high failure raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. The High side voltage should be greater than HVDCDC_CAL_DAIG_6 (480V). 3. Make necessary arrangement to provide Short to High failure at 400V side current shunt (low side). |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 51 ("DIAGNOSTIC_400V_SHUNT_SC_FAULT_ID") and Shall raise after the fault response time of 100ms. |
| Process | 1. Run the Diagnostics for every 10msec. 2. Provide high side voltage at 490V (input to vicor) 3. Provide Short to High failure at 400V side current shunt. 4. If the condition is match raise the level 2 fault with Fault ID 51 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 51 ("DIAGNOSTIC_400V_SHUNT_SC_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC shall detect the 400V DC link circuit current short to ground failure raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_08 |
| Customer Req ID | SDS-HVDCDC-255 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link circuit current short to ground failure raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Short to Ground fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 52 ("DIAGNOSTIC_400V_SHUNT_STG_FAULT_ID") and Shall raise after the fault response time of 100ms. |
| Process | The HVDCDC detects the 400V DC link current circuit failure by circuit short to ground and raise a level 2 fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 52 ("DIAGNOSTIC_400V_SHUNT_STG_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 400V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_09 |
| Customer Req ID | SDS-HVDCDC-255 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link circuit current open circuit failure raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide open circuit fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 50 ("DIAGNOSTIC_400V_SHUNT_OC_FAULT_ID") and Shall raise after the fault response time of 100ms. |
| Process | The HVDCDC detects the 400V DC link current circuit failure by open circuit and raise a level 2 fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 Shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU Shall equal to 50 ("DIAGNOSTIC_400V_SHUNT_OC_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 400V DC link overcurrent detection

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_10 |
| Customer Req ID | ### SDS-HVDCDC-209 |
| Milestone | PF1.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link overcurrent and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. The High side voltage should be greater than 480V. 3. Connect the load at VICOR 400V and set the current limit at the low side higher than over current threshold. |
| Output(s) | Below CAN messages are observed, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x6 (Overcurrent fault on 400V side) and this fault is raised after over current is given to the low side after Calibratable time. |
| Process | 1. Run the Diagnostics for every 10msec. 2. Provide high side voltage at 490V (input to vicor) 3. Provide the load higher than over current threshold at VICOR 400V side current shunt. 4. If the condition is match raise the level 2 fault with Fault ID 6 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - NHVDCDCStatus = 0x00, NHVDCDCStatusDebug = 0x06. - No fault is raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x6 (Overcurrent fault on 400V side) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 800V DC link overcurrent detection

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_11 |
| Customer Req ID | ### SDS-HVDCDC-209 |
| Milestone | PF1.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link overcurrent and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Connect the load at the high side(800V) and set the current to 1A at the load side |
| Output(s) | Below CAN messages are observed, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x5 (Overcurrent fault on 800V side) and this fault is raised after 1A current is given to the high side after 1000ms. The fault should be generated in all the states |
| Process | Verify that HVDCDC shall detect the 800V DC link overcurrent and raise a level 2 fault after 1000ms |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | - NHVDCDCStatus = 0x00, NHVDCDCStatusDebug = 0x06. - No fault is raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x5 (Overcurrent fault on 800V side) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against HVDCDC is not in normal operation and it detects the power module I2C communication is compromised for a calibratable time, the HVDCDC shall raise a Level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_12 |
| Customer Req ID | SDS-HVDCDC-190 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | If the HVDCDC is not in normal operation and it detects the power module I2C communication is compromised for a calibratable time, the HVDCDC shall raise a Level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Fail the VICOR communication by sending invalid data over I2C to HVDCDC. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 62 (“DIAGNOSTIC_VICOR_COMMUNICATION_NOT_ESTABLISHED”) |
| Process | The HVDCDC is not in normal operation, and it detects the power module I2C communication is compromised for a calibratable time, the HVDCDC raises a Level 2 fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus != 0, NHVDCDCStatusDebug != 0, NHVDCDCFault != 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 62 (“DIAGNOSTIC_VICOR_COMMUNICATION_NOT_ESTABLISHED”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 800V side high voltage is applied and the power module I2C communication is not established for a calibratable time

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_13 |
| Customer Req ID | SDS-HVDCDC-191 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the 800V side high voltage is applied and the power module I2C communication is not established for a calibratable time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to "62" and this fault is raised after 100ms. |
| Process | Remove the VICOR module interface from the power module for a calibratable time and check if the HVDCDC shall raise a Level 2 fault if the 800V side high voltage is applied |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 is OFF |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | VHVDCDC800V signal valid value range is 375V - 700V. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against power module I2C communication is compromised for a calibratable time & HVDCDC not in normal state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_14 |
| Customer Req ID | SDS-HVDCDC-193 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the enabling of the power module is not successful after a calibratable time since the power module is commanded to be enabled. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Process | Process 1: 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. Ensure that NHVDCDCFault is equal to 0x0 (no fault) on the CAN trace window. 4. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. KL30 and KL15(IGN) Shall On. 2. HVDCDC= Standby, HVDCDC_Status1=Standby, HVDCDC_Debug= Standby, NHVDCDCFault= No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against fault if the enabling of the power module is not successful after a calibratable time since the power module is commanded to be

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_15 |
| Customer Req ID | SDS-HVDCDC-129 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the enabling of the power module is not successful after a calibratable time since the power module is commanded to be enabled. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. No fault condition shall be observed on the CAN trace window. 4. Disable the converter power module status 5. Keep the VICOR output voltage less than half of the VICOR input voltage minus 10V(VEnable_C) 6. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 7. Check fault response time from the time when the application enters into the "Enable" substate of the "HV_ON" state to the time when the level 2 fault is raised minus 1500ms. |
| Output(s) | At Step 3: Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 (“No Fault") At Step 6: Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Process | Verify that the HVDCDC application does not transit from the "Enable" substate to the “Precharge” substate after 1.5 seconds when the VICOR output voltage is less than half of the VICOR input voltage minus 5V(VEnable_C),if the power module status is set to disable and enters into the “Faulty” state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 off |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x1 "Enable" AND the HVDCDC commanded the enabling of the power module AND the power module is not enabled successfully for time > HVDCDC_CAL |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x11(17) ("DIAGNOSTIC_SM_HVON_TIMEOUT_02_FAULT_ID") and is raised after the fault response time of 2500ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to "Faulty_PowerDN_SafeState" state upon a HVDCDC level 2 fault and Faulty state not able to shutdown the system.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_16 |
| Customer Req ID | SDS-HVDCDC-286 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall transit to "Faulty_PowerDN_SafeState" state upon a HVDCDC level 2 fault and Faulty state not able to shutdown the system. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The power module status should be disabled 2. The HVDCDC application should be in Standby mode 3. Once the HV DCDC application is in the "disable" substate of the "Discharge" state within 10 seconds, increase VICOR output voltage(VHVDCDC400V_VICOR) to greater than half of VICOR input voltage(high side) minus 15V and ensure VICOR output voltage more than 175V. 4. Check fault response time from the time when the application enters into the "Disable" substate of the "Discharge" state to the time when the level 2 fault is raised. |
| Output(s) | At Step 4: Below CAN messages are observed, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x4 ("Disable_power_module") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x7 ( "Shutdown_DisablePM"). NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2"). NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x16(22) and is raised after the fault response time of 10000ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | Verify that the HVDCDC application does transits to the "Faulty_PowerDn_SafeState" state from the “Disable” substate of the Discharge state when the below conditions are met, 1. VICOR output voltage(VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage(VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status is Enabled. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state 2. NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal" 3. No fault shall raise, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x1 "Enable" AND the HVDCDC commanded the enabling of the power module AND the power module is not enabled successfully for time > HVDCDC_CAL |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2"). NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x16(22) and is raised after the fault response time of 10000ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to open the solid state switch on the 400V side DC link upon the power module status confirms to be disabled.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_17 |
| Customer Req ID | SDS-HVDCDC-274 |
| Milestone | Pre C-Sample/PF3.0)- Phase-2 |
| Purpose1 | When the power module confirms disabled, the HVDCDC shall send command to open the Solid State Switch on the 400V side DC link. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The power module status should be disabled 2. The HVDCDC application should be in Standby mode 3. Once the HV DCDC application is in the "Disable" substate of the "Discharge" state within 10 seconds, increase VICOR output voltage (VHVDCDC400V_VICOR) to greater than half of VICOR input voltage(high side) minus 15V. |
| Output(s) | At Step 3: Below CAN messages are observed, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x4 ("Disable_power_module") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x7 ("Shutdown_DisablePM"). NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x5 (“Discharge 400V”) NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0xA ("Shutdown_OpenSolidStateSw") |
| Process | Verify that the HVDCDC application transits to the “Open_400V_SolidStateSw” substate of the Discharge state from the “Disable” substate when the below conditions are met, 1. VICOR output voltage (VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) minus 15V(HVDCDC_CAL) 2. The VICOR output voltage (VHVDCDC400V_VICOR) is more than 175V(HVDCDC_CAL). 3. The power module status is disabled. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state 2. NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal" 3. No fault shall raise, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x1 "Enable" AND the HVDCDC commanded the enabling of the power module AND the power module is not enabled successfully for time > HVDCDC_CAL |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x5 (“Discharge 400V”) NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0xA ("Shutdown_OpenSolidStateSw") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Over voltage..

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_18 |
| Customer Req ID | SDS-HVDCDC-194 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Over voltage. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Set High side status(Status_HI) of VICOR to 0x80 via PMBUS |
| Output(s) | Below CAN messages shall be observed NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 83(DIAGNOSTIC_PM_I2C_OV_FAULT_ID) |
| Process | The HVDCDC shall raises a Level 2 fault if the VICOR reports an over-voltage fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus @ HVDCDC_Status1= Normal(1), NHVDCDCStatusDebug @ HVDCDC_Debug=Normal(6), NHVDCDCFault @ HVDCDC_Status1=(0) No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 83(DIAGNOSTIC_PM_I2C_OV_FAULT_ID) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Under voltage.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_19 |
| Customer Req ID | SDS-HVDCDC-194 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Under voltage. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Set High side status(Status_HI) of VICOR to 0x10 via PMBUS |
| Output(s) | Below CAN messages shall be observed NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 84(DIAGNOSTIC_PM_I2C_UV_FAULT_ID) |
| Process | The HVDCDC shall raises a Level 2 fault if the VICOR reports an over-voltage fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus @ HVDCDC_Status1= Normal(1), NHVDCDCStatusDebug @ HVDCDC_Debug=Normal(6), NHVDCDCFault @ HVDCDC_Status1=(0) No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 84(DIAGNOSTIC_PM_I2C_UV_FAULT_ID) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Over temperature.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_20 |
| Customer Req ID | SDS-HVDCDC-194 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a Level 2 fault if the power module reports any self disabling fault like Over temperature. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Set High side status(Status_Temp) of VICOR to 0x80 via PMBUS |
| Output(s) | Below CAN messages shall be observed NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 85(DIAGNOSTIC_PM_I2C_OT_FAULT_ID) |
| Process | The HVDCDC shall raises a Level 2 fault if the VICOR reports an over-voltage fault |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus @ HVDCDC_Status1= Normal(1), NHVDCDCStatusDebug @ HVDCDC_Debug=Normal(6), NHVDCDCFault @ HVDCDC_Status1=(0) No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCNBMSts @ HVDCDC_NBM_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 85(DIAGNOSTIC_PM_I2C_OT_FAULT_ID) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against report a faulty state of the Power Module by raising a Level 2 fault if the Fault Pin (FLT) is pulled low.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_115 |
| Customer Req ID | SDS-HVDCDC-331 |
| Milestone | Pre-C Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report a faulty state of the Power Module by raising a Level 2 fault if the Fault Pin (FLT) is pulled low. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The input voltage to Vicor module should be greater than 480V (To start vicor conversion) 2. HVDCDC should be in HVON state 2. Set High side status of VICOR by making respective GPIO enable. 2. Fault Pin should be pulled low |
| Output(s) | Below CAN messages shall be observed NHVDCDCNBMFLT @ HVDCDC_NBM2_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for Fault Pin (FLT) is pulled low |
| Process | The HVDCDC shall raises a Level 2 fault if the VICOR reports Fault Pin (FLT) is pulled low. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus @ HVDCDC_Status1= Normal(1), NHVDCDCStatusDebug @ HVDCDC_Debug=Normal(6), NHVDCDCFault @ HVDCDC_Status1=(0) No Fault |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCNBMFLT @ HVDCDC_NBM2_PDU via PMBUS shall equal to 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") 1. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for Fault Pin (FLT) is pulled low |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against raise a Level 2 fault upon the detection of the 800Vind sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_21 |
| Customer Req ID | SDS-HVDCDC-291 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vind sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vind corresponds to the sensor with CAN signal name: "VHVDCDC800VAnalog") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required on power board for OC |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 71 (“DIAGNOSTIC_800V_VICOR_IND_OC_FAULT_ID”) and this fault is raised after 500ms |
| Process | Verify that the HVDCDC raises a level 2 fault upon the detection of the 800Vind sensing circuit failure circuit by open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. HVDCDCStatus @ HVDCDC_Status1 is equal to 0x0 ("Standby") 2. NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0x0 ("Standby"). 3. No fault is raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 71 (“DIAGNOSTIC_800V_VICOR_IND_OC_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vind sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_22 |
| Customer Req ID | SDS-HVDCDC-291 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vind sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vind corresponds to the sensor with CAN signal name: "VHVDCDC800VAnalog") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required on power board for Short to High |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 72 (“DIAGNOSTIC_800V_VICOR_IND_SC_FAULT_ID”) and this fault is raised after 500ms |
| Process | Verify that the HVDCDC raises a level 2 fault upon the detection of the 800Vind sensing circuit failure circuit by circuit short to high. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. HVDCDCStatus @ HVDCDC_Status1 is equal to 0x0 ("Standby") 2. NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0x0 ("Standby"). 3. No fault is raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 72 (“DIAGNOSTIC_800V_VICOR_IND_SC_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 800V side high voltage is applied and the power module I2C communication is not established for a calibratable time.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_23 |
| Customer Req ID | SDS-HVDCDC-291 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vind sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vind corresponds to the sensor with CAN signal name: "VHVDCDC800VAnalog") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required on power board for Short to Ground |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 73 (“DIAGNOSTIC_800V_VICOR_IND_STG_FAULT_ID”) and this fault is raised after 500ms |
| Process | Verify that the HVDCDC raises a level 2 fault upon the detection of the 800Vind sensing circuit failure circuit by circuit short to Ground. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. HVDCDCStatus @ HVDCDC_Status1 is equal to 0x0 ("Standby") 2. NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0x0 ("Standby"). 3. No fault is raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 73 (“DIAGNOSTIC_800V_VICOR_IND_STG_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_24 |
| Customer Req ID | SDS-HVDCDC-292 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vemi corresponds to the sensor with CAN signal name: "VHVDCDC800V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. On power board short R355 with high. 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 39 ("DIAGNOSTIC_400V_EMI_OC_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(EMI) sensing circuit failure by circuit short to high, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 39 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open)

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_25 |
| Customer Req ID | SDS-HVDCDC-292 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vemi corresponds to the sensor with CAN signal name: "VHVDCDC800V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. On the power board short R358 to the ground. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 40 ("DIAGNOSTIC_400V_EMI_STG__FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(EMI) sensing circuit failure by circuit short to ground, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStaus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 40 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_26 |
| Customer Req ID | SDS-HVDCDC-292 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vemi corresponds to the sensor with CAN signal name: "VHVDCDC800V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Remove R355 from the power board. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 38 ("DIAGNOSTIC_400V_EMI_OC_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(EMI) sensing circuit failure by an open circuit, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 38 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_27 |
| Customer Req ID | SDS-HVDCDC-293 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vvic corresponds to the sensor with CAN signal name: "VHVDCDC800V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. On the power board short R365 with high. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 42 ("DIAGNOSTIC_400V_VICOR_SC_5V_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(VICOR voltage) sensing circuit failure by circuit short to high, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 42 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_28 |
| Customer Req ID | SDS-HVDCDC-293 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vvic corresponds to the sensor with CAN signal name: "VHVDCDC800V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. On the power board short R365 with GND. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 43("DIAGNOSTIC_400V_VICOR_STG_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(VICOR voltage) sensing circuit failure by circuit short to ground, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCSTatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 43 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_29 |
| Customer Req ID | SDS-HVDCDC-293 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 800Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (800Vvic corresponds to the sensor with CAN signal name: "VHVDCDC800V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Remove R365 from the power board. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 41 ("DIAGNOSTIC_400V_VICOR_OC_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The power module output of the 800V side voltage is not received or the communication with the power module is compromised because of the 800V DC link voltage(VICOR voltage) sensing circuit failure by an open circuit, the HVDCDC application detects the failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 41 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of plausibility failure of the 800Vemi voltage sensor..

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_30 |
| Customer Req ID | SDS-HVDCDC-294 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of plausibility failure of the 800Vemi voltage sensor. Sensors delays need to be filtered and debounced. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Adjust the voltage of input and output as per specs |
| Output(s) | NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 76 and this fault is raised after 500ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of plausibility failure of the 800Vemi voltage sensor. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of plausibility failure of the 800Vvic voltage sensor.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_31 |
| Customer Req ID | SDS-HVDCDC-310 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of plausibility failure of the 800Vvic voltage sensor. Sensors delays need to be filtered and debounced. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Adjust the voltage of input and output as per specs |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 77 and this fault is raised after 500ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of plausibility failure of the 800Vvic voltage sensor. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_32 |
| Customer Req ID | SDS-HVDCDC-295 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vemi corresponds to the sensor with CAN signal name: "VHVDCDC400V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Open Circuit. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 45 ("DIAGNOSTIC_400V_EMI_OC_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(EMI) sensing circuit failure by circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 45 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_33 |
| Customer Req ID | SDS-HVDCDC-295 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vemi corresponds to the sensor with CAN signal name: "VHVDCDC400V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Short to High fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 46 ("DIAGNOSTIC_400V_EMI_SC_5V_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(EMI) sensing circuit failure by circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 46 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_34 |
| Customer Req ID | SDS-HVDCDC-295 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vemi sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vemi corresponds to the sensor with CAN signal name: "VHVDCDC400V_emi") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Short to GND fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 44 ("DIAGNOSTIC_400V_EMI_STG_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(EMI) sensing circuit failure by circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 44 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_35 |
| Customer Req ID | SDS-HVDCDC-296 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vvic corresponds to the sensor with CAN signal name: "VHVDCDC400V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. On the power board short R375 with high. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 48 ("DIAGNOSTIC_400V_VICOR_SC_5V_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(VICOR output voltage) sensing circuit failure by circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 48 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_36 |
| Customer Req ID | SDS-HVDCDC-296 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vvic corresponds to the sensor with CAN signal name: "VHVDCDC400V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. On power board short R378 to ground |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 49("DIAGNOSTIC_400V_VICOR_STG_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(VICOR output voltage) sensing circuit failure by circuit short to ground and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 49 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_37 |
| Customer Req ID | SDS-HVDCDC-296 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of the 400Vvic sensing circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open). (400Vvic corresponds to the sensor with CAN signal name: "VHVDCDC400V") |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority Medium | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Remove R375 from the power board. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 47 ("DIAGNOSTIC_400V_VICOR_OC_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the 400V DC link voltage(VICOR output voltage) sensing circuit failure by an open circuit and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 47 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the detection of plausibility failure of the 400V DC link.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_38 |
| Customer Req ID | SDS-HVDCDC-297 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault upon the detection of plausibility failure of the 400V DC link Sensors delays need to be filtered and debounced. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Adjust the voltage of input and output as per specs |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 78 and this fault is raised after 500ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of plausibility failure of the 400V DC link. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 78 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against. the 800V DC link voltage higher than a threshold.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_39 |
| Customer Req ID | SDS-HVDCDC-199 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link voltage higher than a threshold and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 800V side (high side) voltage of the power module should greater than HVDCDC_CAL_DAIG_2 (700V). 2. The threshold for 800V side (high side) voltage should be calibratable. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x1 ("DIAGNOSTIC_800V_OV_FAULT_ID") and this fault shall be raised after 500ms after high side voltage crossed 700V. |
| Process | Verify that HVDCDC shall detects the 800V DC link overvoltage and raises a level 2 fault after 500ms. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x0 ("Standby") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x1 ("DIAGNOSTIC_800V_OV_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the 400V DC link overvoltage.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_40 |
| Customer Req ID | SDS-HVDCDC-200 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link overvoltage and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 400V side (low side) voltage of the power module should greater than 350V. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x1 ("DIAGNOSTIC_400V_OV_FAULT_ID") and this fault shall be raised after 500ms after high side voltage crossed 350V. |
| Process | Verify that HVDCDC shall detects the 400V DC link overvoltage and raises a level 2 fault after 500ms. |
| Mandatory Fields | Valid range of Values is not applicable |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x0 ("Standby") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x1 ("DIAGNOSTIC_400V_OV_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERI |

##### Requirement against the 800V DC link under voltage when in normal operation.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_41 |
| Customer Req ID | SDS-HVDCDC-201 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link under voltage when in normal operation and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 800V side (high side) voltage of the power module should less than 375V. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x2 ("DIAGNOSTIC_800V_UV_FAULT_ID") and this fault shall be raised after 500ms after high side voltage is decreased to less than 375V. |
| Process | Verify that HVDCDC shall detects the 800V DC link undervoltage and raises a level 2 fault after 500ms. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x6 ("Normal "). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x2 ("DIAGNOSTIC_800V_UV_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 400V DC link under voltage when in normal operation and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_42 |
| Customer Req ID | SDS-HVDCDC-202 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link under voltage when in normal operation and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 400V side (low side) voltage of the power module should less than 180V. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x4 ("DIAGNOSTIC_400V_UV_FAULT_ID") and this fault shall be raised after 500ms after high side voltage is decreased to less than 180V. |
| Process | Verify that HVDCDC shall detects the 400V DC link undervoltage and raises a level 2 fault after 500ms. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x6 ("Normal "). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x4 ("DIAGNOSTIC_400V_UV_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting 400V DC link under voltage when in normal operation and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_43 |
| Customer Req ID | SDS-HVDCDC-268 (Obsolute in 2.5) |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 400V DC link under voltage when in normal operation and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. 400V side (low side) voltage of the power module should less than 180V. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x4 ("DIAGNOSTIC_400V_UV_FAULT_ID") and this fault shall be raised after 500ms after high side voltage is decreased to less than 180V. |
| Process | Verify that HVDCDC shall detects the 400V DC link undervoltage and raises a level 2 fault after 500ms. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x6 ("Normal "). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x4 ("DIAGNOSTIC_400V_UV_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the power module 400V side voltage output is less than half of the 800V side input minus a calibratable threshold in HV On state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_44 |
| Customer Req ID | SDS-HVDCDC-273 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the power module 400V side voltage output is less than half of the 800V side input minus a calibratable threshold in HV On state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 470V) 3. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 100ms right after the 800V voltage stabilize at 480V 4. Make 400V side voltage output shall less than half of the high side voltage minus 15V. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 63 and this fault is observed after 100ms |
| Process | The application does not transit from substate “Open_precharge_Ctt” to “Normal" substrate when the status of the precharge contractor is confirmed to be closed and transit to "Faulty_PowerDn_SafeState". |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 OFF |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 63 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against the power module 400V side voltage output is less than half of the 800V side input minus a calibratable threshold in Precharge state and the Power Module is enabled.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_116 |
| Customer Req ID | SDS-HVDCDC-418 |
| Milestone | Pre C-Sample/PF3.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the power module 400V side voltage output is less than half of the 800V side input minus a calibratable threshold in Precharge state and the Power Module is enabled. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module such a way that the 400V side (Low Side) voltage should be less than half of the 800V side input minus a calibratable threshold. (For ex. 800V side voltage is 500V then 400V side should be ((500V*0.5V)-15V) i.e. < 235V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface 100ms right after the 800V voltage stabilize. 3. Make 400V side voltage output shall less than half of the high side voltage minus 15V. 4. NHVDCDCStatus shall be set as 0x03 ("Precharge 400V") using the CAN interface |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU |
| Process | The HVDCDC application shall set the state as NHVDCDCFaultyShutdown = 0x2 “Emergency_FaultyShutdown when the below conditions are met, 1. Set the 400V side (low side) voltage of the power module to less than half of the high side voltage minus 15V 2. The power module status shall be Enabled. 3. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 4. NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to <TBD> ("2:1 Ratio failure while precharge") and is raised after the fault response calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to <TBD> ("2:1 Ratio failure while precharge") and is raised after the fault response calibratable time. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 800V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_45 |
| Customer Req ID | SDS-HVDCDC-309 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Short to High. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 54 ("DIAGNOSTIC_800V_SHUNT_SC_FAULT_ID") and shall be raised after the fault response time of 100ms. |
| Process | Verify that when the power module output of the 800V side current shall not be received or the communication with the power module shall be compromised, the HVDCDC application raises a level 2 fault upon the detection of the 800V DC link current sensor circuit failure by circuit short to high |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No Specific valid range of value applicable. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 54 ("DIAGNOSTIC_800V_SHUNT_SC_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against 800V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_46 |
| Customer Req ID | SDS-HVDCDC-309 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide Short to Ground fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 55 ("DIAGNOSTIC_800V_SHUNT_STG_FAULT_ID") and shall be raised after the fault response time of 100ms. |
| Process | Verify that when the power module output of the 800V side current shall not be received or the communication with the power module shall be compromised, the HVDCDC application raises a level 2 fault upon the detection of the 800V DC link current sensor circuit failure by circuit short to ground |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No Specific valid range of value applicable. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 55 ("DIAGNOSTIC_800V_SHUNT_STG_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of Vicor_ind Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_47 |
| Customer Req ID | SDS-HVDCDC-309 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the 800V DC link current circuit failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Make necessary arrangement to provide open circuit fault. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 53 ("DIAGNOSTIC_800V_SHUNT_OC_FAULT_ID") and shall be raised after the fault response time of 100ms. |
| Process | Verify that when the power module output of the 800V side current shall not be received or the communication with the power module shall be compromised, the HVDCDC application raises a level 2 fault upon the detection of the 800V DC link current sensor circuit failure by circuit open. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No Specific valid range of value applicable. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 53 ("DIAGNOSTIC_800V_SHUNT_OC_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of Vicor_ind Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_48 |
| Customer Req ID | SDS-HVDCDC-304 |
| Milestone | Pre C-Sample/PF3.0 Phase -2 |
| Purpose1 | The HVDCDC shall detect the over temperature of Vicor_ind Temp measuring point and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Local VICOR sensor shall detect temperature above HVDCDC_CAL specified temp range in degree. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 90 (VICOR over temperature fault ID) and this fault is raised after specified HVDCDC_CAL debounce time when the local VICOR sensor detects the temperature more than HVDCDC_CAL specified temp range in degree TVHDCDC1 @ HVDCDC_Status3 is greater than HVDCDC_CAL specified temp range in degree. |
| Process | The HVDCDC shall raise the over temperature of VICOR_ind Temp measuring point and raises a level 0 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x90 (VICOR over temperature) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of 400V EMI Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_49 |
| Customer Req ID | SDS-HVDCDC-305 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the over temperature of 400V EMI Temp measuring point and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 400V EMI low side shall detect temperature above HVDCDC_CAL specified temp range in degrees |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x9 (Over temperature fault on the low side) and this fault is raised after specified HVDCDC_CAL debounce time in msec when 400V detected temperature more than above HVDCDC_CAL specified temp range in degrees. TVHDCDC2 @ HVDCDC_Status3 is greater than above HVDCDC_CAL specified temp range in degrees. |
| Process | HVDCDC detects the 400V DC link overtemperature and raises a level 0 fault after specified HVDCDC_CAL debounce time in msec |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x9 (Over temperature fault on the low side) TVHDCDC2 @ HVDCDC_Status3 is greater than above HVDCDC_CAL specified temp range in degrees. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of 800V EMI Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_50 |
| Customer Req ID | SDS-HVDCDC-306 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the over temperature of 800V EMI Temp measuring point and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 800V EMI high side shall detect temperature above above HVDCDC_CAL specified temp range in degrees |
| Output(s) | Below CAN messages are observed NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x10 (Over temperature fault on the high side) and this fault is raised after specified HVDCDC_CAL debounce time in msec when 400V detected temperature more than above HVDCDC_CAL specified temp range in degrees. TVHDCDC3 @ HVDCDC_Status3 is greater than above HVDCDC_CAL specified temp range in degrees. |
| Process | HVDCDC detects the 800V DC link over temperature and raise a level 0 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Valid range of Values is not applicable |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x10 (Over temperature fault on the high side) TVHDCDC3 @ HVDCDC_Status3 is greater than above HVDCDC_CAL specified temp range in degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of Vicor Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_51 |
| Customer Req ID | SDS-HVDCDC-307 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall detect the over temperature of Vicor Temp measuring point and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | VICOR low side shall detect temperature more than above HVDCDC_CAL specified temp range (lower limit) in degree. |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 ("Level 0") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x7 ("Over temperature fault on VICOR low side") and this fault shall be raised after specified HVDCDC_CAL debounce time when the VICOR local sensor detects temperature more than HVDCDC_CAL specified temp range in degree 3. TVHDCDC4 @ HVDCDC_Status3 shall greater than HVDCDC_CAL specified temp range in degree. |
| Process | Verify that the HVDCDC application raises a level 0 fault of VICOR over-temperature after specified HVDCDC_CAL debounce time when the modeled temperature(s) of the power module's internal critical temperature |
| Mandatory Fields | No such mandatory fields is applicable |
| Pre-Loaded Values | 1. The HVDCDC application shall be in HV_ON state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal "). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 ("Level 0") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x7 ("Over temperature fault on VICOR low side") 3. TVHDCDC4 @ HVDCDC_Status3 shall greater than above HVDCDC_CAL specified temp range in degrees. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_510 |
| Customer Req ID | SDS-HVDCDC-211 (This requirement is obsolete as per JIRA HA-169) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall raise a level 0 fault if the modelled temperature(s) of the power module internal critical temperature point is above a threshold for a calibratable time. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | <TBD> shall detect temperature more than above HVDCDC_CAL specified temp range in degree |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 ("Level 0") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x7 ("Over temperature fault on VICOR low side") and this fault shall be raised after specified HVDCDC_CAL debounce time in msec when the VICOR local sensor detects temperature more than above HVDCDC_CAL specified temp range in degrees. 3. TVHDCDC4 @ HVDCDC_Status3 shall greater than above HVDCDC_CAL specified temp range in degrees. |
| Process | Verify that the HVDCDC application raises a level 0 fault of VICOR over-temperature after specified HVDCDC_CAL debounce time in msec when the modeled temperature(s) of the power module's internal critical temperature |
| Mandatory Fields | No such mandatory fields is applicable |
| Pre-Loaded Values | 1. The HVDCDC application shall be in HV_ON state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal "). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 ("Level 0") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x7 ("Over temperature fault on VICOR low side") 3. TVHDCDC4 @ HVDCDC_Status3 shall greater than above HVDCDC_CAL specified temp range in degrees. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of Solid State Switch Temp measuring point

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_52 |
| Customer Req ID | SDS-HVDCDC-308 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the over temperature of Solid State Switch Temp measuring point and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The Solid State Switch local sensor shall detect temperature above 85 degrees |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x8 (SSS over-voltage) and this fault is raised after specified HVDCDC_CAL debounce time in msec when the Solid State Switch local sensor detects a temperature more than 85 degrees. TVHDCDC5 @ HVDCDC_Status3 is greater than 85 degrees. |
| Process | 1. The HVDCDC application raises a level 0 fault of Solid state switch over-temperature after specified HVDCDC_CAL debounce time in msec when the modeled temperature(s) of the power module's internal critical temperature shall above a threshold of 85 degrees for a calibratable time. 2. Continuously monitor the temperature rise of the CAN interface |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x8. TVHDCDC5 @ HVDCDC_Status3 is greater than 85 degrees. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the over temperature of the microcontroller

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_53 |
| Customer Req ID | SDS-HVDCDC-215 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the over temperature of the microcontroller and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Give a temperature more 85 degrees to the controller IC TC397. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 64 (“DIAGNOSTIC_MCU_OT_FAULT_ID”) and is raised after the fault response time of 4000ms. |
| Process | The HVDCDC shall detects the over temperature fault of the microcontroller and raise a level 0 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 64 (“DIAGNOSTIC_MCU_OT_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement raise level 2 fault when the over temperature of the microcontroller may damage the controller

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_119 |
| Customer Req ID | SDS-HVDCDC-434 |
| Milestone | Pre-C-Sample/PF3.0 Phase2 |
| Purpose1 | The HVDCDC shall detect the over temperature of the microcontroller and raise a level 2 fault in case the measured temperature could cause a permanent damage to the microcontroller |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Give a temperature more 100 degrees (HVDCDC_CAL) to the controller IC TC397 for calibratable time HVDCDC_CAL. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID for detected over temperature for configured calibratable time. |
| Process | The HVDCDC shall detects the over temperature fault of the microcontroller and raise a level 2 fault if the temperature exceed the calibratable temp value and remained for calibratable time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2 fault") NHVDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID for detected over temperature for configured calibratable time. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of Vicor_ind Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_101 |
| Customer Req ID | SDS-HVDCDC-341 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of Vicor_ind Temp measuring point shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Local VICOR sensor shall detect temperature above HVDCDC_CAL specified temp range in degrees for calibratable time HVDCDC_CAL |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local VICOR sensor detected over temperature for configured calibratable time. TVHDCDC1 @ HVDCDC_Status3 is greater than <TBD> degrees. |
| Process | The HVDCDC shall detect the over temperature of VICOR_ind Temp measuring point and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local VICOR sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of 400V EMI Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_102 |
| Customer Req ID | SDS-HVDCDC-342 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of 400V EMI Temp measuring point shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 400V EMI sensor shall detect temperature above above HVDCDC_CAL specified temp range in degrees for calibratable time HVDCDC_CAL |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the 400V EMI (Low Side) sensor detects the temperature more than <TBD> degrees and this fault is raised after specified HVDCDC_CAL debounce time in msec TVHDCDC2 @ HVDCDC_Status3 is greater than <TBD> degrees. |
| Process | The HVDCDC shall detect the over temperature of 400V EMI (Low Side)Temp measuring point and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local 400V EMI (Low Side) sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of 800V EMI (High Side) Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_103 |
| Customer Req ID | SDS-HVDCDC-343 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of 800V EMI (High Side) Temp measuring point shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 800V EMI (High Side) sensor shall detect temperature above above HVDCDC_CAL specified temp range in degrees |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local 800V EMI (High Side) sensor detects the temperature more than <TBD> degrees and this fault is raised after specified HVDCDC_CAL debounce time in msec TVHDCDC3 @ HVDCDC_Status3 is greater than <TBD> degrees. |
| Process | The HVDCDC shall detect the over temperature of VICOR_ind Temp measuring point and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local 800V EMI (High Side) sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of VICOR low side Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_104 |
| Customer Req ID | SDS-HVDCDC-344 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of VICOR low side Temp measuring point shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Local VICOR sensor shall detect temperature above above HVDCDC_CAL specified temp range in degrees |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local VICOR low side sensor detects the temperature more than <TBD> degrees and this fault is raised after specified HVDCDC_CAL debounce time in msec TVHDCDC4 @ HVDCDC_Status3 is greater than <TBD> degrees. |
| Process | The HVDCDC shall detect the over temperature of VICOR low side Temp measuring point and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the local VICOR low side sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of Solid State Switch local sensor Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_105 |
| Customer Req ID | SDS-HVDCDC-345 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of Solid State Switch local sensor Temp measuring point shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Local VICOR sensor shall detect temperature above above HVDCDC_CAL specified temp range in degrees |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the Solid State Switch local sensor detects the temperature more than <TBD> degrees and this fault is raised after specified HVDCDC_CAL debounce time in msec TVHDCDC5 @ HVDCDC_Status3 is greater than <TBD> degrees. |
| Process | The HVDCDC shall detect the over temperature of Solid State Switch local sensor Temp measuring point and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the Solid State Switch local sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting the Over temperature of microcontroller Temp measuring point by HVDCDC

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_106 |
| Customer Req ID | SDS-HVDCDC-346 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The Over temperature of microcontroller shall be detected by the HVDCDC and raise a level 2 fault to avoid a cause of permanent damage to any of the surrounding HW components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | MCU Temp sensor shall detect temperature above above HVDCDC_CAL specified temp range in degrees (High limit Temp- Hard) |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the microcontroller detects the temperature more than <TBD> degrees and this fault is raised after specified HVDCDC_CAL debounce time in msec |
| Process | The HVDCDC shall detect the over temperature of microcontroller and raises a level 2 fault after specified HVDCDC_CAL debounce time in msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID when the Solid State Switch local sensor detects the temperature more than <TBD> degrees |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detecting report the main controller internal electronic failures.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_54 |
| Customer Req ID | SDS-HVDCDC-219 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall detect and report the main controller internal electronic failures. The fault severity should be categorised according to the failure effects, guided by [175]. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | MCU will run the Power on self test diagnostics Check for the any fault if any(RAM test , Peripherals test) |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 as per fault status level NHVDCDCFaultID @ HVDCDC_Debug_PDU with respective Fault ID |
| Process | Verify that the HVDCDC shall detect and report the main controller internal electronic failures. The fault severity should be categorized according to the failure effects |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 as per fault status level NHVDCDCFaultID @ HVDCDC_Debug_PDU with respective Fault ID |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to raise level 2 fault if HVDCDC detects all 16 channels of ADC failed to read input.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_117 |
| Customer Req ID | SDS-HVDCDC-419 |
| Milestone | Pre C-Sample/PF3.1 (This will be cover in safety pack integration) |
| Purpose1 | The HVDCDC shall raise level 2 fault if ADC shall failed to get input or read input on all 16 channels. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. All ADC channel not getting proper input 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 ("Level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU with respective Fault ID |
| Process | The HVDCDC application shall set the state as NHVDCDCFaultyShutdown = 0x2 “Emergency_FaultyShutdown when the below conditions are met, 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status shall be Enabled. 3. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") 4. NHVDCDCFaultID @ HVDCDC_Debug_PDU with respective Fault ID |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU with respective Fault ID |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against HVIL status in ideal state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_55 |
| Customer Req ID | SDS-HVDCDC-221 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report the HVIL status as 2 (Unused_NHVDCDCHVIL_NW) when the HVDCDC application is in ideal state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The signal NHVDCDCHVIL @ HVDCDC_Status1 shall be provided on the CAN interface. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2 (Unused_NHVDCDCHVIL_NW). |
| Process | The HVDCDC shall reports the HVIL status as 2 (Unused_NHVDCDCHVIL_NW) when the HVDCDC application is in ideal state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2(Unused_NHVDCDCHVIL_NW). |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against HVIL status in intact state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_56 |
| Customer Req ID | SDS-HVDCDC-221 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report the HVIL status as 0 (Intact_NHVDCDCHVIL_NW) when the HVDCDC application is in intact state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The LV connector shall be connected to the 6V power supply with a 280ohm pot connected in between. 2. The LV connector shall be connected to the GND. 3. Check the signal NHVDCDCHVIL @ HVDCDC_Status1 on the CAN interface. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface NHVDCDCHVIL @ HVDCDC_Status1 is equal to 0 (Intact_NHVDCDCHVIL_NW). |
| Process | The HVDCDC shall reports the HVIL status as 0 (Intact_NHVDCDCHVIL_NW) when the HVDCDC HVIL is in intact state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCHVIL @ HVDCDC_Status1 is equal to 0(Intact _NHVDCDCHVIL_NW).. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against HVIL status when application is in open state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_57 |
| Customer Req ID | SDS-HVDCDC-221 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report the HVIL status as 1 (Open_NHVDCDCHVIL_NW) when the HVDCDC application is in open state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the HVIL circuit open loop by removing 6V power from the LV connector. 2. Check the signal NHVDCDCHVIL @ HVDCDC_Status1 on the CAN interface. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface NHVDCDCHVIL @ HVDCDC_Status1 is equal to 1(Open_NHVDCDCHVIL_NW). |
| Process | The HVDCDC shall reports the HVIL status as 1 (Open_NHVDCDCHVIL_NW) when the HVDCDC HVIL is in open state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCHVIL @ HVDCDC_Status1 is equal to 1(Open _NHVDCDCHVIL_NW).. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against HVIL status in <TBD> state

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_571 |
| Customer Req ID | SDS-HVDCDC-221 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall report the HVIL status as 3 (Invalid_NHVDCDCHVIL_NW) when the HVDCDC application is in <TBD> state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the HVIL circuit open loop by removing 6V power from the LV connector. 2. Check the signal NHVDCDCHVIL @ HVDCDC_Status1 on the CAN interface. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface NHVDCDCHVIL @ HVDCDC_Status1 is equal to 3 (Invalid _NHVDCDCHVIL_NW). |
| Process | The HVDCDC shall reports the HVIL status as 3 (Invalid _NHVDCDCHVIL_NW) when the HVDCDC HVIL is in <TBD> state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCHVIL @ HVDCDC_Status1 is equal to 3(Invalid _NHVDCDCHVIL_NW). |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detection of power supply failure condition of the HVIL detection circuit

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_107 |
| Customer Req ID | SDS-HVDCDC-222 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The power supply failure of the HVIL detection circuit shall be detected and report by HVHVDCDC |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the HVIL power supply failure by removing 6V power from the LV connector. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected power supply failure of the HVIL detection circuit |
| Process | 1. The HVDCDC shall reports the HVIL status as 2 when power supply failure of the HVIL detection circuit. 2. Verify that the HVDCDC raises a level 0 fault (NHVDCDCFault = 0x1) upon the detection of the HVIL detection circuit for power supply failure. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected power supply failure of the HVIL detection circuit 3. The HVDCDC shall raises a level 0 fault (NHVDCDCFault = 0x1) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detection of circuit short to high of the HVIL detection circuit

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_108 |
| Customer Req ID | SDS-HVDCDC-222 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The circuit short to high of the HVIL detection circuit shall be detected and report by HVHVDCDC |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required for short to high on HVIL detection circuit. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit short to high failure on the HVIL detection circuit |
| Process | 1. The HVDCDC shall reports the HVIL status as 2 when power supply failure of the HVIL detection circuit. 2. Verify that the HVDCDC raises a level 0 fault (NHVDCDCFault = 0x1) upon circuit short to high failure detection |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit short to high failure on the HVIL detection circuit 3. The HVDCDC shall raises a level 0 fault (NHVDCDCFault = 0x1) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against detection of HVIL loop circuit short to high failure

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_118 |
| Customer Req ID | SDS-HVDCDC-420 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVIL loop faulty status shall be detected as circuit short to high by HVHVDCDC. HVDCDC shall raise report status as NHVDCDCHVIL = 0x2 "Faulty" and level 0 fault |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required for short to high on HVIL detection circuit. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit short to high failure on the HVIL detection circuit |
| Process | 1. The HVDCDC shall reports the HVIL status as 2 when power supply failure of the HVIL detection circuit. 2. Verify that the HVDCDC raises a level 0 fault (NHVDCDCFault = 0x1) upon circuit short to high failure detection |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit short to high failure on the HVIL detection circuit 3. The HVDCDC shall raises a level 0 fault (NHVDCDCFault = 0x1) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against XCP support on Debug CAN

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_109 |
| Customer Req ID | SDS-HVDCDC-109 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC Debug CAN shall support XCP. Refer to the MTS.18.06.0029. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Threshold, Scaling and Offset variable for all the configurable ID’s shall be provided 2. Enable/Disable command for the applicable feature 3. Sample depth (No. of sample) for each analog input signal shall be configurable. |
| Output(s) | 1. The calibration.hex file shall be generated 1. DTC fault ID shall be occurred and observed over DCAN |
| Process | 1. Vary the input variables and observe the expected output. 2. Value for which the appropriate output is observed shall be generated calibration.hex |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. Vary the input variables and observe the expected output. 2. Value for which the appropriate output is observed shall be generated calibration.hex |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement against circuit open of the HVIL detection circuit

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_110 |
| Customer Req ID | SDS-HVDCDC-222 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The circuit open of the HVIL detection circuit shall be detected and report by HVHVDCDC |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. 2. Provide the changes required for open on HVIL detection circuit. |
| Output(s) | Below CAN messages shall be observed on the MCAN interface 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit open failure on the HVIL detection circuit |
| Process | 1. The HVDCDC shall reports the HVIL status as 2 when power supply failure of the HVIL detection circuit. 2. Verify that the HVDCDC raises a level 0 fault (NHVDCDCFault = 0x1) upon circuit open failure detection |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCHVIL @ HVDCDC_Status1 is equal to 2. 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU signal should provide Fault ID for detected circuit open failure on the HVIL detection circuit 3. The HVDCDC shall raises a level 0 fault (NHVDCDCFault = 0x1) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Response requirement against precharge takes longer than a calibratable time to complete.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_58 |
| Customer Req ID | SDS-HVDCDC-225 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the precharge takes longer than a calibratable time to complete. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. KL30 and KL15(IGN) shall On. 2. The 800V side (high side) voltage of the power module shall set to greater than 480V(e.g. 540V). 3. Ensure that NHVDCDCFault shall equal to 0x0 (no fault) on the CAN trace window. 4. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 5. Once the HV DCDC application shall in the "Enable" substate of the "HV_ON" state within 1.5 seconds, keep the low side EMI voltage (VHVDCDC400V_emi) less than half of the high side voltage(VHVDCDC800V_VICOR) minus 10V(VPrecharge_C). 6. Check fault response time from the time when the application enters into the "Close_precharge_Ctt" substate of the "HV_ON" state to the time when the level 2 fault is raised. |
| Output(s) | At Step 3: NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x0 (“No Fault") At Step 5: NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x13(19) ("DIAGNOSTIC_SM_HVON_TIMEOUT_04_FAULT_IDt") and shall be raised after the fault response time of 2000ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x6 ("Faulty") |
| Process | The application shall not transit from substate “Close_precharge_Ctt” to “Close_400V_SolidStateSw” state when low side voltage (VHVDCDC400V_emi) shall less than half of high side voltage(VHVDCDC800V_VICOR) minus 10V(VPrecharge_C) and it transit to "Faulty" state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 and KL15 shall be off |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x13(19) ("DIAGNOSTIC_SM_HVON_TIMEOUT_04_FAULT_IDt") NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x6 ("Faulty") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Response requirement against 400V side solid state switch takes longer than a calibratable time to open/close.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_59 |
| Customer Req ID | SDS-HVDCDC-230 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the 400V side solid state switch takes longer than a calibratable time to close or to open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide short to high on Power board . 2. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with signal value as 0x00 ("Standby") via Canalyzer to HVDCDC application. 3. Once the HV DCDC application shall in the "Disable" substate of the "discharge" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V. 4. Check fault response time from the time when the application enters into the "Open_400V_SolidStateSw" substate of the "Discharge" state to the time when the level 2 fault shall be raised. |
| Output(s) | At Step 4: NHVDCDCStatus @ HVDCDC_Status1 is set to 0x5 ("Discharge 400V") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0xA ("Shutdown_OpenSolidStateSw"). HVDCDC_SSW_pin_sts @ HVDCDCPin_sts is set to 1("Close). NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2"). NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x17(23) and is raised after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC application shall transits to the "Faulty_PowerDn_SafeState" state from the “Open_400V_SolidStateSw” substate of the Discharge state when the solid state switch status confirms to be Closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal") 2. No fault shall be raised, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2"). NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 0x17(23) and shall be raised after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug shall set to 0x11 ("PowerDown_Safe_State") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Fault requirement against 400V side solid state switch takes longer than a calibratable time to open/close or is stuck.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_60 |
| Customer Req ID | SDS-HVDCDC-230 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the 400V side solid state switch takes longer than a calibratable time to close or the 400V side solid state switch is stuck open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. KL30 and KL15(IGN) shall On. 2. The 800V side (high side) voltage of the power module shall set to greater than 480V(e.g. 540V) 3. Ensure that NHVDCDCFault shall equal to 0x0 (no fault) on the CAN trace window 4. Make necessary changes on power board to make solid state switch stuck open 5 Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 6. Check fault response time from the time when the application enters into the "Close_400V_SolidStateSw" substate of the "HV_ON" state to the time when the level 2 fault is raised. |
| Output(s) | At Step 3: NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x3 ("Precharge 400V") NHVDCDCStatusDebug @ HVDCDC_Debug shall set to 0x4 (""PrechargeLS_CloseSolidStateSwt") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 79 and shall be raised after the fault response time of 1000ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x6 ("Faulty") At Step 4: HVDCDC_SSW_pin_sts @ HVDCDCPin_sts shall set to 0("Open). |
| Process | The HVDCDC application shall transits to the "Faulty_PowerDn_SafeState" state from the “Open_400V_SolidStateSw” substate of the Discharge state when the solid state switch status confirms to be Closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal") 2. No fault shall be raised, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatusDebug @ HVDCDC_Debug shall set to 0x4 (""PrechargeLS_CloseSolidStateSwt") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 79 and shall be raised after the fault response time of 1000ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x6 ("Faulty") HVDCDC_SSW_pin_sts @ HVDCDCPin_sts shall set to 0("Open). |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### DTC requirement against 400V side solid state switch is opened under load.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_61 |
| Customer Req ID | SDS-HVDCDC-233 (Obsolute in 2.5) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall set a DTC if the 400V side solid state switch is opened under load. The HVDCDC shall increment the counter of the event of the 400V side solid state switch open under load in the NvM. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Apply 480V on High side 2. Enable command (application in HVON/Discharge state) |
| Output(s) | HVDCDC shall set the DTC and detect level 2 fault for 400V side Solid state switch HVDCDC shall maintain the number of times the open load fault is triggered |
| Process | Verify that the HVDCDC shall set DTC for 400V side solid state switch opened, 400 side voltage is greater than threshold voltage. Verify that the HVDCDC shall increment the counter every-time when fault is observed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal") 2. No fault shall be raised, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool HVDCDC shall set the DTC and detect level 2 fault for 400V side Solid state switch. HVDCDC shall maintain the number of times the open load fault is triggered |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of UDS routine support to reset the NvM counter of the 400V side solid state switch open under load event.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_111 |
| Customer Req ID | SDS-HVDCDC-234 (Obsolute in 2.5) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall implement a UDS routine to reset the NvM counter of the 400V side solid state switch open under load event. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement if the 400V side solid state switch / eFuse is opened due to a fault condition.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_112 |
| Customer Req ID | SDS-HVDCDC-347 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Level 2 fault shall be raised by HVDCDC if the 400V side solid state switch / eFuse is opened due to a fault condition. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Apply 480V on High side 2. Enable command (application in HVON/Discharge state) 3. Fault condition that will make 400V side Solid state switch / Efuse open in normal/standby state |
| Output(s) | HVDCDC shall detect level 2 fault for 400V side Solid state switch / Efuse is opened. HVDCDC shall set DTC for 400V side Solid state switch / Efuse is opened. |
| Process | Verify that the HVDCDC shall set DTC for 400V side solid state switch opened when Efuse shall opened in normal/standby state due to Fault condition . |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal") 2. No fault shall be raised, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool HVDCDC shall detect level 2 fault for 400V side Solid state switch / Efuse is opened. HVDCDC shall set DTC for 400V side Solid state switch / Efuse is opened. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of response on crash event received on vehicle CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_113 |
| Customer Req ID | SDS-HVDCDC-240 (Obsolete in SDS Version 2.2) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if a crash event received on vehicle CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of response on crash CAN message(s) Rx timeout is detected.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_114 |
| Customer Req ID | SDS-HVDCDC-241 (Obsolete in SDS Version 2.2) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the crash CAN message(s) Rx timeout is detected. The crash CAN Rx timeout diagnostic should only be activated when 800V DC link voltage is high. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDCStatus = 0, NHVDCDCStatusDebug =0, NHVDCDCFault = 0 (No Faults) |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detection for temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_62 |
| Customer Req ID | SDS-HVDCDC-299 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The 800V side (high side) voltage of the power module should greater than 480V(e.g. 540V) |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 71 ("DIAGNOSTIC_800V_VICOR_IND_OC_FAULT_ID") |
| Process | Verify that the HVDCDC shall raises a level 2 fault upon the detection of the temperature sensor Vicor_ind Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x0 ("Standby") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 71 ("DIAGNOSTIC_800V_VICOR_IND_OC_FAULT_ID") |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_63 |
| Customer Req ID | SDS-HVDCDC-299 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The 800V side (high side) voltage of the power module should greater than 480V(e.g. 540V) |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 72 ("DIAGNOSTIC_800V_VICOR_IND_SC_FAULT_ID") |
| Process | Verify that the HVDCDC shall raises a level 2 fault upon the detection of the temperature sensor Vicor_ind Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x0 ("Standby") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 72 ("DIAGNOSTIC_800V_VICOR_IND_SC_FAULT_ID") |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detection for temperature sensor 400V EMI Temp (THVDCDC2) failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_65 |
| Customer Req ID | SDS-HVDCDC-300 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 400V EMI Temp failure circuit by Open Circuit. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 400V EMI Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_64 |
| Customer Req ID | SDS-HVDCDC-299 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor_ind Temp (THVDCDC1, analogue isolator) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The 800V side (high side) voltage of the power module should greater than 480V(e.g. 540V) |
| Output(s) | 1. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") 2. NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 71 ("DIAGNOSTIC_800V_VICOR_IND_OC_FAULT_ID") |
| Process | Verify that the HVDCDC shall raises a level 2 fault upon the detection of the temperature sensor Vicor_ind Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 should equal to 0x0 ("Standby") and signal NHVDCDCStatusDebug @ HVDCDC_Debug should equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 73 ("DIAGNOSTIC_800V_VICOR_IND_STG_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_66 |
| Customer Req ID | SDS-HVDCDC-300 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 400V EMI Temp failure circuit by Short to high. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 400V EMI Temp failure circuit by Short to high. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_67 |
| Customer Req ID | SDS-HVDCDC-300 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 400V EMI Temp failure circuit by Short to Ground. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 400V EMI Temp failure circuit by Short to Ground. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_68 |
| Customer Req ID | SDS-HVDCDC-300 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 400V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 400V EMI Temp failure circuit by Open Circuit. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 400V EMI Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 86 (“DIAGNOSTIC_TEMP_EMI_400V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detection for temperature sensor 800V EMI Temp (THVDCDC2) failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_69 |
| Customer Req ID | SDS-HVDCDC-301 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 800V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 800V EMI Temp failure circuit by Open Circuit. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 800V EMI Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor 800V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_70 |
| Customer Req ID | SDS-HVDCDC-301 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 800V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 800V EMI Temp failure circuit by Short to High fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 800V EMI Temp failure circuit by circuit short to high. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor 800V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_71 |
| Customer Req ID | SDS-HVDCDC-301 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor 800V EMI Temp (THVDCDC2) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor 800V EMI Temp failure circuit by Short to GND fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor 800V EMI Temp failure circuit by circuit short to gnd. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 87 (“DIAGNOSTIC_TEMP_EMI_800V_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detection for temperature sensor Vicor Temp (THVDCDC4) failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_72 |
| Customer Req ID | SDS-HVDCDC-302 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor Temp (THVDCDC4) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor VICOR Temp failure by open circuit fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor VICOR Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor Vicor Temp (THVDCDC4) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_73 |
| Customer Req ID | SDS-HVDCDC-302 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor Temp (THVDCDC4) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor VICOR Temp failure by short to high fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor VICOR Temp failure circuit by short to high. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detect the temperature sensor Vicor Temp (THVDCDC4) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_74 |
| Customer Req ID | SDS-HVDCDC-302 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Vicor Temp (THVDCDC4) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor VICOR Temp failure by short to ground fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor VICOR Temp failure circuit by short to ground. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 88 (“DIAGNOSTIC_TEMP_VICOR_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of detection for temperature sensor Solid State Switch Temp (THVDCDC5) failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_75 |
| Customer Req ID | SDS-HVDCDC-303 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Solid State Switch Temp (THVDCDC5) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor solid state switch Temp failure circuit by open circuit fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor solid state switch Temp failure circuit by an open circuit. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to detect the temperature sensor Solid State Switch Temp (THVDCDC5) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_76 |
| Customer Req ID | SDS-HVDCDC-303 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Solid State Switch Temp (THVDCDC5) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor solid state switch Temp failure circuit by short to high fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor solid state switch Temp failure circuit by an short to high. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to detect the temperature sensor Solid State Switch Temp (THVDCDC5) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_77 |
| Customer Req ID | SDS-HVDCDC-303 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the temperature sensor Solid State Switch Temp (THVDCDC5) failure (power supply failure, circuit short to high, circuit short to ground or circuit open) and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 3. Make necessary arrangement to provide detection of the temperature sensor solid state switch Temp failure circuit by short to ground fault. |
| Output(s) | Below CAN messages are observed on the CAN interface NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) and this fault is raised after 50ms |
| Process | The HVDCDC raises a level 2 fault upon the detection of the temperature sensor solid state switch Temp failure circuit by short to ground. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCStatus @ HVDCDC_Status1 is set to 0x6 ("Faulty") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 89 (“DIAGNOSTIC_TEMP_SSS_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of inhibiting all diagnostics on the vehicle CAN communication for a calibratable period upon wakeup / re-wakeup

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_78 |
| Customer Req ID | SDS-HVDCDC-187 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall inhibit all diagnostics on the vehicle CAN communication for a calibratable period upon wakeup / re-wakeup. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Keep VICOR high side voltage greater than 700V to raise a level 2 fault. 2. Turn ON KL30. 3. Check the inhibit time upon wakeup by checking the time from the time when CAN communication shall start to the time when level 2 fault is raised minus fault response time of respective level 2 fault. 4. Turn off KL15. 5. Turn ON KL15. 6. Check the inhibit time upon re-wakeup by checking the time from the time when CAN communication shall start after step 5 to the time when level 2 fault is raised minus fault response time of respective level 2 fault |
| Output(s) | Application shall provide CAN messages status as below, At Input Step 3: 1) Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 1(0x1)("800V OV fault") and this fault is received after 2 seconds including 500ms inhibit time and 500ms fault response time. At Input Step 6: 2) Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 1(0x1)("800V OV fault") and this fault is received after 2 seconds including 500ms inhibit time and 500ms fault response time. |
| Process | The HVDCDC application inhibits all non CAN diagnostics on the vehicle CAN communication for a calibratable period of 2 seconds upon wakeup and re-wakeup |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 is OFF and KL15 are ON |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 1(0x1)("800V OV fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement of inhibiting all diagnostics on the vehicle CAN communication for a calibratable period upon wakeup / re-wakeup

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_79 |
| Customer Req ID | SDS-HVDCDC-187 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall inhibit all diagnostics on the vehicle CAN communication for a calibratable period upon wakeup / re-wakeup. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x03 ("un-used command") via CAN interface to HVDCDC application to raise a level 2 fault. 2. Turn ON KL30. 3. Check the inhibit time upon wakeup by checking the time from the time when CAN communication is started to the time when level 2 fault is raised minus fault response time of respective level 2 fault. 4. Turn off KL15. 5. Turn ON KL15. 6. Check the inhibit time upon re-wakeup by checking the time from the time when CAN communication is started after step 5 to the time when level 2 fault is raised minus fault response time of respective level 2 fault |
| Output(s) | Application shall provide CAN messages status as below, At Input Step 3: 1) Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal 37(0x25)("Invalid command timeout fault") and this fault is received after 12.5 seconds including 12 seconds inhibit time and 500ms fault response time. At Input Step 6: 2) Below CAN messages are observed on the CAN interface NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 37(0x25)("Invalid command timeout fault") and this fault is received after 12.5 seconds including 12 seconds inhibit time and 500ms fault response time. |
| Process | The HVDCDC application inhibits all non CAN diagnostics on the vehicle CAN communication for a calibratable period of 2 seconds upon wakeup and re-wakeup |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | KL30 is OFF and KL15 are ON |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal 37(0x25)("Invalid command timeout fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to latch NHVDCDCFault = 0x1 "Level 0" for 5 seconds (HVDCDC_CAL) or until the fault trigger is no longer present, whichever is longer.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_80 |
| Customer Req ID | SDS-HVDCDC-176 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | Once a level 0 fault is triggered, the HVDCDC shall latch NHVDCDCFault = 0x1 "Level 0" for 5 seconds (HVDCDC_CAL) or until the fault trigger is no longer present, whichever is longer. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Continuously monitor the temperature rise (TVHDCDC5 @ HVDCDC_Status3) and fault ID (NVHDCDCFaultID @ HVDCDC_Debug_PDU) of the CAN interface, using a heat gun apply heat to the local sensor in turn U402 till the temperature raises above 85 degrees and hold the heat gun for 15 seconds then take it away. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x8 (SSS over-voltage) TVHDCDC5 @ HVDCDC_Status3 is less than or equal to 50 degrees. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Process | That once a level 0 fault is triggered, the HVDCDC latch NHVDCDCFault = 0x1 "Level 0" until the fault is present for more than 5 seconds. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 0") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x8. TVHDCDC5 @ HVDCDC_Status3 is less than or equal to 50 degrees. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for NHVDCDCFault Latch response on level 1 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_81 |
| Customer Req ID | SDS-HVDCDC-177 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | Once a level 1 fault Shall triggered, the HVDCDC shall latch NHVDCDCFault = 0x2 "Level 1" for 5 seconds (HVDCDC_CAL) or until the fault trigger Shall no longer present, whichever is longer. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Continuously monitor the fault ID (NVHDCDCFaultID @ HVDCDC_Debug_PDU) on the CAN interface 2. Raise a level 1 fault and clear the raised fault after 3 seconds |
| Output(s) | Application shall provide CAN messages status as below, CAN messages are observed for 5 seconds, NHVDCDCFault @ HVDCDC_Status1 Shall equal to 0x1 ("Level 1") CAN messages are observed after 5 seconds, NHVDCDCFault @ HVDCDC_Status1 Shall equal to 0x0 ("No fault") |
| Process | Once a level 1 fault shall triggered, the HVDCDC latch NHVDCDCFault = 0x2 "Level 1" for 5 seconds (HVDCDC_CAL) if the fault Shall got cleared within 5 seconds. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 Shall equal to 0x1 ("Level 1") NHVDCDCFault @ HVDCDC_Status1 Shall equal to 0x0 ("No fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for NHVDCDCFault Latch response on level 1 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_82 |
| Customer Req ID | SDS-HVDCDC-177 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | Once a level 1 fault is triggered, the HVDCDC shall latch NHVDCDCFault = 0x2 "Level 1" for 5 seconds (HVDCDC_CAL) or until the fault trigger is no longer present, whichever is longer. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Continuously monitor the fault ID (NVHDCDCFaultID @ HVDCDC_Debug_PDU) on the CAN interface 2. Raise a level 1 fault and clear the raised fault after 10 seconds |
| Output(s) | Below CAN messages are observed for 10 seconds, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 1") Below CAN messages are observed after 10 seconds, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Process | Once a level 1 fault is triggered, the HVDCDC latches NHVDCDCFault = 0x2 "Level 1" until the fault is present for more than 5 seconds. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 1") NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to activate and deactivate the individual diagnostic.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_120 |
| Customer Req ID | SDS-HVDCDC-178 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The activation and deactivation of the individual diagnostic in the HVDCDC software implementation shall be configurable via calibration. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. List of Fault ID’s, DTC’s to be configured via calibration for activation and deactivation |
| Output(s) | The HVDCDC shall activate and deactivate the DTC’s and Faults based on the configuration set. |
| Process | 1. Provided NM message 2. Open the CANape tool and deactivate the any fault/DTC (For. E.g 800V_OV) 3. Provided the overvoltage at 800v side to generate over voltage fault. 4. 800V_OV fault should not be raised |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool and CANape Tool. Verify that HVDCDC shall activate and deactivate the DTC’s and Faults based on the configuration set. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to inhibit all the diagnostics during the wakeup process.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_121 |
| Customer Req ID | SDS-HVDCDC-179 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall inhibit all the diagnostics during the wakeup process: - before transitioning from "Sleep" state to "Standby" state; AND - within HVDCDC_CAL sec since the HVDCDC transitions from "Sleep" state to "Standby" state |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Continuously monitor the fault ID (NVHDCDCFaultID @ HVDCDC_Debug_PDU) on the CAN interface 2. Raise any fault before the ECU entering standby state 3. Set the calibratable time HVDCDC_CAL to inhibit the diagnostics 4. Provide NM message to HVDCDC |
| Output(s) | The diagnostics shall be inhibit when HVDCDC is transitioning from Sleep to Standby state. |
| Process | 1. Raise any fault before the ECU entering standby state 2. Set the calibratable time HVDCDC_CAL to inhibit the diagnostics 3. Provide NM message to HVDCDC 4. Observe that the diagnostics shall be inhibit when HVDCDC is transitioning from Sleep to Standby state. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. Observe that the diagnostics shall be inhibit when HVDCDC is transitioning from Sleep to Standby state. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to inhibit all the diagnostics during the Sleep process.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_122 |
| Customer Req ID | SDS-HVDCDC-179 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall inhibit all the diagnostics during the sleep process: - after the transition from "Standby" to "Sleep" begins; OR - in the "Sleep" state. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Continuously monitor the fault ID (NVHDCDCFaultID @ HVDCDC_Debug_PDU) on the CAN interface 2. Raise any fault before the ECU entering standby state 3. Set the calibratable time HVDCDC_CAL to inhibit the diagnostics 4. Stop sending NM message to HVDCDC |
| Output(s) | The diagnostics shall be inhibit when HVDCDC is transitioning from Standby to Sleep state or in Sleep State. |
| Process | 1. Raise any fault before the ECU entering standby state 2. Set the calibratable time HVDCDC_CAL to inhibit the diagnostics 3. Stop sending NM message to HVDCDC 4. Observe that the diagnostics shall be inhibit when HVDCDC is transitioning from Standby to Sleep state or in Sleep State. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. Observe that the diagnostics shall be inhibit when HVDCDC is transitioning from Standby to Sleep state or in Sleep State |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to assign each failure mode to one unique DTC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_123 |
| Customer Req ID | SDS-HVDCDC-180 |
| Milestone | B-Sample/PF3.0 |
| Purpose1 | The HVDCDC shall assign each failure mode to one unique DTC. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide multiple faults on HVDCDC |
| Output(s) | 1. Observe that each fault should have unique DTC ID |
| Process | 1. Provide the NM message 2. HVDCDC should be in any state except shutdown state 3. Provide multiple faults on HVDCDC 4. Observe that each fault should have unique DTC ID |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. Observe that each fault should have unique DTC ID |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report the equivalent failure codes onto the Debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_124 |
| Customer Req ID | SDS-HVDCDC-181 |
| Milestone | B-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall report the equivalent failure codes onto the Debug CAN. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Provide multiple faults on HVDCDC |
| Output(s) | Observe that each fault ID shall be reported on to Debug CAN |
| Process | 1. Provide the NM message 2. HVDCDC should be in any state except shutdown state 3. Provide multiple faults on HVDCDC 4. Observe that each fault ID shall be reported on to Debug CAN |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 ("Level 1") NHVDCDCFault @ HVDCDC_Status1 is equal to 0x0 ("No fault") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for precharge circuit failure detection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_83 |
| Customer Req ID | SDS-HVDCDC-226 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the precharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the open circuit on the power board precharge circuit 2. Send "Enable" command(0x1) from EDSU. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 58 ("DIAGNOSTIC_PRSCHRG_CKT_OPEN_FAULT_ID") and is raised after the fault response time of 50ms. |
| Process | The HVDCDC application shall detects the failure of the precharge circuit by an open circuit and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool 1. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") 2. NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 58 ("DIAGNOSTIC_PRSCHRG_CKT_OPEN_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for precharge circuit failure detection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_84 |
| Customer Req ID | SDS-HVDCDC-226 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the precharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the Short to High on the power board precharge circuit. 2. Send "Enable" command(0x1) from EDSU. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 56 ("DIAGNOSTIC_PRCHRG_SHORT_TO_HIGH_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the failure of the precharge circuit by the circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 56 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for precharge circuit failure detection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_85 |
| Customer Req ID | SDS-HVDCDC-226 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the precharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the Short to Ground on the power board precharge circuit. 2. Send "Enable" command(0x1) from EDSU. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 57 ("DIAGNOSTIC_PRSCHRG_SHORT_TO_GND_FAULT_ID") and is raised after the fault response time of 50ms. |
| Process | The HVDCDC application detects the failure of the precharge circuit by an open circuit and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall be in Standby state, the value of CAN signal HVDCDCStatus @ HVDCDC_Status1 shall equal to 0x0 ("Standby"), and signal NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x0 ("Standby"). 2. No fault shall be raised, Value of CAN signal NHVDCDCFault @ HVDCDC_Status1 shall equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 57 ("DIAGNOSTIC_PRSCHRG_SHORT_TO_GND_FAULT_ID") |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for detection of failure of the 400V side solid state switch circuit.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_86 |
| Customer Req ID | SDS-HVDCDC-231 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V side solid state switch circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. On the power board remove R402. 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 67 (“DIAGNOSTIC_SSS_OPEN_CKT_FAULT_ID”) and this fault is raised after fault response time 500ms. |
| Process | The HVDCDC application detects the failure of the 400V side solid state switch circuit by an open circuit and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 67 (“DIAGNOSTIC_SSS_OPEN_CKT_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for the failure of the 400V side solid state switch circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_87 |
| Customer Req ID | SDS-HVDCDC-231 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V side solid state switch circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide 400V side solid state switch short circuit/ short to high failure on the power board. 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 65 (“DIAGNOSTIC_SSS_SHRT_CKT_FAULT_ID”) and this fault is raised after fault response time 500ms. |
| Process | The HVDCDC application detects the failure of the 400V side solid state switch by driver failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 65 (“DIAGNOSTIC_SSS_SHRT_CKT_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for the failure of the 400V side solid state switch circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_88 |
| Customer Req ID | SDS-HVDCDC-231 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V side solid state switch circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide 400V side solid state switch driver failure on the power board. 2. Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface cyclically every 10ms. and observe fault in faulty state. |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 66 (“DIAGNOSTIC_SSS_DRVR_FAIL_FAULT_ID”) |
| Process | The HVDCDC application shall detects the failure of the 400V side solid state switch by driver failure and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 66 (“DIAGNOSTIC_SSS_DRVR_FAIL_FAULT_ID”) |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for fault response if 400V side solid state switch is stuck closed or stuck open.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_89 |
| Customer Req ID | SDS-HVDCDC-232 |
| Mileston e | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the 400V side solid state switch is stuck closed or stuck open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. KL30 and KL15(IGN) is On. 2. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 3. Ensure that NHVDCDCFault shall equal to 0x0 (no fault) on the CAN trace window 4. Provide Short to GND on 400V side solid state switch of the power board. 5 Send NHVDCDCCommand as 0x01 ("Enable") using the CAN interface. 6. Check fault response time from the time when the application enters into the "Close_400V_SolidStateSw" substate of the "HV_ON" state to the time when the level 2 fault is raised. |
| Output(s) | Below CAN messages shall be observed sequentially NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x3 ("Precharge 400V") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x4 (""PrechargeLS_CloseSolidStateSwt") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 79 and is raised after the fault response time of 1000ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x6 ("Faulty") At Step 4: HVDCDC_SSW_pin_sts @ HVDCDCPin_sts shall set to 0("Open). |
| Process | HVDCDC shall raise a level 2 fault if the 400V side solid state switch takes longer than a calibratable time to close or the 400V side solid state switch is stuck open. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for fault response if 400V side solid state switch is stuck closed or stuck open.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_90 |
| Customer Req ID | SDS-HVDCDC-232 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault if the 400V side solid state switch is stuck closed or stuck open. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide Short to High on 400V side solid state switch of the power board. 2. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with the signal 0x00 ("Standby") via Canalyzer to the HVDCDC application. 3. Once the HV DCDC application shall in the "Disable" substate of the "Discharge" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V. 4. Check fault response time from the time when the status of "HVDCDC_SSW_pin_sts" shall 1 to the time when the level 2 fault is raised. |
| Output(s) | Below CAN messages are observed sequentially, NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x5 ("Discharge 400V") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0xA ("Shutdown_OpenSolidStateSw"). HVDCDC_SSW_pin_sts @ HVDCDCPin_sts shall set to 1("Close). NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x3 (“level 2"). NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 80 and is raised after the fault response time of 1000ms. NHVDCDCStatus @ HVDCDC_Status1 shall set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug shall set to 0x11 ("PowerDown_Safe_State") |
| Process | The HVDCDC shall raise a level 2 fault if the 400V side solid state switch is stuck closed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for detetction of 400V active discharge is not effective

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_91 |
| Customer Req ID | SDS-HVDCDC-237 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect if the 400V active discharge is not effective and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with signal value as 0x02 ("Safe State") via Canalyzer to HVDCDC application. 2. Once the HV DCDC application shall in the "Disable_3" substate of the "Faulty" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V and give more than 45V to 400V bus(400V EMI) 3. Check the fault response time by checking the HVDCDC applications transition time from "Active_Discharge_400V" to "Disable_400V_AD" substate of the "Discharge" state. |
| Output(s) | At Step 3: Below CAN messages are observed sequentially, NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 70 and is raised after the fault response time of 1900ms. |
| Process | The HVDCDC shall detects the 400V active discharge shall not effective and raise a level 0 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for the 400V active discharge is not effective and raise a level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_92 |
| Customer Req ID | SDS-HVDCDC-237 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect if the 400V active discharge is not effective and raise a level 0 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN command NHVDCDCCommand @ EDSU_HVDCDC_Cmd with signal value as 0x00 ("Standby") via Canalyzer to HVDCDC application. 2. Once the HV DCDC application shall in the "Disable" substate of the "discharge" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V and give more than 60V to 400V bus(400V EMI). 3. Check the fault response time by checking the HVDCDC applications transition time from "Active_Discharge_400V" to "Disable_400V_AD" substate of the "Discharge" state. |
| Output(s) | At Step 3: Below CAN messages are observed sequentially, NHVDCDCFault @ HVDCDC_Status1 is equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 24 and is raised after the fault response time of 1800ms. |
| Process | The HVDCDC detects the 400V active discharge is not effective and raise a level 0 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 1, NHVDCDCStatusDebug = 6, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 24 and is raised after the fault response time of 1800ms. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for detetction of failure of the 400V active discharge circuit.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_93 |
| Customer Req ID | SDS-HVDCDC-238 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V active discharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide open circuit for 400V active discharge circuit on the power board 2. Send Standby command(0x2) from EDSU |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 61 ("DIAGNOSTIC_DISCHRG_CKT_OPEN_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the failure of the 400V active Discharge circuit by an open circuit and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 61 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for the failure of the 400V active discharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_94 |
| Customer Req ID | SDS-HVDCDC-238 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V active discharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide short to high on power module for 400V active discharge circuit 2. Send Standby command(0x2) from EDSU |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 59 ("DIAGNOSTIC_DISCHRG_SHORT_TO_HIGH_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the failure of the 400V active Discharge circuit by a circuit short to high and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 59 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for detect the failure of the 400V active discharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_95 |
| Customer Req ID | SDS-HVDCDC-238 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall detect the failure of the 400V active discharge circuit (power supply failure, circuit short to high, circuit short to ground, or circuit open) and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide short to GND on power module for 400V active discharge circuit 2. Send Standby command(0x2) from EDSU |
| Output(s) | Application shall provide CAN messages status as below, NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 60 ("DIAGNOSTIC_DISCHRG_SHORT_TO_GND_FAULT_ID") and is raised after the fault response time of 500ms. |
| Process | The HVDCDC application detects the failure of the 400V active Discharge circuit by circuit short to ground and raises a level 2 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 is set to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU is equal to 60 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for support ECU Variant Type in accordance with the existing MSL.01.04.0028 Generic Diagnostics Specification.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_96 |
| Customer Req ID | SDS-HVDCDC-312 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall support ECU Variant Type in accordance with the existing MSL.01.04.0028 Generic Diagnostics Specification. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send a physical request to read data by an identifier($22 F1 0F) 2. Send a functional request to read data by an identifier($22 F1 0F) |
| Output(s) | HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX" HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX" |
| Process | The HVDCDC detects the ECU variant type in accordance with flashed software. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC shall have valid software. HVDCDC is in default, extended, or programming session. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for ECU Variant Type value shall be in the range of 0000 - FFFF.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_97 |
| Customer Req ID | SDS-HVDCDC-313 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The ECU Variant Type value shall be in the range of 0000 - FFFF |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send a physical request to read data by an identifier($22 F1 0F) 2. Send a functional request to read data by an identifier($22 F1 0F) |
| Output(s) | HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Process | The HVDCDC detects the ECU variant type value in range of 0000- FFFF. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC shall have valid software. HVDCDC is in default, extended, or programming session. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### First CDD release Requirement.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_98 |
| Customer Req ID | SDS-HVDCDC-314 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | First CDD release shall include one Base Variant (which includes all services) under "Common Diagnostics". Variant "HVDCDC_VARIANT_0000" shall be added under "Variants". |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send a physical request to read data by an identifier($22 F1 0F) 2. Send a functional request to read data by an identifier($22 F1 0F) |
| Output(s) | HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Process | The HVDCDC detects the ECU variant type value in range of 0000- FFFF. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC shall have valid software. HVDCDC is in default, extended, or programming session. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Following CDD release Requirement.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_99 |
| Customer Req ID | SDS-HVDCDC-315 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | Following CDD releases shall include new variants under "Variants" adding "HVDCDC_VARIANT_xxxx"(if new services or modification to existing services are made). Variant name value shall be incremented by 1 within the range of [313]. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send a physical request to read data by an identifier($22 F1 0F) 2. Send a functional request to read data by an identifier($22 F1 0F) |
| Output(s) | HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Process | The HVDCDC detects the ECU variant type value in range of 0000- FFFF. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC shall have valid software. HVDCDC is in default, extended, or programming session. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Service F10F Requirement.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DIAG_100 |
| Customer Req ID | SDS-HVDCDC-316 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | Service F10F shall return the ECU Variant value intended for the correspondent SW release. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send a physical request to read data by an identifier($22 F1 0F) 2. Send a functional request to read data by an identifier($22 F1 0F) |
| Output(s) | HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Process | The HVDCDC detects the ECU variant type value in range of 0000- FFFF. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | HVDCDC shall have valid software. HVDCDC is in default, extended, or programming session. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. HV DC-DC bootloader gives a positive response to the request to read the ECU variant type($62 F1 0F) with ECU variant type data as "HVDCDC_VARIANT_XXXX in range of 0000 -FFFF" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for reporting overtemperature warning for the power module

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Diag_101 |
| Customer Req ID | SDS-HVDCDC-435 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Raise the level 0 fault, If power module reports the over temperature warning. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Low |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Command response for the STATUS_TEMPERATURE (0x7D) from Power Module |
| Output(s) | 1.Raise a level 0 fault if the debounce time (HVDCDC_CAL) is overlapped. 2.Fault ID <TBD> to be raised. 3.Set the DTC |
| Process | 1.The standard PMBus command STATUS_TEMPERATURE (0x7D) shall be read and checked across the value 0x40 2.Increment the debounce counter 3.Check if the debounce counter maximum physical value is exceeded 4.The fault should be raised after a configurable debounce time is elapsed. |
| Mandatory Fields | No such mandatory fields is applicable |
| Pre-Loaded Values | No preloaded values |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.Verify that the application raises a Level 0 fault, in case of over temperature warning. 2.Verify the fault 3.Verify the DTC |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## General Diagnostics

### Requirements

#### Requirement to comply with the MAL Generic Diagnostics.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_01 |
| Customer Req ID | SDS-HVDCDC-277 |
| Milestone | Pre C-Sample/PF3.5 |
| Purpose1 | ## The HVDCDC shall comply to the MAL Generic Diagnotics requirements. Refer to MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | ## Input as expected to a specific requirement mention in MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Output(s) | ## Output as expected to a specific requirement mention in MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Process | Process shall be as per specific Software requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific Software requirement. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to comply with the MAL Generic Diagnostics.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_17 |
| Customer Req ID | SDS-HVDCDC-250 |
| Milestone | Pre C-Sample/PF3.0 |
| Purpose1 | ## The HVDCDC shall comply to the MAL Generic Diagnotics (UDS and software flashing) requirements. Refer to MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input as expected to a specific UDS and software flashing requirement mention in MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Output(s) | Output as expected to a specific UDS and software flashing requirement mention in MSL.01.04.0028 Generic Diagnostic Specification.docx |
| Process | Process shall be as per specific UDS and software flashing requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific UDS and software flashing requirement. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: Software reflash flow (PF2.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_02 |
| Customer Req ID | SDS-HVDCDC-349 |
| Milestone | B-Sample/PF2.0(Need to confirm with MAL as it is Design Constraints) |
| Purpose1 | ## The PF2.0 HVHVDCDC shall cover the following generic diagnostic features: ## - Software reflash flow |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. New feature updated binary/hex file should be used 2. Re-Program shall be selected from PCOTAPP. |
| Output(s) | 1. The New feature binary shall be reflash successfully on HVDCDC HW. 2. PCOTAPP shall show firmware upgrade successful popup. |
| Process | Verify that Software reflash shall successfully implemented on HVDCDC |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. ECU shall be in application mode. It shall not be in bootloader mode. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that Software reflash shall successfully implemented on HVDCDC |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: ECU (CDD) Variant Type (PF2.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_03 |
| Customer Req ID | SDS-HVDCDC-349 |
| Milestone | B-Sample/PF2.0(Need to confirm with MAL as it is Design Constraints) |
| Purpose1 | ## The PF2.0 HVHVDCDC shall cover the following generic diagnostic features: - ECU (CDD) Variant Type |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Modified CDD file which will lead to change in software and change in ECU variant type |
| Output(s) | The ECU variant type and software shall be changed with change in CDD file |
| Process | Verify that the ECU variant type is changed and the change in software shall be observed |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the ECU variant type is changed and the change in software shall be observed |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: Software reflash flow (PF3.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_04 |
| Customer Req ID | SDS-HVDCDC-350 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | ## The PF3.0 HVHVDCDC shall cover the following generic diagnostic features: ## - Software reflash flow |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. New feature updated binary/hex file should be used 2. Re-Program shall be selected from PCOTAPP. |
| Output(s) | 1. The New feature binary shall be reflash successfully on HVDCDC HW. 2. PCOTAPP shall show firmware upgrade successful popup. |
| Process | Verify that Software reflash shall successfully implemented on HVDCDC |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. ECU shall be in application mode. It shall not be in bootloader mode. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that Software reflash shall successfully implemented on HVDCDC |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: Monaco compatible development bootloader (PF3.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_05 |
| Customer Req ID | SDS-HVDCDC-350 |
| Milestone | Pre C-Sample/PF3.0 Phase 2 |
| Purpose1 | ## The PF3.0 HVHVDCDC shall cover the following generic diagnostic features: ## - Monaco compatible development bootloader |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. MAL specific reflash request ID ($FD 01) shall be supported by Monaco tool 2. The bootloader shall be implemented for Monaco tool compatibility |
| Output(s) | Monaco tool shall successfully reflash the new SW binary on HVDCDC HW |
| Process | Verify that Monaco tool shall successfully reflash the new SW binary on HVDCDC HW |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that Monaco tool shall successfully reflash the new SW binary on HVDCDC HW |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: Development Key (PF3.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_06 |
| Customer Req ID | SDS-HVDCDC-350 |
| Milestone | Pre C-Sample/PF3.0 Phase 2 |
| Purpose1 | ## The PF3.0 HVHVDCDC shall cover the following generic diagnostic features: - Development Key |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The Development Keys shall be generated by supplier 2. The development keys shall be used for signature verification before flashing the code into HW |
| Output(s) | 1. The Signature verification shall be successfully done on supplier generated development keys |
| Process | Verify that Signature verification shall be successfully done on supplier generated development keys |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that Signature verification shall be successfully done on supplier generated development keys |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: Snapshot and Extended Data (PF3.5).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_10 |
| Customer Req ID | SDS-HVDCDC-423 (Will be cover in 3.5) |
| Milestone | Pre C-Sample/PF3.5 |
| Purpose1 | ## The PF3.5 HVHVDCDC shall cover the following generic diagnostic features: - Snapshot and Extended Data |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: DTC Details (Set, reset, enable conditions and special instructions if any) (PF3.5).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_11 |
| Customer Req ID | SDS-HVDCDC-423 |
| Milestone | Pre C-Sample/PF3.5 |
| Purpose1 | ## The PF3.5 HVHVDCDC shall cover the following generic diagnostic features: - DTC Details (Set, reset, enable conditions and special instructions if any) |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for the generic diagnostic feature: DTC Status Mask confirmation (PF3.5).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_12 |
| Customer Req ID | SDS-HVDCDC-423 |
| Milestone | Pre C-Sample/PF3.5 |
| Purpose1 | ## The PF3.5 HVHVDCDC shall cover the following generic diagnostic features: - DTC Status Mask confirmation |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement a watchdog monitor.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_07 |
| Customer Req ID | SDS-HVDCDC-290 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC Shall implement a Watchdog monitor |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC shall configure an internal watchdog. 2.The watchdog shall be programmed to trigger a reset or take some corrective action. 3. Provide the fault conditions so that watchdog can trigger HVDCDC reset |
| Output(s) | 1.The Internal watchdog timer shall monitor the proper functioning of the HVDCDC 2. If the HVDCDC fails to respond within a specified timeframe the watchdog shall trigger a reset |
| Process | Verify that the internal watchdog is functioning properly by following input and output methods |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the internal watchdog is functioning properly by following input and output methods |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to report the identification data of the PM by request of a UDS routine.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_13 |
| Customer Req ID | SDS-HVDCDC-424 (Need to discuss Appl team) |
| Milestone | Pre C-Sample/PF3.0 |
| Purpose1 | The HVDCDC shall report the identification data of the PM by request of a UDS routine. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values |  |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to verify that the ETC is erased.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_08 |
| Customer Req ID | SDS-HVDCDC-351 |
| Milestone | Pre C-Sample/PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall verify that the ETC [Section 6] is erased as the flashing pre-condition. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall check for the availability of Embedded Test Code integrate with the basic software stacks. The HVDCDC shall do all the necessary steps to erase the ETC |
| Output(s) | After the EOL test completed, HVDCDC shall erase ETC code when actual SW binary will be flashed first time (After EOL test completion). |
| Process | Verify that the EOL test is completed and HVDCDC shall erase ETC code when actual SW binary will be flashed first time. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. EOL test should be completed 2. Embedded Test Code shall be integrated with the basic software stacks. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the EOL test is completed and HVDCDC shall erase ETC code when actual SW binary will be flashed first time. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to perform Power On Safe Test for checking the safety of the system.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_09 |
| Customer Req ID | SDS-HVDCDC-400 |
| Milestone | Pre C-Sample/PF3.0 Phase 2 |
| Purpose1 | After IGN ON is detected, the HVDCDC shall perform Power On Safe Test for checking the safety of the system before start of operation |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Provide the NM message over CAN. OR - The IGN ON shall be provided to HVDCDC. |
| Output(s) | 1. The HVDCDC shall perform Power On Safe Test which includes below test, - Verifying that the HVDCDC shall receive proper voltage at each terminal of power board as well as controller board - Verify the individual Communication modules like SPI, I2C, UART, CAN etc shall operational - Verify that the Temperature Sensors and current circuitry shall function properly - Verifying the integrity of stored data in non-volatile memory used by the ECU for storing calibration parameters and diagnostic trouble codes. |
| Process | - NM message’s shall be received over CAN. - The HVDCDC shall wakeup from Sleep state. - The HVDCDC shall start performing power on self test. - Verifying that the HVDCDC shall receive proper voltage at each terminal of power board as well as controller board. - Verify the individual Communication modules like SPI, I2C, UART, CAN etc shall operational. - Verify that the Temperature Sensors and current circuitry shall function properly. - Verifying the integrity of stored data in non-volatile memory used by the ECU for storing calibration parameters and diagnostic trouble codes. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. Configure a CAN drivers and CAN routine to handle CAN message received over vehicle CAN/main CAN. 2. The HVDCDC should be in Sleep state 3. The HVDCDC shall not receive CAN NM message “NMZC1Message_Hybrid_ASR_PDU” |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC shall perform Power On Safe Test for checking the safety of the system before start of operation. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### VIN shall be received and stored by the HVDCDC in accordance with the existing MSL.01.04.0028 Generic Diagnostics Specification.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_13 |
| Customer Req ID | SDS-HVDCDC-427 (Need to check with SW team) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The VIN shall be received and stored by the HVDCDC in accordance with the existing MSL.01.04.0028 Generic Diagnostics Specification. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values |  |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement the specific requirements for the DCDC stated in [SDS18-DIA01-01_DCDC].

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_13 |
| Customer Req ID | SDS-HVDCDC-428 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall implement the specific requirements for the DCDC stated in [SDS18-DIA01-01_DCDC] |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input as expected to a specific requirement mention in SDS18-DIA01-01_DCDC document |
| Output(s) | Output as expected to a specific requirement mention in SDS18-DIA01-01_DCDC document |
| Process | Process shall be as per specific Software requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific Software requirement. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement the specific requirements for the DCDC stated in [SDS18-DIA01-01_DCDC].

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_14 |
| Customer Req ID | SDS18-DIA01-01-249 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Read Original VIN when the ECU is in virgin state , once UDS request (0x22F102) is received. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | UDS Request (0x22F102) for reading VIN Number. |
| Output(s) | Upon reading the response of UDS service 0x22F102, the response shall be 0x0000000000000 |
| Process | 1.The HVDCDC shall be in virgin state (state after the controller is reprogrammed) 2. Send the UDS service request(0x22F102) for reading the original VIN. 3. Read original VIN from NVM |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No preloaded values |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that upon reading the response of UDS service 0x22F102, the response shall be 0x0000000000000 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement the specific requirements for the DCDC stated in [SDS18-DIA01-01_DCDC].

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_15 |
| Customer Req ID | SDS18-DIA01-01-249 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Write 13 Byte current VIN into NVM upon UDS request (0x22F190) received. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.Provide 13 Byte VIN (current VIN) with the UDS service. 2. System is in Extended Session for reading the Security level 1 3. System is in programming Session for reading the Security level 2 4.UDS Request (0x22F190) service for writing current VIN. |
| Output(s) | Upon writing the UDS (0x22F190) service, we shall receive a positive response |
| Process | 1. Send the UDS service request (0x22F190) for writing the current VIN with security level 1 access and security level 2 access. 2. Store the current VIN in NVM |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No preloaded values |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the current VIN is correctly written in the NVM, a positive response shall be recorded after writing the UDS service. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement the specific requirements for the DCDC stated in [SDS18-DIA01-01_DCDC].

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Gen_Diag_16 |
| Customer Req ID | SDS18-DIA01-01-249 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | Read 13 Byte current VIN from NVM upon UDS request (0x22F102) is received. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | UDS Request for reading (0x22F102) service. |
| Output(s) | Upon reading the UDS (0x22F102) service ,the stored current VIN shall be displayed (13 Byte data). |
| Process | Send the UDS (0x22F102) service for reading the current VIN. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No preloaded values |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## Power Management Controller interface driver

- Infenion TL35584 PMIC IC is used to manage the Power Supply and Power Safety of the Controller.

- Interface – SPI

- Different operating states of the PMIC such as PowerDown, Init, Normal, Wakup, Sleep, Standby, SafeState.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_02 |
| Customer Req ID | SDS-HVDCDC-171 (This requirement is obsolute) |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall model the power module internal critical temperatures with ±TBD% tolerance across the operating range and temperature. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NA |
| Output(s) | NA |
| Process | The modelled power module internal critical temperatures can correlate to the measured temperatures. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NA |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Requirements

#### Requirement to enable the converter power module (PF2.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_03 |
| Customer Req ID | SDS-HVDCDC-129 |
| Milestone | PF2.1 |
| Purpose1 | The converter power module shall be enable if the conditions are met to enable it. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | EDSU command NHVDCDCCommand == 0x1 "Enable" The HVDCDC converter shall set the HVDCDC_CAL time |
| Output(s) | Enabling HVDCDC power module. NHVDCDCStatus = 0x2 Enable power module. The HVDCDC shall set NHVDCDCStatusDebug = 0x2 Enable the converter power module NHVDCDCStatus = 0x2 Enable power module NHVDCDCStatusDebug = 0x2 Enable power module |
| Process | The HVDCDC shall enable the converter power module if the following conditions are checked and fulfilled: - The 800V side DC link voltage is greater than 480V (HVDCDC_CAL); - The EDSU command NHVDCDCCommand == 0x1 "Enable" is received; - The converter power module is initialised and the communication is established; - No level 2 fault present. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | As per input/Process point (Check with Aayush) |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | 800V side DC Link Voltage > 480V (HVDCDC_CAL) AND NHVDCDCCommand == 0x1 "Enable" AND the power module is initialised and the communication is established AND NHVDCDCFault != 0x3 "Level 2" |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus = 0x02, NHVDCDCSTatusDebug = 0x02. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to enable the converter power module (PF1.0).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_04 |
| Customer Req ID | SDS-HVDCDC-130 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | *The HVDCDC shall enable the converter power module *The power module reports the 400V side output greater than a calibration threshold; |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NHVDCDCStatus = 0x2, NHVDCDCStatusDebug = 0x2 |
| Output(s) | enable the converter power module Enable the converter power module NHVDCDCStatus = 0x2 Enable power module NHVDCDCStatusDebug = 0x2 Enable power module |
| Process | The HVDCDC shall enable the converter power module if the following conditions are checked and fulfilled: - The 800V side DC link voltage is greater than 480V (HVDCDC_CAL); - The EDSU command NHVDCDCCommand == 0x1 "Enable" is received; - The converter power module is initialised and the communication is established; - No level 2 fault present; The HVDCDC shall transit to "Enable power module" state and set NHVDCDCStatus = 0x2 Enable power module. The HVDCDC shall set NHVDCDCStatusDebug = 0x2 Enable power module. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 2, NHVDCDCStatusDebug = 2. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | 800V side DC Link Voltage > 480V (HVDCDC_CAL) AND NHVDCDCCommand == 0x1 "Enable" AND the power module is initialised and the communication is established AND NHVDCDCFault != 0x3 "Level 2" |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus = 0x2 Enable power module NHVDCDCStatusDebug = 0x2 Enable power module |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_05 |
| Customer Req ID | SDS-HVDCDC-140 (Obsolete) |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall disable the converter power module. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NHVDCDCStatus = 0x4, NHVDCDCStatusDebug = 0x7 |
| Output(s) | Disable_power_module. Disable the converter power module NHVDCDCStatus = 0x4 Disable_power_module NHVDCDCStatusDebug = 0x7 Disable_power_module |
| Process | Upon the EDSU's request to shut down the DCDC while the HVDCDC is in "Normal" state, if the HVDCDC detects that the 800V side DC Link is being discharged (detection of main ESS contactors open), the HVDCDC shall disable the converter power module. The HVDCDC shall transit to "Disable" state. The HVDCDC shall set NHVDCDCStatus = 0x4 Disable_power_module. The HVDCDC shall set NHVDCDCStatusDebug = 0x7 Disable_power_module. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x01, NHVDCDCStatusDebug = 0x06. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | NA |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x0 "Standby" AND NHVDCDCStatus == 0x1 "Normal" AND No Level 2 fault present AND [d/dt(VHVDCDC800V) < dVHI_AD_threshold_C V/s] is TRUE within tShutdown_AD_Check_C ms |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus = 0x4 Disable_power_module NHVDCDCStatusDebug = 0x7 Disable_power_module |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to shut down the DCDC while the HVDCDC is in "Normal" state.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_06 |
| Customer Req ID | SDS-HVDCDC-141 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | ### Upon the EDSU's request to shutdown the DCDC while the HVDCDC is in "Normal" state, the HVDCDC shall disable the converter power module. The HVDCDC shall transit to "Disable" state. The HVDCDC shall set NHVDCDCStatus=0x4"Disable_power_module". The HVDCDC shall set NHVDCDCStatusDebug = 0x7 "Shutdown_DisablePM". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NHVDCDCStatus = 0x4, NHVDCDCStatusDebug = 0x7 |
| Output(s) | Disable_power_module. Disable the converter power module NHVDCDCStatus = 0x4 Disable_power_module NHVDCDCStatusDebug = 0x7 Disable_power_module |
| Process | Upon the EDSU's request to shutdown the DCDC while the HVDCDC is in "Normal" state, the HVDCDC shall disable the converter power module. The HVDCDC shall transit to "Disable" state. The HVDCDC shall set NHVDCDCStatus = 0x4 Disable_power_module. The HVDCDC shall set NHVDCDCStatusDebug = 0x7 Shutdown_DisablePM. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x01, NHVDCDCStatusDebug = 0x06. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x0 "Standby" AND NHVDCDCStatus == 0x1 "Normal" AND No Level 2 fault present |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus = 0x4 Disable_power_module. The HVDCDC shall set NHVDCDCStatusDebug = 0x7 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for a command to actively discharge the 400V DC link upon the power module.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_07 |
| Customer Req ID | SDS-HVDCDC-142 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall command to actively discharge the 400V DC link upon the power module status confirms as disabled from [141]. The HVDCDC shall transit to "Active Discharge 400V" state. The HVDCDC shall set NHVDCDCStatus = 0x5 "Discharge 400V". The HVDCDC shall set NHVDCDCStatusDebug = 0x8 "Shutdown_EnableAD400V". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NHVDCDCStatus = 0x5 Discharge 400V. |
| Output(s) | power module status confirms as disabled from Command to actively discharge 400V side DC Link; NHVDCDCStatus = 0x5 "Discharge 400V" NHVDCDCStatusDebug = 0x8 "Shutdown_AD400V" |
| Process | The HVDCDC shall command to actively discharge the 400V DC link upon the power module status confirms as disabled from [141]. The HVDCDC shall transit to "Active Discharge 400V" state. The HVDCDC shall set NHVDCDCStatus = 0x5 Discharge 400V. The HVDCDC shall set NHVDCDCStatusDebug = 0x8 Shutdown_EnableAD400V. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x05, NHVDCDCStatusDebug = 0x0A. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCStatus = 0x4 "Disable_power_module" [141] AND the power module status confirms as disabled [141] AND No Level 2 fault present |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus = 0x5 "Discharge 400V" NHVDCDCStatusDebug = 0x8 "Shutdown_AD400V" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for a command to disable the power module.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_10 |
| Customer Req ID | SDS-HVDCDC-153 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall command to disable the power module after the status of the 400V active discharge switch confirms to be open from or a calibratable period of time has passed since . The HVDCDC shall set NHVDCDCStatusDebug = 0xE "Faulty_DisablePM". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | NHVDCDCStatusDebug = 0xE "Faulty_DisablePM" |
| Output(s) | Disable_power_module. Disable the converter power module; NHVDCDCStatusDebug = 0xE "Faulty_DisablePM" |
| Process | The HVDCDC shall command to disable the power module after the status of the 400V active discharge switch confirms to be open from [152] or a calibratable period of time has passed since [152]. The HVDCDC shall set NHVDCDCStatusDebug = 0xE "Faulty_DisablePM". |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x06, NHVDCDCStatusDebug = 0x0D. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | the status of the 400V active discharge switch confirms to be open [152] OR HVDCDC_CAL sec has passed since [152] |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatusDebug = 0xE "Faulty_DisablePM" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for a command for the power module to clear its internal fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_12 |
| Customer Req ID | SDS-HVDCDC-162 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | If the power module is in faulty status , the HVDCDC shall command the power module to clear its internal fault. The HVDCDC shall set NHVDCDCStatusDebug = 0xF "FaultyUnlatch_ClearPM". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | HVDCDC |
| Output(s) | NHVDCDCStatusDebug = 0xF "FaultyUnlatch_ClearPM". Command the converter power module to clear fault NHVDCDCStatusDebug = 0x0F "FaultyUnlatch_ClearPM" |
| Process | If the power module is in faulty status [161], the HVDCDC shall command the power module to clear its internal fault. The HVDCDC shall set NHVDCDCStatusDebug = 0xF "FaultyUnlatch_ClearPM". |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHDCDCStatus = 0x06. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NFaultUnlatchRequestHVDCDC == 0x1 AND Fault unlatching pre-conditions [160] are met AND the power module is in faulty status [161] |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatusDebug = 0x0F "FaultyUnlatch_ClearPM" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement for a command to unlatch the HVDCDC level 2 faults.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_13 |
| Customer Req ID | SDS-HVDCDC-163 |
| Milestone | B- Sample/PF2.0 |
| Purpose1 | If the power module is not in faulty status [161], or the power module fault clearing [162] is successful within a calibratable period time, then the HVDCDC shall command to unlatch the HVDCDC level 2 faults. The HVDCDC shall set NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys". |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | HVDCDC shall set NHVDCDCStatusDebug = 0x10 |
| Output(s) | unlatch the HVDCDC level 2 faults. Command to unlatch the main controller faults NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys" |
| Process | The HVDCDC shall command the power module to clear its internal fault. HVDCDC shall set NHVDCDCStatusDebug = 0x10 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x06. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NFaultUnlatchRequestHVDCDC == 0x1 AND Fault unlatching pre-conditions [160] are met AND (the power module is not in faulty status [161] OR the power module fault clearing is successful [162] ) |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatusDebug = 0x10 "FaultyUnlatch_ClearSys" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to raise a Level 0 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_15 |
| Customer Req ID | SDS-HVDCDC-189 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | If the HVDCDC is in normal (buck/boost) operation with HV power applied and it detects the power module I2C communication is compromised for a calibratable time, the HVDCDC shall raise a Level 0 fault. . The HVDCDC shall latch the last valid values received on these messages/signals until the communication integrity is recovered. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | HVDCDC shall raise a Level 0 fault. After applying HV power. |
| Output(s) | NHVDCDCFault = 0x1 "Level 0" Set DTC |
| Process | The HVDCDC shall latch the last valid values received on these messages/signals until the communication integrity is recovered. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCFault = 0x01 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCStatus == 0x1 "Normal" AND abs(IHVDCDC400V) > HVDCDC_CAL AND the power module I2C communication integrity is compromised for time > HVDCDC_CAL |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault = 0x01 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to raise a level 2 fault and command to actively discharge the power module.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_17 |
| Customer Req ID | SDS-HVDCDC-203 |
| Milestone | A-Sample/PF1.0 |
| Purpose1 | The HVDCDC shall raise a level 2 fault and command to actively discharge the power module 400V side when it detects the 400V side DC link or output voltage is higher than half of the 800V side DC link voltage AND the power module is disabled AND the 800V side is under load. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Set the 800V side (high side) voltage of the power module to greater than 480V(e.g. 540V) 2. The power module status should be disabled 3. The HVDCDC application should be in Standby mode |
| Output(s) | Below CAN messages are observed, 1. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x4 ("Disable_power_module") 2. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x7 ("Shutdown_DisablePM"). 3. NHVDCDCFaultyShutdown == 0x2 "Emergency_FaultyShutdown" 4. The level 2 fault should be raised in All States except Sleep state and Power down Safe state |
| Process | Verify that The HVDCDC shall raise a level 2 fault and discharge the power module when the below conditions are met, 1. VICOR output voltage (VHVDCDC400V_VICOR) is more than half of the high side voltage (VHVDCDC800V_VICOR) plus 10V(HVDCDC_CAL) 2. The power module status is disabled. 3. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x4 ("Disable_power_module") 4. NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x7 ("Shutdown_DisablePM"). 5. NHVDCDCFaultyShutdown == 0x2 "Emergency_FaultyShutdown" |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application shall in Normal substate of HV_ON state 2. NHVDCDCStatusDebug @ HVDCDC_Debug shall equal to 0x6 ("Normal" 3. No fault shall raise, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | NHVDCDCCommand == 0x1 "Enable" AND the HVDCDC commanded the enabling of the power module AND the power module is not enabled successfully for time > HVDCDC_CAL |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x4 ("Disable_power_module") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x7 ("Shutdown_DisablePM"). NHVDCDCFaultyShutdown == 0x2 "Emergency_FaultyShutdown" |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to turn ON and turn OFF the Power board 5VV supply via PMIC control.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_PMCID_18 |
| Customer Req ID | SDS-HVDCDC-287 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall be able to turn ON and turn OFF the Power board 5VV supply via PMIC control. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Send CAN signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd message as 0x02 ("Safe State") via CAN interface to HVDCDC application. 2. Once the HV DCDC application is in the "Disable_3" substate of the "Faulty" state within 10 seconds, decrease VICOR output voltage(VHVDCDC400V_VICOR) to less than half of VICOR input voltage(high side) minus 15V. 3. Check the fault response time by checking the HVDCDC applications transition time from the "Disable_400V_AD_3" substate of the "Discharge" state to the time when the level 2 fault is raised. |
| Output(s) | At Step 4: NHVDCDCStatus @ HVDCDC_Status1 is equal to 0x6 (“Faulty") NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0xD ("Faulty_Disable400VAD") NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 (“level 2") NHVDCDCFaultID @ HVDCDC_Debug_PDU is equal to 0x23(35) after the fault response time of 200ms. NHVDCDCStatus @ HVDCDC_Status1 is set to 0x7 ("PowerDown_Safe_State_NHVDCDCStatus_NW") NHVDCDCStatusDebug @ HVDCDC_Debug is set to 0x11 ("PowerDown_Safe_State") The signals IHVDCDC400V @ HVDCDC_Status2, IHVDCDC800V @ HVDCDC_Status2, VHVDCDC400V @ HVDCDC_Status2 and VHVDCDC800V @ HVDCDC_Status2 shows last received value from the power board, values are not getting updated. |
| Process | ON power board connect TP412 to 3.3V |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0x01, NHVDCDCStatusDebug = 0x06, NHVDCDCFault = 0x00. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values are not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## Specific OBD Requirements

### Requirements

#### Diagnostic Addresses

##### Requirement to support DTC Range according to SAE SJA2012A Document.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_01 |
| Customer Req ID | SDS18-DIA01-02-3448 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall support DTC Range according to the DTC’s provided in SAE SJA2012A document. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | -The DTC Range for HVDCDC shall be as per SAE document. |
| Output(s) | -The HVDCDC shall use the DTC range from P1E01 to P1E6F to be report to primary controller. |
| Process | - HVDCDC shall list all the DTC to be sent to primary controller. - HVDCDC shall assign IDs from range P1E01 to P1E6F to all the DTC’s. - HVDCDC shall continuously report all the logged DTC ID’s to primary controller. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No DTC shall be an initial preloaded value |
| Default Values | Default value is not required |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the DTC reported by HVDCDC to the Primary controller shall be as per the range P1E01 to P1E6F |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to support DTC Range according to the MSL.01.04.0047 Diagnostic Network ID.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_36 |
| Customer Req ID | SDS18-DIA01-02-3448 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall support DTC Range according to the MSL.01.04.0047 Diagnostic Network ID Design Guide. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The DTC is to be raised is not listed in SAE document -The DTC Range for HVDCDC shall be as per MSL.01.04.0047 design guide. |
| Output(s) | -The HVDCDC shall use the DTC range from P1E00xx to P1EFFxx to be report to primary controller. |
| Process | - HVDCDC shall list all the DTC’s which are not listed in SAE document and to be sent to primary controller. - HVDCDC shall assign IDs from range P1E00xx to P1EFFxx to all the non-listed DTC’s. - HVDCDC shall continuously report all the logged DTC ID’s to primary controller. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No DTC shall be a initial preloaded value |
| Default Values | Default value is not required |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the DTC reported by HVDCDC to the Primary controller shall be as per the range P1E00xx to P1EFFxx. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to support the Network DTC’s according to the MSL.01.04.0047 Diagnostic Network ID.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_37 |
| Customer Req ID | SDS18-DIA01-02-3448 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall support the Network DTC’s according to the MSL.01.04.0047 Diagnostic Network ID Design Guide. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Provide the CAN Communication Timeout fault by taking HVDCDC into bus-off conditions or excessive errors on the CAN bus so that acknowledge does not receive. - Provide invalid data or missing message byte/bit which results in incorrect CRC. This will generate Invalid Safety Data check DTC. - <Need to add for Invalid Safety check> - Provide incorrect data length in message with respect to number of bytes transmitted over the message frame. -The HVDCDC shall use the Network DTC’s as per MSL.01.04.0047 design guide. |
| Output(s) | -The HVDCDC shall raise below network DTC’s coming from DCU ZC1 1. CAN Communication Timeout (U180087) 2. Invalid Safety Data Check (U180183) 3. Invalid Safety Counter Check (U180182) 4. Invalid Data Length (U18018F) |
| Process | - HVDCDC shall list all the network DTC’s to be sent to primary controller. - The HVDCDC shall detect the network DTC’s mentioned in output - The HVDCDC shall report all network DTC’s mentioned in output - Store all the Network DTC’s into NVM |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | The Fault detection mechanism should be implemented in HVDCDC |
| Default Values | Default value is not required |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the Network DTC’s shall be reported by HVDCDC to the Primary controller shall be as per the range P1E00xx to P1EFFxx. Verify that the HVDCDC shall Send DTC over CAN. Verify that the Status should return after successful store in NVM. Verify that the DTC’s store in NVM after power on reset. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Dependent Secondary OBD modules and smart devices.

##### Requirement to maintain the status as “Test Currently Active”.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_02 |
| Customer Req ID | SDS18-DIA01-02-2886 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall be able to maintain the status as “Test Currently Active” for each diagnostics monitor test. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the fault for the respective functionality in HVDCDC 2. The HVDCDC shall get trigger to start diagnostics test or monitoring function |
| Output(s) | 1. The HVDCDC shall detect the faults or issues in respective functionality 2. HVDCDC shall report the status of diagnostics test that are currently active in operation or has been recently triggered to the primary controller. |
| Process | 1. HVDCDC shall receive a fault in particular functionality. 2. HVDCDC shall detect the fault in this functionality. 3. The HVDCDC shall trigger to start the diagnostics monitor test functionality. 4. The HVDCDC shall start the diagnostics monitor test functionality. 5. HVDCDC shall continuously report the diagnostics test results to primary controller |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. |
| Default Values | Default value is not required |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the status of diagnostics tests that are currently active in operation or have been recently triggered to the primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to maintain the status as “Test Currently Failing”.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_03 |
| Customer Req ID | SDS18-DIA01-02-2886 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall be able to maintain the status as “Test Currently Failing” for each diagnostics monitor test. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the fault for respective functionality in HVDCDC 2. Fail the diagnostic monitor test by providing issue at application. wrong threshold and debounce 3. Refer SAE document to add application that causes diagnostics monitor test failing. |
| Output(s) | 1. The HVDCDC shall detect issues or malfunction in the diagnostics monitor test 2. HVDCDC shall report the status of diagnostics monitor as “test currently failed” to primary controller. 3. HVDCDC shall report last test results when test currently failed detected |
| Process | 1. HVDCDC shall receive a fault in particular functionality. 2. HVDCDC shall detect the fault in this functionality. 3. The HVDCDC shall trigger to start the diagnostics monitor test functionality. 4. The HVDCDC shall start the diagnostics monitor test functionality but failed due to issue at application. 5. HVDCDC shall continuously report the diagnostics test that are failed to primary controller |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default value is not required |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the status of diagnostics monitor as “test currently failed” to primary controller. Verify that HVDCDC shall report Malfunction in diagnostics test to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the battery short circuit.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_09 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the short circuit to high failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Do the changes required on the power board to provide the Short to high failure. - Create a CAN frame format to provide the DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Short to High Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. - HVDCDC shall continuously report Short to high DTC information for respective fault using <Add here > CAN frame to primary controller. - The DTC for respective short to high fault shall be in range as per SAE document. |
| Process | - HVDCDC shall receive a Short to High fault in various (800V_EMI, 400V_EMI, 800V_VICOR_IND, 800V_VICOR, 400V_VICOR, 800V_SHUNT, 400V_SHUNT, Precharge, Discharge) sections/terminals of power module. - HVDCDC shall detect the Short to High fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Short to High fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Short to High fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the short to High fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the short circuit to ground.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_10 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the short circuit to ground failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Do the changes required on the power board to provide the Short to ground failure. - Create a CAN frame format to provide the DTC information to primary controller. |
| Output(s) | 1. HVDCDC shall raise the Short to Ground Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. 2. HVDCDC shall continuously report Short to ground DTC information for respective fault using <Add here > CAN frame to primary controller. 3. The DTC for respective short to ground fault shall be in range of P1E00xx to P1EFFxx. |
| Process | HVDCDC shall receive a Short to Ground fault in various (800V_EMI, 400V_EMI, 800V_VICOR_IND, 800V_VICOR, 400V_VICOR, 800V_SHUNT, 400V_SHUNT, Precharge, Discharge) sections/terminals of power module. - HVDCDC shall detect the Short to Ground fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Short to Ground fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Short to Ground fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the short to Ground fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the system Open Circuit Failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_32 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the Open Circuit failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Do the changes required on the power board to provide the Open Circuit failure. - Create a CAN frame format to provide the DTC information to primary controller. |
| Output(s) | 1. HVDCDC shall raise the Open Circuit Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. 2. HVDCDC shall continuously report Open Circuit DTC information for respective fault using <Add here > CAN frame to primary controller. 3. The DTC for respective Open Circuit fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a Open Circuit fault in various (800V_EMI, 400V_EMI, 800V_VICOR_IND, 800V_VICOR, 400V_VICOR, 800V_SHUNT, 400V_SHUNT, Precharge, Discharge) sections/terminals of power module. - HVDCDC shall detect the Open Circuit fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Open Circuit fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Open Circuit fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Open Circuit fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

##### Requirement for monitoring the system over voltage.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_11 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the over voltage failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The respective side of HVDCDC power board shall receive voltage greater than Threshold voltage HVDCDC_CAL. - Create a CAN frame format to provide the Over voltage DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Over Voltage Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. - HVDCDC shall continuously report Over Voltage DTC information for respective fault using <Add here > CAN frame to primary controller. The DTC for respective Over Voltage fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a Over Voltage fault in various (800V, 400V, VICOR_5VS, LOCAL_5VS, PM_I2C) sections/terminals of power module. - HVDCDC shall detect the Over Voltage fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Over Voltage fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Over Voltage fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0 |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Over Voltage fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the system under voltage fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_12 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the under-voltage failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The respective side of HVDCDC power board shall receive voltage less than Threshold voltage HVDCDC_CAL. - Create a CAN frame format to provide the under-voltage DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Under Voltage Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. - HVDCDC shall continuously report Under Voltage DTC information for respective fault using <Add here > CAN frame to primary controller. -The DTC for respective under Voltage fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive an under-Voltage fault in various (800V, 400V, VICOR_5VS, LOCAL_5VS, PM_I2C) sections/terminals of power module. - HVDCDC shall detect the under-Voltage fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the under-Voltage fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the under-Voltage fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Under Voltage fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the system over current.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_13 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the over current failure to primary controller. The HVHVDCDC shall provide a DTC for monitoring the system over current. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The respective side of HVDCDC power board shall receive current greater than Threshold current value HVDCDC_CAL. - Create a CAN frame format to provide the Over current DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Over Current Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of HVDCDC_CAL. - HVDCDC shall continuously report Over Current DTC information for respective fault using <Add here > CAN frame to primary controller. The DTC for respective Over Current fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive an Over Current fault in various (800V, 400V) sections/terminals of power module. - HVDCDC shall detect the Over Current fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Over Current fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Over Current fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Over Current fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the voltage sensor stuck.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_14 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor faults and provide DTC for the voltage sensor stuck failure to primary controller. The HVHVDCDC shall provide a DTC for monitoring the voltage sensor stuck. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The External ADC of HVDCDC power board & MCU shall not receive proper voltage from the voltage sensors available at various sections/terminals of power board. - Create a CAN frame format to provide the voltage sensor stuck DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Voltage Sensor Stuck Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of 100msec. - HVDCDC shall continuously report Voltage Sensor Stuck DTC information for respective fault using <Add here > CAN frame to primary controller. The DTC for respective Voltage Sensor Stuck fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a Voltage Sensor Stuck in various (800V_EMI, 400V_EMI, 800V_VICOR_IND, 800V_VICOR, 400V_VICOR, 800V_SHUNT, 400V_SHUNT,) sections/terminals of power module. - HVDCDC shall detect the Voltage Sensor Stuck fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Voltage Sensor Stuck fault DTC over CAN within the HVDCDC_CAL - HVDCDC shall continuously report the Voltage Sensor Stuck fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Voltage Sensor Stuck fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the current sensor stuck.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_15 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the current sensor stuck failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The External ADC of HVDCDC power board shall not receive proper current from the current sensors available at various sections/terminals of power board. - Create a CAN frame format to provide the DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the current sensor stuck Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of 100msec. - HVDCDC shall continuously report current sensor stuck DTC information for respective fault using <Add here > CAN frame to primary controller. The DTC for respective current sensor stuck fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a current sensor stuck fault in various (800V, 400V) sections/terminals of power module. - HVDCDC shall detect the current sensor stuck fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the current sensor stuck fault DTC over CAN within the HVDCDC_CAL -HVDCDC shall continuously report the current sensor stuck fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. NA |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the current sensor stuck fault DTC across respective sections/terminals of power module to primary controller. CANape or CAN BUS Analyzer is a software tool for developing and debugging high-speed CAN networks |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the temperature sensor stuck.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_16 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the temperature sensor stuck failure to primary controller. The HVHVDCDC shall provide a DTC for monitoring the temperature sensor stuck. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The External ADC of HVDCDC power board shall not receive proper temperature from the temperature sensors available at various sections/terminals of power board. -Create a CAN frame format to provide the temperature sensor stuck DTC information to primary controller. - Document that contains the OBD CAN Frames information. - Document that contains the DTC information for temperature sensor stuck fault. |
| Output(s) | - HVDCDC shall raise the temperature sensor stuck Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of 100msec. - HVDCDC shall continuously report temperature sensor stuck DTC information for respective fault using <Add here > CAN frame to primary controller. The DTC for respective temperature sensor stuck fault shall be in range of P1E00xx to P1EFFxx. Upon requesting the DTC through CANalyzer, it will report the status of temperature sensor stuck fault. |
| Process | - HVDCDC shall receive a temperature sensor stuck fault in various (800V, 400V, MCU, Vicor, SSS, PM_I2C) sections/terminals of power module. - HVDCDC shall detect the temperature sensor stuck fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the temperature sensor stuck fault DTC over CAN within the HVDCDC_CAL -HVDCDC shall continuously report the temperature sensor stuck fault DTC to primary controller. - Establish the CAN communication with CAN analyzer. - Force a temperature sensor stuck or inject temperature sensor stuck DTC value to indicate the temperature sensor stuck fault. - Request the value of temperature sensor stuck DTC. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. NA |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the temperature sensor stuck fault DTC across respective sections/terminals of power module to primary controller. CANape or CAN BUS Analyzer is a software tool for developing and debugging high-speed CAN networks |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement for monitoring the Solid State Switch stuck.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_33 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the Solid state switch stuck failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - The solid-state switch status (SSS_Status) shall not change open/close with respect to the provided Solid State Switch Enable (SSS_EN) status Open/close within the calibratable time HVDCDC_CAL. -Create a CAN frame format to provide the solid-state switch stuck DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the solid-state switch stuck Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of 100usec. - HVDCDC shall continuously report solid-state switch stuck DTC information for respective fault using <Add here > CAN frame to primary controller. -The DTC for respective solid-state switch stuck fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a solid-state switch stuck fault either in open or closed state. - HVDCDC shall detect the solid-state switch stuck fault either in open or closed state within the HVDCDC_CAL time. - HVDCDC shall send the solid-state switch stuck fault DTC over CAN within the HVDCDC_CAL -HVDCDC shall continuously report the solid-state switch stuck fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the solid-state switch stuck fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

##### Requirement for monitoring the Plausibility Check Failure.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_34 |
| Customer Req ID | SDS18-DIA01-02-3165 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall implement to monitor fault and provide DTC for the Plausibility check failure to primary controller. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific restrictions is applicable. |
| Input(s) | - The HVDCDC shall get the invalid range value for various voltage terminals (800V_EMI, 800V_Vicor, 400V_EMI) within the calibratable time HVDCDC_CAL. -Create a CAN frame format to provide the Plausibility check failure DTC information to primary controller. |
| Output(s) | - HVDCDC shall raise the Plausibility check Failure and the respective fault ID should be provided in NHVDCDCFaultID@ HVDCDC_Debug_PDU signal after the fault response time of 100usec. - HVDCDC shall continuously report Plausibility check DTC information for respective fault using <Add here > CAN frame to primary controller. -The DTC for respective Plausibility check fault shall be in range of P1E00xx to P1EFFxx. |
| Process | - HVDCDC shall receive a Plausibility check fault in various (800V_EMI, 400V_EMI, 800V_Vicor) sections/terminals of power module. - HVDCDC shall detect the Plausibility check fault in the respective sections/terminals of power module within the HVDCDC_CAL time. - HVDCDC shall send the Plausibility check fault DTC over CAN within the HVDCDC_CAL -HVDCDC shall continuously report the Plausibility check fault DTC to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No fault should be logged and reported to primary controller. NHVDCDCStatus = 0, NHVDCDCStausDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall continuously report the Plausibility check fault DTC across respective sections/terminals of power module to primary controller. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

##### Requirement for to submit all ECU specific certificate document as per CARB 1968.2 specification.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_35 |
| Customer Req ID | SDS18-DIA01-02-2899 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall submit all ECU specific certification documentation as per section I of the CARB 1968.2 specification. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values |  |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

#### Calibration ID & Calibration Verification Number

##### Requirement to maintain a unique 16 bytes (calibration ID).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_17 |
| Customer Req ID | SDS18-DIA01-02-2904 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | - The HVHVDCDC shall use 16 bytes calibration ID for each software release. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - CAL ID shall be of 16 Byte value. - CAL ID shall be in ASCII Format. |
| Output(s) | - The HVDCDC shall provide 16 Byte CAL ID to primary controller in the mentioned ASCII format. |
| Process | - HVDCDC shall set 16 Byte CAL ID with default value. - HVDCDC shall validate 16 Byte CAL ID for correct ASCII format. - HVDCDC shall report 16 Byte CAL ID to primary controller in the mentioned ASCII format. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | The 16 Byte CAL ID shall be predefined and used with value “xxxxxxxxxxxxxxxx“ |
| Default Values | Default value is as per preloaded value |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC shall maintain 16 Byte CAL ID with defined ASCII format. |
| Constraints | The CAL ID should be in ASCII format |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance criteria is applicable as per section 6. Acceptance Criteria |

##### Requirement to use incremental calibration ID for multiple software releases.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_18 |
| Customer Req ID | SDS18-DIA01-02-2904 |
| Milestone | C SAMPLE/PF3.0 Phase-2 |
| Purpose1 | - The HVDCDC shall use incremental calibration ID for multiple software releases. |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - CAL ID shall be of 16 Byte value. - CAL ID shall be in ASCII Format. |
| Output(s) | - HVDCDC shall use the incremental 16-byte ASCII CAL ID value as the previous software release. |
| Process | - HVDCDC shall set 16 Byte CAL ID with default value. - HVDCDC shall increment last byte of CAL ID with respect to previous release. - HVDCDC shall report incremental 16 Byte CAL ID value to primary controller. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | The 16 Byte CAL ID shall be preloaded with default value as “xxxxxxxxxxxxxxxx” |
| Default Values | Default value is as per preloaded value |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall send the incremental 16 byte ASCII CAL ID value w.r.to the previous software release to primary controller continuously. |
| Constraints | Incremental 16 Byte ASCII format for CAL ID |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance criteria is applicable as per section 6. Acceptance Criteria |

##### Requirement to store calibration ID used for the software release.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_19 |
| Customer Req ID | SDS18-DIA01-02-2904 |
| Milestone | C SAMPLE/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall store calibration ID used for the software release in D-FLASH |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | High |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - CAL ID shall be of 16 Byte value. - CAL ID shall be in ASCII Format. |
| Output(s) | - HVDCDC shall save 16 Byte CAL ID into D-FLASH - HVDCDC shall return status as WRITE_SUCCESSFUL |
| Process | - HVDCDC shall set 16 Byte CAL ID with default value. - HVDCDC shall increment last byte of CAL ID with respect to previous release. - HVDCDC shall store 16 Byte CAL ID into D-FLASH. - HVDCDC shall return status as WRITE_SUCCESSFUL |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | The 16 Byte CAL ID shall be preloaded with default value as “xxxxxxxxxxxxxxxx” |
| Default Values | Default value is as per preloaded value |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC shall successfully complete the D-Flash write operation to for storing 16 byte ASCII CAL ID. |
| Constraints | 16 Byte ASCII format for CAL ID |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance criteria is applicable as per section 6. Acceptance Criteria |

##### Requirement to calculate and store the 4 bytes CVN (Calibration Verification Number).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_20 |
| Customer Req ID | SDS18-DIA01-02-2933, SDS18-DIA01-02-2942 |
| Milestone | C SAMPLE/PF3.0 Phase -2 |
| Purpose1 | - The HVDCDC shall calculate & Store the 4 Byte CVN (Calibration Verification Number) with in 120s of ECU power ON. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Calibration Verification Number (CVN) shall be of 4 byte. - CVN shall be read from D-Flash on ECU power ON. - CVN timeout parameter of 120 sec |
| Output(s) | - Upon ECU startup, HVDCDC shall calculate the corresponding CVN value for the Calibration within 120sec of power on. - HVDCDC shall verify the calculated CVN with the CVN number stored in D-Flash. - HVDCDC shall report CVN to primary controller. |
| Process | - Provide power on cycle to HVDCDC - HVDCDC shall calculate the 4 Byte CVN with in 120sec - HVDCDC shall read the CVN stored in NVM - HVDCDC shall validate the 4 Byte CVN with Stored value of CVN - HVDCDC shall update the respective CVN parameter - HVDCDC shall report CVN to primary controller |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | The 4 Byte CVN shall be preloaded with default value as “xxxx” |
| Default Values | Default value is same as preloaded value |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the time required for HVDCDC to calculate the corresponding CVN value for the Calibration is less than or equal to 120sec. Verify that the 4-byte CVN shall be maintained by HVDCDC in the specified format. Verify that the HVDCDC shall report verified CVN to primary controller continuously. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report CAL ID bytes (1-7).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_21 |
| Customer Req ID | SDS18-DIA01-02-3028, SDS18-DIA01-02-3026 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall report CAL ID bytes (1-7) if the mux value is equal to zero. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Read CAL ID from NVM (To Be confirm with MAL) - Mux value should be Zero |
| Output(s) | Transmit “DCDC_CALID_CVN” PDU with bytes 1 to 7 periodically. (To be confirm with MAL - if need to send sequentially as per mux value) |
| Process | - Read the CALIB ID from Dflash. - Set the value of mux to zero. 3- DCDC shall send CAL ID bytes (1-7). |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded is same as CAL ID stored in D-Flash. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that “DCDC_CALID_CVN” byte position from 1 to 7 are received on CANanalyzer. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report CAL ID bytes (8-14).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_22 |
| Customer Req ID | SDS18-DIA01-02-3029, SDS18-DIA01-02-3026 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall report CAL ID bytes (8-14) if the mux value is equal to one. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Mux value should be One |
| Output(s) | Transmit “DCDC_CALID_CVN” PDU with bytes 8 to 14 periodically |
| Process | - Read the CALIB ID from Dflash. - Set the value of mux to one. - DCDC shall send CAL ID bytes (8-14). |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded is same as CAL ID stored in Dflash |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that “DCDC_CALID_CVN” byte position from 8 to 14 are received on CANanalyzer. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report CAL ID bytes (15-16).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_23 |
| Customer Req ID | SDS18-DIA01-02-3170, SDS18-DIA01-02-3026 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall report CAL ID bytes (15-16) if the mux value is equal to two. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Mux value should be Two |
| Output(s) | Transmit “DCDC_CALID_CVN” PDU with bytes 15 to 16 periodically |
| Process | - Read the CALIB ID from Dflash. - Set the value of mux to two. - DCDC shall send CAL ID bytes (15-16). |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded is same as CAL ID stored in Dflash |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that “DCDC_CALIB_CVN” byte position from 15 and 16 are received on CANanalyzer. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report Calibration Verification Number (CVN) if the mux value is equal to three.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_24 |
| Customer Req ID | SDS18-DIA01-02-3030, SDS18-DIA01-02-3026 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall report Calibration Verification Number (CVN) if the mux value is equal to three. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | - Read CVN from NVM (To Be confirm with MAL) - Mux value should be Three |
| Output(s) | Transmit “DCDC_CALIB_CVN” PDU |
| Process | - Read the CVN from Dflash. - Set the value of mux to three. - DCDC shall send CVN PDU. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded is same as CVN stored in Dflash |
| Default Values | Default value is same as Pre-Loaded value |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that “DCDC_CALIB_CVN” PDU is received on CANanalyzer. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to report the previously stored CVN when requested via diagnostics for up to 120 seconds.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_25 |
| Customer Req ID | ##### SDS18-DIA01-02-2951 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVDCDC shall report the previously stored Calibration Verification Number (CVN) when requested via diagnostics for up to 120 seconds from ECU initialization until the CVN is recalculated. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Diagnostics request to read the store CVN value |
| Output(s) | - Transmit the previous stored value in “DCDC_CALIB_CVN” PDU for 120 seconds after Power ON - Transmit the Calculated CVN value in “DCDC_CALIB_CVN” PDU after 120 seconds |
| Process | - Initialize the ECU. - Start the CVN_TX_TIMER for timeout - Check if CVN_TX_TIMER is less than or equal to 120s. - Read the CVN from Dflash. - HVDCDC shall send CVN value in “DCDC_CALIB_CVN” PDU when request is received from diagnostic - Check if CVN_TX_TIMER is greater than 120s. - HVDCDC shall send calculated CVN value in “DCDC_CALIB_CVN” PDU |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded value is CVN stored in Dflash |
| Default Values | Default value is same as Pre-Loaded value |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that “DCDC_CALIB_CVN” PDU is received on CANanalyzer within 120s from start up. |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

##### Requirement to report the default CVN with value $00 when requested via diagnostics for up to 120 seconds.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_26 |
| Customer Req ID | ##### SDS18-DIA01-02-2960 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The DCDC shall return the default Calibration Verification Number (CVN) with value $00 when requested via diagnostics within the first 120 seconds after the ECU has been reprogrammed and the new CVN is calculated. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Reprogram the ECU 2. Calculate the CVN value with in first 120 Seconds. |
| Output(s) | - Transmit CVN values as “$00” in the “DCDC_CALIB_CVN” PDU for first 120 seconds after power on - Send calculated CVN after 120 seconds of power ON. |
| Process | - Reprogram the ECU - Set the “DCDC_CALIB_CVN” PDU value to $00 - HVDCDC shall send “DCDC_CALIB_CVN” PDU within 120s from reprogramming - Send calculated CVN after 120 seconds of power ON. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | Pre-Loaded is CVN stored in Dflash |
| Default Values | Default value is same as Pre-Loaded value |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the CVN values shall be “00” in “DCDC_CALIB_CVN” PDU received on CANanalyzer within 120s from ECU reprogramming |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |

#### Diagnostic Services

##### Requirement to renew its OBD diagnosis results.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_28 |
| Customer Req ID | SDS18-DIA01-02-3093 |
| Milestone | C SAMPLE/PF3.0 |
| Purpose1 | The HVHVDCDC shall renew its OBD diagnosis results in case ZC1_ObdMode4_Request CAN signal is 0x01. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | ZC1_ObdMode4_Request CAN signal received by 0x01 value. |
| Output(s) | DCDC shall refresh (renew) the OBD diagnosis results |
| Process | - Establish the CAN communication. - Set ZC1_ObdMode4_Request CAN signal with 0x01 through canalyzer. - All OBD diagnosis results shall be erased from NVM. - Recalculate and Renew all OBD diagnosis result. - Store OBD diagnosis result into NVM. - Send OBD diagnosis result over CAN |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default value is required |
| Valid range of Values | Valid range value is required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is the key cycle. |
| Data Rate/ <Daily> Number of transactions | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1- Recalculate and Renew all OBD diagnosis result. 2- Store OBD diagnosis result into NVM. 3. Send OBD diagnosis result over CAN |
| Constraints | Constraints are not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Fault Management.

#### Requirement to configure as a secondary OBD II device.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_30 |
| Customer Req ID | SDS-HVDCDC-249, SDS18-DIA01-02-2886 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall be configure as secondary OBD II device |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the various faults like Short To Ground, Short To High, Open Circuit, Over Temp, Over current, Over Voltage, Under Voltage, Sensor Stuck, Implausible input to HVDCDC 2. Calibration ID and CVN shall be provided to HVDCDC |
| Output(s) | 1. The HVDCDC shall send DTC for each fault type (Short To Ground, Short To High, Open Circuit, Over Temp, Over current, Over Voltage, Under Voltage, Sensor Stuck, Implausible input) over vehicle CAN. 2. The HVDCDC shall store DTC for each fault into NVM 3. The HVDCDC shall continuously report all the diagnostics fault, CALID and CVN to primary controller 4. HVDCDC shall store Calibration ID and CVN into NVM |
| Process | 1. Provide the fault (Short To Ground, Short To High, Open Circuit, Over Temp, Over current, Over Voltage, Under Voltage, Sensor Stuck, Implausible input) one by one on HVDCDC Hardware 2. HVDCDC shall detect the fault within the calibratable time 3. HVDCDC shall send the fault DTC over CAN within the calibratable time 4. The HVDCDC shall store DTC for each fault into NVM 5. provide the CALID and CVN to HVDCDC 6. HVDCDC shall store CALID and CVN into NVM 7. The HVDCDC shall continuously report all the diagnostics fault, CALID and CVN to primary controller |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Calibration ID and CVN shall be define |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1. Verify that the HVDCDC shall continuously report all the diagnostics fault, CALID and CVN to primary controller 2. The HVDCDC shall send DTC for each fault type over vehicle CAN |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Requirement to implement the specific requirements for the OBD II device.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_OBD_31 |
| Customer Req ID | SDS-HVDCDC-358 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC shall implement the specific requirements for the OBD II device stated in [SDS18-DIA01-02_DCDC] |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input as expected to a specific requirement mention in SDS18-DIA01-02_DCDC document |
| Output(s) | Output as expected to a specific requirement mention in SDS18-DIA01-02_DCDC document |
| Process | Process shall be as per specific Software requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific Software requirement. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Non-MIL/OBD Components document.

- A justification must be supplied for every component identified as non-OBD relevant.

- However, for HVDCDC no such a Non-MIL/OBD components observed. (Need confirmation from MAL).

#### General Description document.

##### Hybrid Architecture Overview.

- High Voltage DC to DC Converter: The HV HVDCDC performs the following functions which are necessary for ensuring the integrity of operation of the Hybrid Vehicle:

- 400V Power Conversion (Buck) – The HV DCDC shall convert electrical power from the 800V HV bus to the 400V HV bus. The HV DCDC shall supply power to the 400V bus in form of electrical current and maintain the voltage at the 400V bus to sustain the loads on 12V HVDCDC, PTC and eAC.

- 800V Power Conversion (Boost) – The DCDC shall convert electrical power from the 400V HV bus to the 800V ESS HV bus during charging. The DCDC shall control voltage and current on the 800V bus during charging.

- Control Topology – The HV DCDC shall respond to commands from the Electric Drive Supervisory Unit for all supported functions.

- HVDCDC shall report all the faults and status messages over CAN.

##### System voltage high – 800V side Diagnostic text Descriptions.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a text description must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Description:

System voltage high – 800V side

Associated DTC: <TBD>

McLaren Identifier: <TBD>

Description: The diagnostic is continuous in operation. A fault is detected if the 800V side voltage  measured at the ECU is greater than a calibrated threshold. To ensure robustness of diagnostic a debounce time is required to have elapsed prior to failure judgement. At ignition on, the voltage of the internal power stages is also checked against a threshold.

Monitoring Conditions: See McLaren Federal Certification - ECU Diagnostic Summary Table

##### Flow Charts.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a flow chart must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Flow Chart for 800V side (To be confirmed with MAL):

##### System voltage high – 400V side Diagnostic text Descriptions.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a text description must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Description:

System voltage high – 400V side

Associated DTC: <TBD>

McLaren Identifier: <TBD>

Description: The diagnostic is continuous in operation. A fault is detected if the 400V side voltage  measured at the ECU is greater than a calibrated threshold. To ensure robustness of diagnostic a debounce time is required to have elapsed prior to failure judgement. At ignition on, the voltage of the internal power stages is also checked against a threshold.

Monitoring Conditions: See McLaren Federal Certification - ECU Diagnostic Summary Table

##### Flow Charts.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a flow chart must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Flow Chart for 400V side (To be confirmed with MAL):

##### System voltage low – 400V side Diagnostic text Descriptions.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a text description must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Description:

System voltage low – 400V side

Associated DTC: <TBD>

McLaren Identifier: <TBD>

Description: The diagnostic is continuous in operation. A fault is detected if the 400V side voltage  measured at the ECU is less than a calibrated threshold. To ensure robustness of diagnostic a debounce time is required to have elapsed prior to failure judgement. At ignition on, the voltage of the internal power stages is also checked against a threshold.

Monitoring Conditions: See McLaren Federal Certification - ECU Diagnostic Summary Table

##### Flow Charts.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a flow chart must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Flow Chart for 400V side (To be confirmed with MAL):

##### System voltage low – 800V side Diagnostic text Descriptions.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a text description must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Description:

System voltage low – 800V side

Associated DTC: <TBD>

McLaren Identifier: <TBD>

Description: The diagnostic is continuous in operation. A fault is detected if the 800V side voltage  measured at the ECU is less than a calibrated threshold. To ensure robustness of diagnostic a debounce time is required to have elapsed prior to failure judgement. At ignition on, the voltage of the internal power stages is also checked against a threshold.

Monitoring Conditions: See McLaren Federal Certification - ECU Diagnostic Summary Table

##### Flow Charts.

- For each diagnostic included in the summary table (section 4.7.1.2 above), a flow chart must be made available to explain how a diagnostic operates.

- The following is an example of a Diagnostic Flow Chart for 400V side (To be confirmed with MAL):

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## P18 OBD Required Documents

.

### Requirements

#### A-P Document

- Each module owner shall provide a table of all diagnostics faults codes and categorize it like the following table:

- The OBD DTC code is subject to change if any modifications required after MAL review

- This format is supplied by CARB here: https://www.arb.ca.gov/msprog/obdprog/obdii_gas_monitor_checklist.xls

#### Summary Table Document.

- The summary table must be completed giving detail of every OBD relevant diagnostic detailed in the checklist, along with the conditions required for the diagnostic to run and detect a fault.

- The following screenshot is for example:

#### Input/Output Signals List Document.

- Every HW pin on every OBD module must be listed and categorized as OBD relevant or not.

- Example documentation, below:

- Every OBD relevant pin must have associated diagnostics listed in the Checklist and Summary Table.

- Every component connected to a non-OBD relevant pin must have associated justification in the Non-MIL/OBD components 4.6.1.8 section.

Note:

- In addition to hardwired inputs/outputs, also a separate list of each sent/received CAN message should be supplied, and if not OBD monitored, and explanation of why not.

### Safety Requirements.

Please refer to section 4.8 for details.

### Business Rules.

As defined in SoW document.

## Safety Requirements

### Requirements

#### Functional Safety

##### Requirement to achieve the FSRs allocated to the HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_FUSA_01 |
| Customer Req ID | SDS-HVDCDC-253 |
| Milestone | Pre-C Sample/PF3.0 |
| Purpose1 | The HVDCDC implementation of the TSRs shall achieve the FSRs allocated to the HVDCDC. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input as expected to a specific requirement covered in TSC and SSR document for FUSA |
| Output(s) | Output as expected to a specific requirement covered in TSC and SSR document for FUSA |
| Process | Process shall be as per specific Software requirement covered in TSC and SSR document for FUSA |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific Software requirement covered in TSC and SSR document for FUSA. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range of Values is not applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other systems |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to categorise and report the fault levels of all safety diagnostics.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_FUSA_01 |
| Customer Req ID | SDS-HVDCDC-254 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall categorise and report the fault levels of all safety diagnostics using the vehicle CAN signal NHVDCDCFault. Refer to Section 3.4 HVDCDC Fault Management for HVDCDC fault categorization. Refer to Section 3.2.6 HVDCDC Faulty State and 3.2.7 Exit HVDCDC Faulty State for HVDCDC state management on safety level 2 faults. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Input as expected to a specific requirement mention in section “4.2.1.3 Application State: HVDCDC Fault unlatching” of this document 2. Input as expected to a specific requirement mention in section “4.3 Diagnostics” of this document 3. Input as expected to a specific requirement mention in section “4.2. State-Machine” of this document |
| Output(s) | 1. Output as expected to a specific requirement mention in section “4.2.1.3 Application State: HVDCDC Fault unlatching” of this document 2. Output as expected to a specific requirement mention in section “4.3 Diagnostics” of this document 3. Output as expected to a specific requirement mention in section “4.2. State-Machine” of this document |
| Process | 1. Process as expected to a specific requirement mention in section “4.2.1.3 Application State: HVDCDC Fault unlatching” of this document 2. Process as expected to a specific requirement mention in section “4.3 Diagnostics” of this document 3. Process as expected to a specific requirement mention in section “4.2. State-Machine” of this document |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per individual Software requirement. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No such valid range is applicable. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other systems |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### HVDCDC Input Diagnostics (DFMEA Monitor1)

##### Requirement to perform input diagnostics by HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DFMEA_01 |
| Customer Req ID | SDS-HVDCDC-329 |
| Milestone | Pre C-Sample/PF3.0 Phase 2 |
| Purpose1 | The input diagnostics shall be performed by HVDCDC before any inputs are used in HVDCDC functions or functional diagnostics. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The TSC and SSR document for FUSA shall provide input for this requirement |
| Output(s) | The TSC and SSR document for FUSA shall provide output for this requirement |
| Process | The TSC and SSR document for FUSA shall provide process for this requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | The TSC and SSR document for FUSA shall provide Validation Rules/ Verification criteria for this requirement |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to check CAN signal transmission quality.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DFMEA_02 |
| Customer Req ID | SDS-HVDCDC-329(This requirement will be covered in TSC and SSR document for FUSA) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | ### The HVDCDC shall check communication, CAN signal transmission quality, E2E status, value plausibility and/or invalid enums where applicable for signal input diagnostics. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The TSC and SSR document for FUSA shall provide input for this requirement |
| Output(s) | The TSC and SSR document for FUSA shall provide output for this requirement |
| Process | The TSC and SSR document for FUSA shall provide process for this requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | The TSC and SSR document for FUSA shall provide Validation Rules/ Verification criteria for this requirement |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to set the input to a DEFAULT value.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DFMEA_03 |
| Customer Req ID | SDS-HVDCDC-329(This requirement will be covered in TSC and SSR document for FUSA) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | ### The HVDCDC shall discard the input, and/or set the input to a DEFAULT value, and/or trigger an appropriate fault as per individual input diagnostic reaction instructed. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The TSC and SSR document for FUSA shall provide input for this requirement |
| Output(s) | The TSC and SSR document for FUSA shall provide output for this requirement |
| Process | The TSC and SSR document for FUSA shall provide process for this requirement |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | The TSC and SSR document for FUSA shall provide Validation Rules/ Verification criteria for this requirement |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Monitor 2: HVDCDC Status Monitor (Contactor Diagnostics)

##### Requirement to report the equivalent failure codes onto the Debug CAN.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_DFMEA_05 |
| Customer Req ID | SDS-HVDCDC-511 |
| Milestone | Pre C-Sample/PF3.0 |
| Purpose1 | The HVDCDC shall monitor the Discharge contactor status against the command of closing the contactor, within calibratable time period and reset threshold. When the fault is set, Level 0 fault shall be raised |
| Derived Requirement | No Such derived requirement is applicable. |
| Requirement Priority | Medium |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The Signal NHVDCDCCommand @ EDSU_HVDCDC_Cmd shall set with value as 0x00 ("Standby") via Canalyzer to HCDCDC application. 2. The HVDCDC application shall be set in the "Disable" substate of the "discharge" state within 10 seconds, 3. The VICOR output voltage(HVDCDCPwrModule400V) shall be decrease to less than half of VICOR input voltage(high side) minus 15V and give more than 60V to 400V bus(400V EMI). 3. The fault response time by checking the HVDCDC applications transition time from "Active_Discharge_400V" to "Disable_400V_AD" substate of the "Discharge" state. |
| Output(s) | NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 24 and is raised after the fault response time of 1800ms. |
| Process | Verify that the HVDCDC detects the 400V active discharge is not effective and raise a level 0 fault. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | 1. The HVDCDC application is in Normal substate of HV_ON state, value of CAN signal HVDCDCStatus @ HVDCDC_Status1 is equal to 0x1 ("Normal") and signal NHVDCDCStatusDebug @ HVDCDC_Debug is equal to 0x6 ("Normal") 2. No fault is raised, value of CAN signal NHVDCDCFault @ HVDCDC_Status1 is equal to 0 |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool. NHVDCDCFault @ HVDCDC_Status1 shall equal to 0x1 (“level 0") NHVDCDCFaultID @ HVDCDC_Debug_PDU shall equal to 24 and is raised after the fault response time of 1800ms. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Signal Integrity and Plausibility Checks

##### Requirement to use AUTOSAR profile 1 for the E2E checksum calculation and E2E diagnostics.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_01 |
| Customer Req ID | SDS-HVDCDC-348 (It is an information. We will going to use non autosar profile, Need to confirm with MAL) |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall use AUTOSAR profile 1 for the E2E checksum calculation and E2E diagnostics. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 |  |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to perform E2E checksum and counter diagnostics on the Rx PDU.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_02 |
| Customer Req ID | SDS-HVDCDC-333 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall perform E2E checksum and counter diagnostics on the following Rx PDU: - EDSU_HVDCDC_Cmd shall be E2E protected. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Receive the frame/PDU from EDSU |
| Output(s) | The HVDCDC shall validate correct CRC byte and frame count for the received PDU. |
| Process | Verify that the CRC byte and frame count shall be validate by HVDCDC as part of E2E protection |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the CRC byte and frame count shall be validate by HVDCDC as part of E2E protection |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### Requirement to perform E2E checksum and counter diagnostics on the Tx PDU.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_03 |
| Customer Req ID | SDS-HVDCDC-334 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall perform E2E checksum and counter diagnostics on the following Tx PDU: - NHVDCDCStatus1 shall be E2E protected. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall add CRC byte and frame count for the transmit PDU before sending |
| Output(s) | The HVDCDC shall transmit TX PDU with CRC byte and frame count included. The HVDCDC shall not transmit TX PDU if CRC byte and frame count is missing. |
| Process | Verify that the CRC byte and frame count shall be included by HVDCDC as part of E2E protection in the TX PDU |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the CRC byte and frame count shall be included by HVDCDC as part of E2E protection in the TX PDU |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall continuously monitor if it receives an invalid combination of requests for NHVDCDCCommand from the domain controller.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_04 |
| Customer Req ID | SDS-HVDCDC-512 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall continuously monitor if it receives an invalid combination of requests from the controller and raise a fault if it does, if the situation persists for more than a calibratable time HVDCDC_CAL |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall receive either of the below values, 1. NHVDCDCCommand signal with value 0x3 (Unused_NHVDCDCCommand_NW) OR 2.NHVDCDCCommand signal with value 0x4 (Unused1_NHVDCDCCommand_NW) OR 3.NHVDCDCCommand signal with value 0x5 (Unused2_NHVDCDCCommand_NW) OR 4. NHVDCDCCommand signal with value 0x6 (Unused3_NHVDCDCCommand_NW) |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID 37 2.When the invalid value for NHVDCDCCommand are received for a configured calibratable time HVDCDC_CAL and the debounce time for this fault shall be calibratable time (Default- 50 msec). 3.Set the DTC “0x1E2500” |
| Process | 1.The HVDCDC shall receive CAN NM message. 2. The HVDCDC shall receive signal NHVDCDCCommand with any value mentioned in the input section. 3.Run the diagnostic to validate input command value EDSU_HVDCDC_Cmd @ NHVDCDCCommand 4.If the invalid value is received increment the debounce counter 5.Check if the debounce counter maximum physical value is exceeded 6.The fault should be raised after a configurable debounce time(50 msec) is elapsed. |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool 2.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU should provide Fault ID 37 3. Verify the DTC “0x1E2500” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall continuously monitor if it receives an invalid combination of requests for NHVDCDCCommand from the domain controller (2).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_05 |
| Customer Req ID | SDS-HVDCDC-512 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall continuously monitor if it receives an invalid combination of requests from the controller and raise a fault if it does, if the situation persists for more than a calibratable time HVDCDC_CAL |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall receive the below value, 1. NHVDCDCCommand signal with value 0x7 (SNA_NHVDCDCCommand_NW) |
| Output(s) | The HVDCDC shall raise the respective SNA Fault . |
| Process | The HVDCDC shall check for the value of NHVDCDCCommand == 0x7, If found then the software should raise respective SNA Fault if it exceeds a HVDCDC_CAL time in msecs. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the respective SNA Fault is raised in case of receiving the value 0x7 on NHVDCDCCommand signal. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall continuously monitor if it receives an invalid combination of requests for NHVDCDCFaultUnlatchRequestHVDCDC from the domain controller.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_SI_06 |
| Customer Req ID | SDS-HVDCDC-512 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall continuously monitor if it receives an invalid combination of requests from the controller and raise a fault if it does, if the situation persists for more than a calibratable time HVDCDC_CAL |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall receive either of the below values, 1.NHVDCDCFaultUnlatchRequestHVDCDC == 0x2 (Unused_NFaultUnlatchRequestHVDCDC_NW) OR 2. NHVDCDCFaultUnlatchRequestHVDCDC == 0x3 (Invalid_NFaultUnlatchRequestHVDCDC_NW) |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 37 2.When the invalid value for NHVDCDCFaultUnlatchRequestHVDCDC are received for a configured calibratable time HVDCDC_CAL and the debounce time for this fault shall be calibratable time (Default- 50 msec). |
| Process | 1.The HVDCDC shall receive CAN NM message. 2. The HVDCDC shall receive signal NHVDCDCFaultUnlatchRequestHVDCDC with any value mentioned in the input section. 3.Run the diagnostic to validate input command value EDSU_HVDCDC_Cmd @ NHVDCDCFaultUnlatchRequestHVDCDC 4.If the invalid value is received increment the debounce counter 5.Check if the debounce counter maximum physical value is exceeded 6.The fault should be raised after a configurable debounce time(50 msec) is elapsed. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.Verify the CAN messages and CAN signal mentioned in input and output section on CANalyzer Tool 2.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 37 |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Voltage rail monitoring

##### The HVDCDC software shall monitor the overvoltage fault of the 3V3 rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_01 |
| Customer Req ID | SDS-HVDCDC-542 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the overvoltage condition occurs in 3V3 rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense the 3V3 Rail that if it goes beyond the HVDCDC_CAL value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 11 2.When the analog sensor detects the voltage more than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 100 msec). 3.Set the DTC ”0x1E0B00” |
| Process | 1.The HVDCDC analog sensor shall sense the overvoltage. 2. The fault with level 2 shall be raised within a configurable debounce time(100 msec) ,with Fault ID 11 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the overvoltage fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 11 after 100 msec. 2. Verify the DTC ”0x1E0B00” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the undervoltage fault of the 3V3 rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_02 |
| Customer Req ID | SDS-HVDCDC-543 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the undervoltage condition occurs in 3V3 rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense the 3V3 Rail that if it goes below the HVDCDC_CAL value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 12 2.When the analog sensor detects the voltage more than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 100 msec). 3.Set the DTC ” 0x1E0C00” |
| Process | 1.The HVDCDC analog sensor shall sense the overvoltage. 2. The fault with level 2 shall be raised within a configurable debounce time(100 msec) ,with Fault ID 12 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the undervoltage fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 12 after 100 msec. 2.Verify the DTC ” 0x1E0C00” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the overcurrent fault of the 3V3 rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_03 |
| Customer Req ID | SDS-HVDCDC-544 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the overcurrent condition occurs in 3V3 rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Read the 3V3 rail voltage value from ADC sensor 2. Threshold value for 3V3 Rail “HVDCDC_CAL” shall be a calibratable parameter 3.The debounce time shall be “500ms” |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 97 2.When the analog sensor detects the voltage more than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC ”0x1E6100” |
| Process | 1.The ADC shall be powered up and communicating. 2.The HVDCDC shall sense the current if it is more than HVDCDC_CAL and raise a level 2 fault with Fault ID 97 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the overcurrent fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 97 after 500 msec. 2. Verify the DTC ”0x1E6100” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the short circuit fault of the 3V3 rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_04 |
| Customer Req ID | SDS-HVDCDC-545 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the short circuit condition occurs in 3V3 rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense the voltage of 3V3 Rail that if it goes above the HVDCDC_CAL |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 98 2.When the analog sensor detects the voltage more than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC ”0x1E6200” |
| Process | The HVDCDC shall sense the voltage if it is more than HVDCDC_CAL and raise a level 2 fault with Fault ID 98 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the short circuit fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 98 after 500 msec. 2. Verify the DTC ”0x1E6200” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the short to ground fault of the 3V3 rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_05 |
| Customer Req ID | SDS-HVDCDC-546 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the short to ground condition occurs in 3V3 rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense the voltage of 3V3 Rail that if it goes beyond the HVDCDC_CAL value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 99 2.When the analog sensor detects the voltage more than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC ”0x1E6300” |
| Process | 1.The HVDCDC shall sense the voltage if it is more than HVDCDC_CAL indicating a short to ground condition. 2.Raise a level 2 fault with Fault ID 99 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the short to ground fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 99 after 500 msec. 2. Verify the DTC ”0x1E6300” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the overvoltage fault of the 5VS voltage rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_06 |
| Customer Req ID | SDS-HVDCDC-547 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the overvoltage condition occurs in 5VS rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The sensor shall sense the voltage of 5VS rail that if it goes beyond the HVDCDC_CAL value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 13 when the sensor detects the voltage more than HVDCDC_CAL and this fault is raised after 100 msec. 2.Set the DTC “0x1E0D00” |
| Process | 1.The HVDCDC analog sensor shall sense the overvoltage. 2. The fault with level 2 shall be raised within a configurable debounce time(100 msec) ,with Fault ID 13 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense overvoltage fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 13 after 100 msec. 2. Verify the DTC “0x1E0D00” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the undervoltage fault of the 5VS voltage rail and raise level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_07 |
| Customer Req ID | SDS-HVDCDC-548 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) when the undervoltage condition occurs in 5VS rail to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense the voltage of 5VS rail that if it goes below the HVDCDC_CAL value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 14 2.When the analog sensor detects the voltage less than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 100 msec). 3.Set the DTC “0x1E0E00” |
| Process | 1. The HVDCDC shall sense the voltage if it is less than HVDCDC_CAL. 2. The fault with level 2 shall be raised within a configurable debounce time(100 msec) ,with Fault ID 14 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense undervoltage fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 14 after 100 msec. 2.Verify the DTC “0x1E0E00” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the overcurrent fault of the 5VS short to ground voltage rail and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_08 |
| Customer Req ID | SDS-HVDCDC-549 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall monitor the overcurrent fault of the 5VS short to ground voltage rail and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense overcurrent fault of the 5VS voltage rail that if it goes below the HVDCDC_CAL (7.9V) value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 100 2.When the analog sensor detects the voltage less than HVDCDC_CAL a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC ”0x1E6400” |
| Process | 1.The ADC shall be powered up and communicating. 2.The HVDCDC shall sense the overcurrent fault and raise a level 2 fault with Fault ID 100 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the overcurrent fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 100 after 500 msec. 2. Verify the DTC ”0x1E6400” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the short circuit fault of the 5VS voltage rail and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_09 |
| Customer Req ID | SDS-HVDCDC-550 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall monitor the short circuit fault of the 5VS voltage rail and raise a level 2 fault. The fault diagnostics shall be done as stated on the Interface Specification Document. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The analog sensor shall sense short circuit fault of the 5VS voltage rail that if it goes below the HVDCDC_CAL (5.4V) value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 101 2.When the analog sensor detects a short circuit condition a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC “0x1E6500” |
| Process | 1.The ADC shall be powered up and communicating. 2.The HVDCDC shall sense the short circuit fault and raise a level 2 fault with Fault ID 101 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the short circuit fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 101 after 500 msec. 2. Verify the DTC “0x1E6500” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the short to ground fault of the 5VS voltage rail and raise a level 2 fault.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_VRM_10 |
| Customer Req ID | SDS-HVDCDC-551 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall monitor the short to ground fault of the 5VS voltage rail and raise a level 2 fault. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The sensor shall sense short to ground fault of the 5VS voltage rail that if it goes below the HVDCDC_CAL (0.1V) value. |
| Output(s) | 1.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 102 2.When the analog sensor detects a short to ground condition a fault shall be raised after a debounce time (Default- 500 msec). 3.Set the DTC ”0x1E6600” |
| Process | 1.The ADC shall be powered up and communicating. 2.The HVDCDC shall sense the short to ground fault and raise a level 2 fault with Fault ID 102 after 500 msec. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense the short to ground fault and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 102 after 500 msec. 2. Verify the DTC ”0x1E6600” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Current monitors

##### The HVDCDC software shall monitor the implausible measurement of the 400V and 800V current sensors and raise a level 2 fault in case it fails.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_CM_01 |
| Customer Req ID | SDS-HVDCDC-555 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) the implausible measurement of the 400V and 800V current sensors to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.The HVDCDC shall monitor the implausible measurement of the 400V and 800V current sensors. 2. The HVDCDC shall be in Normal state of application. 3.Load shall be applied on any operating mode(buck/boost) |
| Output(s) | Monitor shall be inhibited for 800V current measurement within the range -3A to +3A for a defined calibratable time HVDCDC_CAL |
| Process | The HVDCDC shall monitor the implausible measurement of the 400V and 800V current sensors, if it goes beyond HVDCDC_CAL |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense implausible measurement of the 400V and 800V current sensors 2. Monitor shall be inhibited for 800V current measurement within the range -3A to +3A for a defined calibratable time HVDCDC_CAL |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

##### The HVDCDC software shall monitor the implausible measurement of the 400V and 800V current sensors and raise a level 2 fault in case it fails.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_CM_02 |
| Customer Req ID | SDS-HVDCDC-555 |
| Milestone | Pre C-Sample/PF3.0 Phase-2 |
| Purpose1 | The HVDCDC shall transit to the Faulty state (Level 2 fault) the implausible measurement of the 400V and 800V current sensors to avoid the damage of surrounding H/W components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1.The HVDCDC shall monitor the implausible measurement of the 400V and 800V 2.Application state shall be in Normal state 3. Load shall be applied on any operating mode(buck/boost) in such a way that 800V current measurement goes out of range |
| Output(s) | 1.Monitor shall be inhibited for 800V current measurement out of the range -3A to +3A 2.NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 108 3.Set the DTC ” 0x1E6C00” |
| Process | 1.The HVDCDC shall monitor the implausible measurement of the 400V and 800V if it goes beyond HVDCDC_CAL 2.If the debounce time (500ms) is crossed, the application shall raise NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 108 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | NHVDCDCStatus = 0, NHVDCDCStatusDebug = 0, NHVDCDCFault = 0. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | 1.To verify that the HVDCDC shall sense implausible measurement of the 400V and 800V and NHVDCDCFault @ HVDCDC_Status1 is equal to 0x3 ("Level 2") 2.NVHDCDCFaultID @ HVDCDC_Debug_PDU shall provide Fault ID 108 after 500 msec. 3.Verify the DTC ” 0x1E6C00” |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### Safety Requirements

Please refer to section 4.8 for details.

### Business Rules

As defined in SoW document.

## Non-Functional Requirements

### Requirements

#### The HVDCDC shall implement unique calibratables to individual diagnostics such that all diagnostic functions can be enabled/disabled individually.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_01 |
| Customer Req ID | SDS-HVDCDC-281 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall implement unique calibratables to individual diagnostics such that all diagnostic functions can be enabled/disabled individually. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The Calibration.hex file s update to make diagnostics function enable/disable |
| Output(s) | 1. Each individual function shall be enabled or disabled based on the calibration input. |
| Process | After providing the calibration.hex file, individual diagnostics or all diagnostic functions shall be enabled/disabled individually |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the individual diagnostics or all diagnostic functions shall be enabled/disabled individually using calibration.hex file, |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement unique calibratables to individual diagnostic thresholds and debounces such that all diagnostic functions can be calibrated individually.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_02 |
| Customer Req ID | SDS-HVDCDC-282 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall implement unique calibratables to individual diagnostic thresholds and debounces such that all diagnostic functions can be calibrated individually. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The Calibration parameters (thresholds and debounces) for all the individual diagnostic functions shall provide in calibration hex and through XCP for EOL testing |
| Output(s) | The Calibration parameters (thresholds and debounces) for all the individual diagnostic functions shall be configured a functioning as per the provided input |
| Process | Verify that all diagnostic functions can be calibrated individually using individual diagnostic thresholds and debounces |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement debouncing for each digital input signal in the system.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_03 |
| Customer Req ID | SDS-HVDCDC-321 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall implement debouncing for each digital input signal in the system. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Digital Input signal shall be provided to HVDCDC HW 2. Appropriate debouncing should be applied for each digital input |
| Output(s) | 1. The digital signal shall be handled by HVDCDC Software after validating it using debounce delay. 2. HVDCDC shall be processed received validated digital input |
| Process | To apply appropriate debouncing to digital input and process the same. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC shall mature the digital input received on HW pin and process it. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement Software filtering for each analog input signal in the system.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_04 |
| Customer Req ID | SDS-HVDCDC-322 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVDCDC shall implement Software filtering for each analog input signal. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The HVDCDC SW shall apply average filter to all the converted data for each analog signal 2. The HVDCDC SW shall be defined Sample depth (No. of sample) for each analog input signal. |
| Output(s) | 1. The final digital value for each converted analog input signal shall be calculated after applying average filter and defined sample depth |
| Process | The average filter and sample depth for each analog filter shall be correctly applied. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the final digital value for each converted analog input signal shall be calculated after applying average filter and defined sample depth |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall comply to the MAL quality standards.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_05 |
| Customer Req ID | SDS-HVDCDC-251 |
| Milestone | B-Sample/PF2.0 |
| Purpose1 | The HVHVDCDC shall comply to the MAL quality standards. Refer to MAL Supplier Quality Manual version 9 for the software documentation required at each platform release. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The SW shall be following Misra guidelines, coding guidelines, ASPICE process compliance and all other MAL quality standards |
| Output(s) | The SW shall be qualified with Misra guidelines, coding guidelines, ASPICE process compliance and all other MAL quality standards |
| Process | Follow the MAL quality standard for HVHVDCDC |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range value is not required. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC SW shall be qualified with Misra guidelines, coding guidelines, ASPICE process compliance and all other MAL quality standards |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Definition requirement on software architecture.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_ |
| Customer Req ID | SDS-HVDCDC-245 |
| Milestone | Pre-C Sample/ PF3.0 Phase-1 (waiting more details from MAL) |
| Purpose1 | [MAL to define requirement on software architecture.] |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Support for a multilayer SW architecture.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_ |
| Customer Req ID | SDS-HVDCDC-246 |
| Milestone | B - Sample/ PF1.0 (This requirement shall be fulfill when all the requirements will be tested) |
| Purpose1 | The HVDCDC shall support a multilayer SW architecture, including: - Application Software - RTE - Basic Software |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) |  |
| Output(s) |  |
| Process |  |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC SW architecture shall provide the flexibility to add on additional modulised application software components.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_06 |
| Customer Req ID | SDS-HVDCDC-247 |
| Milestone | B - Sample/ PF1.0 () |
| Purpose1 | The HVDCDC SW architecture shall provide the flexibility to add on additional modulised application software components. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. List of all the Application software component that can be modulised. 2. SW architecture shall have provision to add modulised application software components |
| Output(s) | 1. Make sure all the listed application SW components be modulised. 2. SW architecture shall allow easy addition, removal, or modification of components without affecting the entire system 3. SW architecture shall reduce dependencies between modules by using interfaces and contracts instead of direct dependencies |
| Process | The SW architecture should design such a way that it shall follow methods mentioned in input and output |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that SW architecture shall follow methods mentioned in input and output |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | No |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC state machine software runnable/execution rate should not be slower than 5ms.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_07 |
| Customer Req ID | SDS-HVDCDC-248 |
| Milestone | B - Sample/ PF1.0 |
| Purpose1 | The HVDCDC state machine software runnable/execution rate should not be slower than 5ms. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. The software runnable/execution rate shall not be greater than or equal to 5 msec |
| Output(s) | 1. Check the periodicity and execution time for all the runnable of state machine shall not be greater than or equal to 5 msec 2. Check the periodicity and execution time for all the tasks of state machine shall not be greater than or equal to 5 msec |
| Process | Verify that the tasks and runnable of the state machine shall follow the conditions mentioned in input and output |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the tasks and runnable shall follow the conditions mentioned in input and output. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Required resource usage limits : 60% of RAM, Flash, EEPROM resources used at PF3.0 level

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_08 |
| Customer Req ID | SDS-HVDCDC-280 |
| Milestone | Pre-C Sample/ PF3.0 Phase-2 (Need to confirm with MAL as it seems design constraint) |
| Purpose1 | The following resource usage limits are desired: <= 60% of RAM, Flash, EEPROM resources used at PF3.0 level |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. On PF3.0 release SW, calculate the RAM, Flash and EEPROM resources used in full load condition 2. On PF3.0 release SW, calculate the RAM, Flash and EEPROM resources used in normal condition |
| Output(s) | 1. On PF3.0 release SW, the RAM, Flash and EEPROM resources used in full load condition shall be less than or equal to 60 percent. 2. On PF3.0 release SW, the RAM, Flash and EEPROM resources used in normal condition shall be less than or equal to 60 percent. |
| Process | Verify that on PF3.0 release SW the RAM, Flash and EEPROM resources shall follow the conditions mentioned in input and output |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that on PF3.0 release SW the RAM, Flash and EEPROM resources shall follow the conditions mentioned in input and output |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Required resource usage limits : 80% of RAM, Flash, EEPROM resources used at PF4.0+ level.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_10 |
| Customer Req ID | SDS-HVDCDC-280 |
| Milestone | Pre-C Sample/ PF4.0 (Need to confirm with MAL as it seems design constraint) |
| Purpose1 | The following resource usage limits are desired: <= 80% of RAM, Flash, EEPROM resources used at PF4.0+ level |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. On PF4.0 release SW, calculate the RAM, Flash and EEPROM resources used in full load condition 2. On PF4.0 release SW, calculate the RAM, Flash and EEPROM resources used in normal condition |
| Output(s) | 1. On PF4.0 release SW, the RAM, Flash and EEPROM resources used in full load condition shall be less than or equal to 60 percent. 2. On PF4.0 release SW, the RAM, Flash and EEPROM resources used in normal condition shall be less than or equal to 60 percent. |
| Process | Verify that on PF4.0 release SW the RAM, Flash and EEPROM resources shall follow the conditions mentioned in input and output |
| Mandatory Fields | No such mandatory fields are applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values are applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that on PF4.0 release SW the RAM, Flash and EEPROM resources shall follow the conditions mentioned in input and output |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement unique calibratables to individual diagnostics such that all diagnostic functions can be enabled/disabled individually.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_19 |
| Customer Req ID | SDS-HVDCDC-425 |
| Milestone | Pre-C-Sample/PF3.0 Phase-2 |
| Purpose1 | The application and calibration part numbers should be increased by one unit for each new release of the Software using the shared part numbers. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | The HVDCDC shall use read previous 16 Byte value of application and calibration part number. |
| Output(s) | HVDCDC shall use the incremental 16-byte ASCII application and calibration value as the previous software release. |
| Process | - HVDCDC shall set 16 Byte application and calibration value with default value. - HVDCDC shall increment last byte of application and calibration value with respect to previous release. HVDCDC shall report incremental 16 Byte application and calibration value to primary controller. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | The 16 Byte application and calibration value shall be preloaded with default value as “xxxxxxxxxxxxxxxx” |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that HVDCDC shall send the incremental 16 byte ASCII application and calibration value w.r.to the previous software release to primary controller continuously. |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The filtering implementation shall be calibratable through unique parameters for each input signal in the HVDCDC.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_09 |
| Customer Req ID | SDS-HVDCDC-352 |
| Milestone | Pre-C Sample/ PF3.0 Phase-2 |
| Purpose1 | The filtering implementation shall be calibratable through unique parameters for each input signal in the HVDCDC. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | HVDCDC shall have independent scaling factor and offset for each input signal |
| Output(s) | To check both scaling factor and offset shall be calibratable for each input signal |
| Process | Verify that the HVDCDC shall have calibratable independent scaling factor and offset for each input signal |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | No such mandatory fields is applicable |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the HVDCDC shall have calibratable independent scaling factor and offset for each input signal |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement a Trap/Exception handling to activate the trap.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_11 |
| Customer Req ID | SDS-HVDCDC-323 |
| Milestone | B Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall implement a Trap/Exception handling to activate the trap. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the trap condition by making CTRAP pin to low state (0 V). 2. Trap input shall be enabled. (TRPPEN =1), Trap state bit TRPS set to 1 (Trap state active) |
| Output(s) | 1. The HVDCDC SW shall perform Trap handling, It shall switch off the power devices if the trap input becomes activated. 2. The HVDCDC shall deactivate all the power outputs and set them into passive state. |
| Process | Verify the Trap functionality shall be activated by the HVDCDC when the input conditions are met. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the Trap functionality shall be activated by the HVDCDC when the input conditions are met. Check the output on DMM or DSO for the particular Pin |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall implement a Trap/Exception handling to deactivate the trap.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_12 |
| Customer Req ID | SDS-HVDCDC-323 |
| Milestone | B Sample/ PF2.0 |
| Purpose1 | The HVDCDC shall implement a Trap/Exception handling to deactivate the trap. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Provide the trap condition by making CTRAP pin to low state (1 V). 2. Trap input shall be enabled. (TRPPEN =0), Trap state bit TRPS set to 0 (Trap state deactive) |
| Output(s) | 1. The HVDCDC SW shall perform Trap handling, It shall switch on the power devices if the trap input becomes deactivated (Provided that the IGN pin is ON). 2. The HVDCDC shall activate all the power outputs and set them into passive state. |
| Process | Verify the Trap functionality shall be deactivated by the HVDCDC when the input conditions are met. |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the Trap functionality shall be deactivated by the HVDCDC when the input conditions are met. Check the output on DMM or DSO for the Pin |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC shall comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002 Cybersecurity Interface Agreement between MAL and Suppliers (CIA).

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_20 |
| Customer Req ID | SDS-HVDCDC-256 |
| Milestone | Pre-C Sample/ PF3.0 |
| Purpose1 | The HVDCDC shall comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002 Cybersecurity Interface Agreement between MAL and Suppliers (CIA). |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input as expected to a specific requirement comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002 |
| Output(s) | Output as expected to a specific requirement comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002 |
| Process | Process shall be as per specific Software requirement comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | As per specific Software requirement comply to the MAL Cyber Security Requirement MTL.66.0001, and the the MTL.66.0002. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVHVDCDC shall be compatible with the B-sample and C-sample of hardware using the same application.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_13 |
| Customer Req ID | SDS-HVDCDC-353 |
| Milestone | Pre-C Sample/ PF3.0 |
| Purpose1 | The HVHVDCDC shall be compatible with the B-sample and C-sample of hardware using the same application. But different calibration must be used for activating/deactivating features and differences of the two hardware samples. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Same Application Binary shall be provided for testing on B and C Sample HW. 2. Separate calibration hex file shall be provided for B and C Sample HW testing |
| Output(s) | 1. The same application Binary shall be worked on B and C Sample HW. 2. The features shall be activated /deactivated based on different Calibration file |
| Process | Verify that the same application file shall be worked on B and C sample. Individual features can be enable or disable using Calibration file for both the HW samples |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the same application file shall be worked on B and C sample. Individual features can be enable or disable using Calibration file for both the HW samples |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Guidelines for configuring the unused pins of the Microcontroller for better EMI rejection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_14 |
| Customer Req ID | SDS-HVDCDC-354 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 (Need to confirm with MAL as it is design constraint) |
| Purpose1 | The unused pins of the Microcontroller shall be configured as per the below guidelines for better EMI rejection: - Analog Unused pins Pad shall be disabled (PDISC register in FW is available to do so) |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Analog Unused pins shall be set to Hight state 2. Analog Unused pins pad shall be disabled using PDISC register 3. For more details refer Controller_pinmapper.xls |
| Output(s) | Observe that the Analog unused pins shall be disabled |
| Process | Verify that the unused analog pins are properly disabled from software |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the unused analog pins are properly disabled from software |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Guidelines for configuring the unused pins of the Microcontroller for better EMI rejection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_15 |
| Customer Req ID | SDS-HVDCDC-354 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 (Need to confirm with MAL as it is design constraint) |
| Purpose1 | The unused pins of the Microcontroller shall be configured as per the below guidelines for better EMI rejection: - Digital Pins shall be configured as output and then Pulled Down from FW – (With Weakest Driver Mode) |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Digital Unused pins shall be set to Pull down/Pull down (Weakest driver mode) as per the better EMI rejection 2. Digital Unused pins shall be set to Input/Output as per the better EMI rejection 3. For more details refer Controller_pinmapper.xls |
| Output(s) | Observe that the Digital unused pins shall be disabled |
| Process | Verify that the unused Digital pins are properly disabled from software |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the unused Digital pins are properly disabled from software |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | Yes |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Guidelines for configuring the unused pins of the Microcontroller for better EMI rejection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_16 |
| Customer Req ID | SDS-HVDCDC-354 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 (Need to confirm with MAL as it is design constraint) |
| Purpose1 | The unused pins of the Microcontroller shall be configured as per the below guidelines for better EMI rejection: - The Slew rate /Edges shall be configured to low/ medium (“pad driver mode registers PDx bit” is available to do so from FW). |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Slew rate /Edges shall be configured to low/ medium using “pad driver mode registers PDx bit” as per the better EMI rejection 2. For more details refer Controller_pinmapper.xls |
| Output(s) | Observe that the Slew rate /Edges shall be configured to low/medium |
| Process | Verify that the Slew rate /Edges shall be configured properly |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the Slew rate /Edges shall be configured properly |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | No |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Guidelines for configuring the unused pins of the Microcontroller for better EMI rejection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_17 |
| Customer Req ID | SDS-HVDCDC-354 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 (Need to confirm with MAL as it is design constraint) |
| Purpose1 | The unused pins of the Microcontroller shall be configured as per the below guidelines for better EMI rejection: - Medium/ Low slew rate control can be applied to Connected pins as well e.g., SPI Pins for ADC communication with Power Board. This will reduce the sharp edges leading to low noise. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. Medium/ Low slew rate control shall be applied to the pins that are connected to power board. 2. For more details refer Controller_pinmapper.xls |
| Output(s) | The pins that are connected to power board shall produce low noise by configuring Medium/Low slew rate control |
| Process | Verify that the Medium/Low slew rate shall be configured properly pins connected to power board |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | No such Pre-Loaded Values is applicable. |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the Medium/Low slew rate shall be configured properly pins connected to power board |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | No |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### Guidelines for configuring the unused pins of the Microcontroller for better EMI rejection.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_18 |
| Customer Req ID | SDS-HVDCDC-354 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 (Need to confirm with MAL as it is design constraint) |
| Purpose1 | The unused pins of the Microcontroller shall be configured as per the below guidelines for better EMI rejection: -The JTAG /DAP port shall be disabled / Pulled Low after the programming is done. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | 1. JTAG /DAP port shall be set to Pulled down (Weakest driver mode) as per the better EMI rejection. 2. For more details refer Controller_pinmapper.xls |
| Output(s) | Observe that the JTAG /DAP port shall be disabled |
| Process | Verify that the JTAG /DAP port are properly disabled from software |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Programming on the HW shall be done |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify that the JTAG /DAP port are properly disabled from software |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | No |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | Yes |
| Critical (Yes/No) | Yes |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVHVDCDC shall implement the Pin Configuration stated in [356].

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_21 |
| Customer Req ID | SDS-HVDCDC-355 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVHVDCDC shall implement the Pin Configuration stated in [356] |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input is same as requirement ID SWRS_Non_Funct_22 |
| Output(s) | Output is same as requirement ID SWRS_Non_Funct_22 |
| Process | Process is same as requirement ID SWRS_Non_Funct_22 |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the CAN messages and CAN signal mentioned in input and out section on CANalyzer Tool |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested |  |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 |  |
| Critical (Yes/No) |  |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

#### The HVDCDC Software shall be configured as per the document with the Pin Mapped configuration.

| Term | Description |
| --- | --- |
| REQ ID | SWRS_Non_Funct_22 |
| Customer Req ID | SDS-HVDCDC-356 |
| Milestone | Pre-C Sample/ PF3.0 Phase 2 |
| Purpose1 | The HVDCDC Software shall be configure as per the document with the Pin Mapped configuration. |
| Derived Requirement | No Such derived requirement is applicable. |
| Access Restrictions | No Specific access restrictions is applicable. |
| Input(s) | Input shall be with respect to the Pin Mapped configuration document |
| Output(s) | Output shall be with respect to the Pin Mapped configuration document |
| Process | Preocess shall be with respect to the Pin Mapped configuration document |
| Mandatory Fields | No such mandatory fields is applicable. |
| Pre-Loaded Values | Valid range of Values is not applicable |
| Default Values | Default values shall be applicable with respect to Pre-Loaded Values |
| Valid range of Values | Valid range value is not required. |
| Data Latency Period | Data latency period is not applicable |
| Data Retention Period | Data Retention period is not applicable. |
| Data Rate/ <Daily> Number of transaction | Data rate is not applicable. |
| External Events | External event is not required. |
| Temporal Events | Temporal event is not applicable. |
| Validation Rules/ Verification criteria2 | Verify the Pin mapping configuration mentioned in input and out section with respect to relevant functionality |
| Constraints | Constraints is not applicable. |
| Effects on other systems/sub system | No effects on other system |
| Testability with respect to test environment (Yes/No) Note: Mention in which phase of testing, this requirement will be tested | YES |
| Feasible within project constraints (Yes/No) Note: Add the impact of requirement on operating environment defined in section 4.2 | YES |
| Critical (Yes/No) | YES |
| Acceptance Criteria | Acceptance Criteria for this requirement should be as per SECTION-6 ACCEPTANCE CRITERIA |

### 4.9.2  Safety Requirements

Not applicable

### Business Rules

As defined in SoW document.

### Screens / Wireframes

Not applicable

## Operating environment

This section defines the operating conditions that describe the component status for each of the requirements defined herein.

Unless different operating conditions are specifically defined or referenced within a given requirement, requirements will reference specific Operating Conditions defined in Table 1, which define quantitative values for external factors that are anticipated to strongly influence the HV DCDC operation.

|  | Operating Condition - Abbreviation | Operating Condition Description | DC Link High Voltage Input [V] | Coolant Inlet Temperature [degrees C] | Surrounding Component Temperature [degrees C] |
| --- | --- | --- | --- | --- | --- |
| Installed | NOR | Normal Operation | 480-672 | -20 to +90 | -20 to +85 |
|  | ND | Normal – Disabled |  | -40 to +90 | -40 to +85 |
|  | LVO | Low Voltage Operation | 400-480 | -20 to +90 | -20 to +85 |
| Not Installed | SHP | Shipping | 0 | N/A | -50 to +95 |
|  | SRV | Service |  |  |  |

## Configuration Parameters

| Parameter ID | Name | Unit | Default Value | MIN | MAX |
| --- | --- | --- | --- | --- | --- |
| CAN Baud rate | CAN_BAUD_RATE | Kbps | 500 | 100 | 1000 |
| CAN Msg Periodicity | CAN_MSG_PERIODICTY | Ms | 10 | 1 | 100 |

# External Interface

Below interfaces shall be used for HVDCDC project.

## User Interface

No specific GUI interface used for HVDCDC project.

## Hardware Interface

1. CANCASEXL HW shall be connected to HVDCDC HW to check CAN communication.

2. UART Serial cable shall be connected to HVDCDC HW to observe Serial Consol logs.

3. CANCASEXL HW shall be connected to HVDCDC HW to flash application binary and calibration.hex.

4. MiniWiggler debugger shall be used to flash Primary and Secondary bootloader on HVDCDC

5. Power Supply with 12V shall be provided on IGN pin of HVDCDC controller board.

6. High Side Power Supply with 800V shall be connected to HVDCDC power board.

7. 8W Low Side Load shall be connected to HVDCDC power board.

8. DMM is used to observe 800V and 400V supply from power supply and Vicor IC respectively.

## Software Interface

1. CANalyzer version 15 and 17 shall be used to check CAN communication with HVDCDC.

2. Teraterm or Similar tool will be used to observe Serial Consol logs.

3. PCOTApp tool shall be used to flash application binary and calibration.hex with bootloader implementation.

4. Infineon Memtool shall be used for flashing Primary and Secondary bootloader

5. Trace32 tool shall be used for debugging od SW code